Supporting Information

A versatile probe for serum albumin and its application in monitoring wound in live zebrafish

Rongrong Zhao,a‡  Ti Jia,a,‡ Hongyuan Shi,b and Chusen Huang*a

Table of contents

1. Synthetic procedures

1.1. Materials and apparatus

1.2. General procedures

2. Detailed protocol for in vitro assay

2.1. Materials and instruments

2.2. Structures of DA, DP, C7H and control probe Con-C7H and Cy7-Cl

2.3. Environmental polarity and pH effect on the fluorescence intensity of C7H

2.4. Fluorescence response of C7H, Con-C7H and Cy7-Cl towards HSA

2.5. Effect of DA or DP on the fluorescence emission of C7H

2.6. Determination of binding constant

2.7. Calculation of limit of detection

2.8. Nonlinear fitting and linear fitting between fluorescence intensity of C7H and concentration of HSA by origin software

2.9. Fluorescent analysis for HSA in real urine and serum samples

3. Tracing serum albumins with C7H in live cells

4. Tracing endogenous serum albumins with C7H in larval zebrafish

5. Movies

6. NMR spectrum and HRMS

1. Synthetic procedures

1.1. Materials and apparatus

All chemical reagents and solvents were commercially available and used without further purification. Thin-layer chromatography (TLC) was performed on silica gel plates. Column chromatography was performed using silica gel (Hailang, Qingdao) 200-300 mesh. 1H and 13C NMR spectra were recorded employing a Bruker AV-400 spectrometer with chemical shifts expressed in parts per million (in deuterochloroform, Me4Si as internal standard). Electrospray ionization (ESI) mass spectrometry was performed in a HP 1100 LC-MS spectrometer. MALDI-TOF/TOF-MS spectrum was obtained from SCIEX TOF/TOF 5800 System.

1.2. General procedures

Scheme S1. Synthesis of target probe C7H.

3-hydroxybutyl 4-methylbenzenesulfonate (1): Compound 1 was synthesized according to the reported procedures1. To a solution of butane-1,3-diol (5.00 g, 55.48 mmol) in anhydrous CH2Cl2 (150 mL) cooled in an ice bath, pyridine (5.81 mL, 72.15 mmol) and p-toluenesulfonyl chloride (11.60 g, 61.05 mmol) were added, and then the mixture was kept stirring at room temperature overnight under nitrogen atmosphere. The reaction was monitored by TLC with eluent of petroleum ether (PE)/ acetic acetylate (EA, v/v, 3:1, Rf = 0.3). After the starting material disappeared, the reaction was quenched with saturated NH4Cl solution. The resultant mixture was extracted with EA three times (3×150 mL). The combined organic layers were washed with 10% HCl solution, saturated NaHCO3 solution, and saturated brine, and dried with anhydrous Na2SO4. Then the organic layer was filtrated and evaporated under reduced pressure. The crude product was purified by silica gel column chromatography (PE/EA, from 10/1 to 2/1, v/v) to yield the target product as a colorless oil (12.284 g, yield 91%). 1H NMR (400 MHz, CDCl3) δ 7.79 (d, J = 8.2 Hz, 2H), 7.35 (d, J = 8.1 Hz, 2H), 4.27-4.19 (m, 1H), 4.15-4.07 (m, 1H), 3.97-3.87 (m, 1H), 2.45 (s, 3H), 1.84-1.77 (m, 1H), 1.70 (ddt, J = 14.2, 8.9, 5.2 Hz, 1H), 1.17 (d, J = 6.3 Hz, 3H). 13C NMR (100 MHz, CDCl3) δ 144.91, 132.85, 129.92, 127.86, 67.95, 64.02, 37.83, 23.54, 21.63.

4-iodobutan-2-ol (2): A solution of the above 3-hydroxybutyl 4-methylbenzenesulfonate (5.01 g, 20.51 mmol) was added to a 250 mL round bottomed flask containing 180 mL of dry tetrahydrofuran (THF), and to the solution was added LiI (3.29 g, 24.58 mmol). The mixture was heated under reflux for 40 min, and the reaction was monitored by TLC with eluent of PE/EA (5/1, v/v, Rf= 0.4) with H2SO4 - EtOH chromogenic agent. After cooling to room temperature, the resultant was filtered off through a plug of celite, and then washed with EA. The filtrate was diluted with EA (200 mL) and washed by saturated brine, then the organic layer was dried with anhydrous Na2SO4, filtrated and concentrated under reduced pressure. The crude product was purified by silica gel column chromatography (PE/EA, from 10/1 to 5/1, v/v) to give 2 as a pale-yellow oil (3.44 g, yield 84%). 1H NMR (400 MHz, CDCl3) δ 3.93 (dq, J = 12.4, 6.2 Hz, 1H), 3.28 (t, J = 7.0 Hz, 2H), 1.96 (dd, J = 14.2, 5.3 Hz, 2H), 1.24 (d, J = 6.2 Hz, 3H). 13C NMR (100 MHz, CDCl3) δ 67.87, 42.26, 23.24, 2.89.

2-((E)-2-((E)-2-chloro-3-((E)-2-(1-ethyl-3,3-dimethylindolin-2-ylidene)ethylidene)cyclohex-1-en-1-yl)vinyl)-1-ethyl-3,3-dimethyl-3H-indol-1-ium iodide (Cy7-Cl)

Cy7-Cl was synthesized according to the literature2. 1H NMR (400 MHz, DMSO-d6) δ 8.27 (d, J = 16.5 Hz, 2H), 7.65 (d, J = 15.3 Hz, 2H), 7.49-7.42 (m, 4H), 7.32-7.27 (m, 2H), 6.34 (d, J = 12.7 Hz, 2H), 4.27 (q, J = 2.9 Hz, 4H), 2.73 (t, J = 13.8 Hz, 4H), 1.89-1.83 (m, 2H), 1.68 (S, 12H), 1.32 (t, J = 1.3 Hz, 6H). 13C NMR (100 MHz, DMSO-d6) δ 172.29, 148.48, 143.57, 142.13, 141.68, 129.13, 126.59, 125.65, 123.04, 111.79, 101.76, 49.49, 27.87, 26.36, 20.85, 12.69. HRMS (ES+): calc. for C34H40N2Cl 511.2880, found 511.2881 [M]+.

(2E,6E)-2,6-bis(2-((E)-1-ethyl-3,3-dimethylindolin-2-ylidene)ethylidene)cyclohexan-1-one (Cy7-OH)

To a solution of Cy7-Cl (375.00 mg, 0.73 mmol) in 20 mL of dry N,N-dimethylformamide (DMF), was added sodium acetate (180.00 mg, 2.19 mmol), and the mixture was stirred at 90 oC for 5 h under nitrogen atmosphere. The reaction was monitored by TLC with eluent of PE/dichloromethane (CH2Cl2, 1/5, v/v, Rf= 0.3). After cooling to room temperature, the resultant was diluted with CH2Cl2 (100 mL), and washed with saturated brine. The combined organic layer was dried with anhydrous Na2SO4 overnight, filtered and then the solvent was evaporated. The crude product was purified by silica gel column chromatography (PE/CH2Cl2, from 50/1 to 1/5, v/v) to afford a red powder (0.21 g, yield 59%). 1H NMR (400 MHz, CDCl3) δ 8.18 (d, J = 13.2 Hz, 2H), 7.19 (t, J = 6.5 Hz, 4H), 6.91 (t, J = 7.3 Hz, 2H), 6.68 (d, J = 8.0 Hz, 2H), 5.47 (d, J = 13.2 Hz, 2H), 3.74 (q, J = 7.2 Hz, 4H), 2.62 (t, J = 5.7 Hz, 4H), 1.90-1.83 (m, 2H), 1.67 (s, 12H), 1.26 (d, J = 7.3 Hz, 6H). 13C NMR (100 MHz, CDCl3) δ 161.85, 143.74, 139.82, 132.89, 127.66, 126.49, 121.88, 120.45, 106.41, 92.13, 46.57, 37.05, 28.71, 25.87, 22.58, 11.16. HRMS (ES+): calc. for C34H40N2O 492.3141, found 493.3219 [M+H]+.

1-ethyl-2-((E)-2-((E)-3-(2-((E)-1-ethyl-3,3-dimethylindolin-2-ylidene)ethylidene)-2-(3-hydroxybutoxy)cyclohex-1-en-1-yl)vinyl)-3,3-dimethyl-3H-indol-1-ium iodide (C7H)

Cy7-OH (100.00 mg, 0.20 mmol) and NaH (5.85 mg, 0.24 mmol) were added to a dried three-necked flask containing 16 mL of anhydrous DMF under the protection of nitrogen, and the mixture was stirred in ice-water bath for 30 min. After adding compound 2 (1.22 g, 6.10 mmol) in anhydrous DMF (2 mL), the reaction was continued to stir for 30 min, and then heated to 65 oC after removing the ice-water bath to react for 4 days. The reaction was monitored by TLC CH2Cl2/methanol (MeOH, 30/1, v/v, Rf= 0.3) until the starting Cy7-OH disappeared. After cooling to room temperature, the solvent was removed under reduced pressure and the resultant mixture was diluted with CH2Cl2. The organic layer was washed with ice saturated NH4Cl solution and saturated brine, and then dried with anhydrous Na2SO4, filtrated and concentrated under reduced pressure. The crude product was purified by silica gel column chromatography (from pure CH2Cl2 to CH2Cl2/MeOH, 40/1, v/v) to afford green oily product. The target product was crystallized out with adding hexane and dried in vacuum to afford dark-green powder (53.4 mg, 38% yield). 1H NMR (400 MHz, CD3OD) δ 8.21-8.11 (m, 1H), 7.89-7.79 (m, 1H), 7.64-7.49 (m, 3H), 7.44-7.37 (m, 2H), 7.31-7.22 (m, 2H), 7.07-6.99 (m, 2H), 5.86 (d, J=12.4 Hz), 4.74 (br, 4H), 4.24-4.16 (m, 1H), 3.97-3.66 (m, 2H), 2.81-2.64 (m, 4H), 2.00-1.92 (m, 2H), 1.78-1.67 (m, 12H), 1.33-1.27 (m, 11H). 13C NMR (100 MHz, CD3OD) δ 166.12, 144.21, 143.41, 142.48, 141.06, 132.59, 130.38, 129.84, 129.49, 127.94, 125.97, 123.94, 123.52, 123.13, 114.85, 113.22, 111.56, 109.46, 95.25, 50.27, 38.62, 34.98, 33.10, 30.79, 28.90, 26.13, 24.86, 23.86, 14.50, 11.78. Maldi-Tof/Tof-MS: calc. for C38H49N2O2+ 565.3789, found 565.4582 [M]+.

Scheme S2. Synthesis of control probe Con-C7H.

2-((E)-2-((E)-2-(4-(tert-butyl)phenoxy)-3-((E)-2-(1-ethyl-3,3-dimethylindolin-2-ylidene)ethylidene)cyclohex-1-en-1-yl)vinyl)-1-ethyl-3,3-dimethyl-3H-indol-1-ium iodide (Con-C7H)

To a solution of NaH (60% in mineral oil, 5.3 mg, 0.132 mmol) in 1 mL of anhydrous DMF under nitrogen atmosphere at 0°C, 4-(tert-butyl) phenol (31.4 mg 0.209 mmol) in 2mL of DMF was added. After the reaction mixture was stirring at 0°C for 30 min, the ice bath was removed and the reaction was allowed to warm to room temperature. Next, 5 mL of anhydrous DMF containing Cy7-Cl (57.1 mg, 0.089 mmol) was added to the solution, the reaction was continued to stirring for 24 hours. After TLC showed the starting material Cy7-Cl disappeared, 200 µL of water was added to quench the reaction. The solvent was evaporated by rotary evaporator, and the crude product was purified by silica gel column chromatography with eluent of CH2Cl2/MeOH (30/1, v/v) to afford dark-green solids (13.6 mg, yield 20%). 1H NMR (400 MHz, DMSO-d6) δ 7.83 (d, J = 26.8 Hz, 2H), 7.51 (d, J = 20.0 Hz, 2H), 7.38 (dd, J = 9.1 Hz, 6H), 7.20 (dd, J = 6.4 Hz, 2H), 7.06 (d, J = 16.8 Hz, 2H), 6.17 (d, J = 29.6 Hz,2H), 4.17 (br, 4H), 2.71 (t, J = 14.0 Hz,4H), 1.24 (t, J = 10.0, 29 Hz, 29H); 13C NMR (100 MHz, DMSO-d6) δ 171.56, 163.45, 157.98, 144.98, 142.06, 141.48, 141.45, 128.96, 127.35, 125.20, 122.89, 121.76, 114.37, 111.39, 100.24, 49.0, 40.45, 40.24, 40.03, 39.82, 39.62, 39.41,31.62, 27.52, 12.54. HRMS (ES+): calc. for C44H53N2O+ 625.4158, found 625.4157 [M]+.

2. Detailed protocol for in vitro

2.1. Materials and instruments

Ultrapure water (Heal Force, Shanghai) was used to prepare all aqueous solutions. DMEM (High glucose) medium was purchased from Hyclone. Fetal Bovine Serum (FBS) was from Gibco. Cation stock solutions (0.0376 M) of Zn2+, Ca2+, Cu2+, Fe3+ and Fe2+ were obtained by dissolving their corresponding chlorides or sulfate, such as ZnCl2, CaCl2, CuSO4∙5H2O, FeCl3∙6H2O and FeCl2∙4H2O, in water separately. Sodium salts, NaCl, NaBr, NaI, NaF, Na2SO4 and Na2SO3, were utilized to prepare aqueous anion stock solutions (0.0376 M) for Cl-, Br-, I-, F-, SO42- and SO32- respectively. H2S was prepared by adding Na2S∙9H2O into water. All amino acids were dissolved in water to give stock solutions with concentration of 0.0376 M. The stock solution (8×10-4 M) of probe C7H for assay in vitro was dissolved in DMSO, stored under argon or nitrogen, at ≤-20 oC and protected from light, and diluted to the final working concentration (4 μM). Human urine and serum were both obtained from healthy volunteers (provided by Rui Jin Hospital Shanghai Jiao Tong University School of Medicine, Shanghai), preserved at -20 oC pending analysis, and allowed to thaw at ambient temperature prior to use. Absolute quantum yields were measured by HAMAMATSU UV-NIR absolute PL quantum yield spectrometer C13534 Quantaurus-QY Plus. All pH measurements were carried on with a pH-Meter PHB-4 (INESA Scientific Instrument Co., Ltd.). Fluorescence spectra were recorded with a Hitachi fluorescence spectrophotometer (F-7000).

2.2. Structures of DA, DP, C7H and control probe Con-C7H and Cy7-Cl

Figure S1. a) The chemical structures of two typical site-specific fluorescence probes for HSA, dansylamide (DA, for site I) and dansylproline (DP, for site II), as well as the synthesized target near-infrared probe C7H and control probe Con-C7H. b, c and d) molecular docking between DA (C7H or Con-C7H) and HSA. DA, C7H and Con-C7H (cyan color) and key residues of HSA were represented as sticks (wheat color). HSA were represented as cartoon (smudge color), the HSA in d) was represented as surface in smudge color. Hydrogen bond was represented as yellow dotted lines. Cation-π was represented as green dotted lines. The crystalline data of HSA is from RCSB Protein Data Bank (http://www.rcsb.org), PDB: 2BXA. The molecular docking was conducted on Dock 6.7 software, the final photos were generated with PyMOL1.8.6.

2.3. Environmental polarity and pH effect on the fluorescence intensity of C7H

Figure S2. Exploration the effect of environmental polarity on the fluorescence of C7H. Fluorescence changes of C7H in the mixture solution of pure water and ethanol. The mixture solution was prepared with increasing addition of ethanol into pure water and then further addition of increasing amounts of pure water. Note: the concentration of C7H in the mixture solution should be kept in same. With the addition of ethanol, the polarity of mixture solution decreased and the fluorescence intensity of C7H increased. Similarly, the polarity of mixture solution increased with further addition of pure water, and the fluorescence of C7H decreased.

Figure S3. Fluorescence intensities of 4 µM of probe C7H (a) and HSA-C7H complex (b) at 710 nm (λex = 630 nm) in PBS buffer (0.2 M) with different pH.

2.4. Fluorescence response of C7H, Con-C7H and Cy7-Cl towards HSA

Figure S4. a) Absorbance spectrum of 4 µM of C7H, Con-C7H and Cy7-Cl in the presence of HSA (0.25 mg mL-1). b-d) fluorescence response of C7H, Con-C7H and Cy7-Cl for different concentration of HSA (0-50 mg mL-1). e, f) Fluorescence response of C7H, Con-C7H and Cy7-Cl to different concentration of HSA at the maximum emission intensity in PBS buffer (pH 7.4, 0.2 M). λex= 630 nm, λem= 710 nm for C7H, λex= 770 nm, λem= 793 nm for Con-C7H, λex= 780 nm, λem= 798 nm for Cy7-Cl.

2.5. Effect of DA or DP on the fluorescence emission of C7H

Figure S5. Effect of DA (a) or DP (b) on fluorescence intensity of C7H (4 µM) in PBS buffer (pH 7.4, 0.2 M). λex = 630 nm.

2.6. Determination of binding constant

According to the MALDI-TOF MS result, there is one probe was bound to the sit I of HSA, indicating the binding mode between C7H and HSA is 1:1 binding. Thus, the binding constant was determined with one host vs one guest binding mode by using the following formula:

Int=Igh+(Ig-Igh)*(((Go-Ho-1/Kg)/2+sqrt(((Ho+Go+1/Kg)^2)/4-Ho*Go))/Go)    (1)

Whereby, the Int is fluorescence intensity of the solution containing C7H and HSA. Igh is the fluorescence intensity of C7H-HSA complex, Ig is the fluorescence intensity of free C7H, Go is the initial concentration of C7H (herein is fixed to 4 μM), Ho is the added concentration of HSA (herein, Ho changed with the concentration of HSA increased). The fitting module can be obtained from the website http://wernernau.user.jacobs-university.de/ under “Fitting Functions”.

Figure S6. Binding constant determination of C7H for HSA in PBS buffer (pH 7.4). a) Fluorescence changes of C7H/Con-C7H/Cy7-Cl with the increasing concentrations of HSA. The inset: fluorescence intensity as the function of low concentration (0-0.5 mg mL-1). b) Apparent binding curve of C7H for HSA.

2.7. Calculation of limit of detection

The limit of detection (LOD) was calculated according to IUPAC and ACS method.

LOD=3×σs                                          (2)

In this formula, 3 represents confidence level, σ is the standard deviation of blank samples, and s is defined as the sensitivity of analysis (the slope of linear equation). σ are afforded through calculating the standard deviations of 11 times of independent fluorescence intensity (IF) of C7H (4 μM) without adding HSA in PBS buffer (0.20 M, pH 7.4) at 710 nm, the value is 0.379152. As shown in the Figure S7, the relationship between fluorescence intensity of C7H (IF) at 710 nm and low range of HSA concentration (from 0 to 0.04 mg mL-1) meets the liner equation (y = 9.11903 + 1142.13314 x), so s is the slope of this liner equation, that is 1142.13314. Then put the generated values into above formula, and the LOD was calculated to be about 9.96 × 10-4 mg mL-1.

Figure S7. The fluorescence response of C7H (4 µM) at 710 nm was linearly related to the HSA concentrations (0-0.04 mg mL-1). Error bars (SD) represent three independent experiments.

2.8. Nonlinear fitting and linear fitting between fluorescence intensity of C7H and concentration of HSA by origin software

As displayed in Figure S7, when the concentration of HSA is extremely low (from 0 to 0.04 mg mL-1), the relationship between fluorescence intensity of C7H (IF) at 710 nm and low range of HSA concentration (from 0 to 0.04 mg mL-1) meets the liner equation (y = 9.11903 + 1142.13314 x). While the concentration of HSA increased, this relationship between fluorescence intensity of C7H (IF) at 710 nm and HSA concentration also could be fitted to nonlinearly. The nonlinear fitting through origin software displayed this relationship (Table S1).

Table S1. Nonlinear fitting of fluorescence intensity of C7H to different concentrations of HSA in PBS buffer (0.2 M, pH 7.4).

| HSA concentration range (mg mL-1) | Emission wavelength (nm) | Fitting equation | R2 |
| --- | --- | --- | --- |
| 0-0.5 | 710 | y = 488.03087 x0.60755 | 0.99151 |
| 1-50 | 710 | y = 301.24232 x0.17673 | 0.98875 |

In order to facilitate the quantitative detection of HSA, the vertical axis y and the horizontal axis x are refined as log(IF) and log [HSA] respectively, then the nonlinear relationship of the IFC7H and HSA concentration can be transformed to a linear relationship of log(IF) and log[HSA], the linear fitting equation will be obtained through origin software in Table S2. This fitting method was also suitable for the real sample detections including the detection of HSA in human urine and human serum.

Table S2. Linear fitting between log(IF) and log[HSA] in PBS buffer (0.2 M, pH 7.4).

| HSA concentration range (mg mL-1) | Emission wavelength (nm) | Fitting equation | R2 |
| --- | --- | --- | --- |
| 0-0.5 | 710 | y = 2.72253 + 0.66854 x | 0.99123 |
| 1-50 | 710 | y = 2.4758+ 0.17975 x | 0.98545 |

2.9. Fluorescent analysis for HSA in real urine and serum samples

To investigate C7H could be potentially used for clinical analysis of HSA, human urine and serum were taken as the real samples. This two samples with different concentration of HSA were prepared by using a standard addition method. Human urine was diluted 2 times with PBS buffer (0.20 M, pH 7.4) before adding target HSA. For the human serum, as serum containing a large amount of proteins, salting-out method was introduced to separate serum proteins to meet the experimental requirements. Then different HSA concentrations was added by the standard addition method. In brief, human serum was diluted 2 times with PBS buffer, and to the diluted serum solution was gradually added the ground ammonium sulfate. After being vortexed and then kept still for 2 h, the required supernatant was then obtained by ultrafiltration or centrifugation, stored at -20 oC, and diluted 10 times prior to the assay. Then, the treated human serum was used to prepare real samples with different HSA concentrations. An amount of 2.0 mL of the serum or urine samples was mixed thoroughly with 10 μL of stock solution (8×10-4 M) of C7H (final working concentration, 4 μM). After the incubation at room temperature (25 oC) for 5 min, the obtained mixture solutions were transferred to quartz cuvettes and the fluorescence measurement was performed at spectrophotometer.

The procedures of recovery test for HSA was similar to the above real sample assay. Recovery was calculated based on average of fluorescence intensity according to the following equation:

Recovery*=x710xAdded×100%               (3)

Herein, X710 is the average concentration from three independent assay. The concentration is calculated according to the fluorescence intensity at 710 nm, which can be inserted into the linear relationship in Figure S8b (for urine sample) and Figure S8d (for serum sample).

Figure S8. Fluorescence spectra of C7H (4 μM) with addition of increasing concentrations of HSA in human urine (a) and serum (c), λex = 630 nm. (b, d) Linear relationship between emission intensity of C7H at 710 nm (log (IF)) and [HSA] (log ([HSA])). Inset: nonlinear fitting between emission intensity of C7H at 710 nm and [HSA]. [HSA] (a, b) 0 - 0.5 mg mL-1; (c, d) 1 - 40 mg mL-1. Error bars (SD) represent three independent experiments.

Table S3. Recovery of HSA in human urine and serum samples.

Type; Added (mg mL-1); λem = 710 nm. Found (mg mL-1); RSD (%); Recovery (%). urine; 0.065; 0.0662±0.00149; 2.3; 101.8. 0.15; 0.152±0.0018; 1.2; 101.3. 0.19; 0.194±0.0034; 1.8; 102.1. serum; 3; 2.966±0.0758; 2.6; 98.9. 19; 18.745±0.4215; 2.2; 98.7. 35; 34.683±1.2338; 3.6; 99.1.

3. Tracing serum albumins with C7H in live cells

3.1. MTT assay

A standard MTT (3-(4,5-Dimethylthiazol-2-yl)-2,5-diphenyltetrazolium bromide) assay was used to test the cytotoxicity of C7H. The Chang liver cells were first seeded into a plate with complete medium (90% DMEM and 10% FBS), and then the cells were incubated overnight (37 oC, 5% CO2) to allow them to adhere to the bottom of each well. Afterwards, the origin culture medium was siphoned off and the wells were washed three times with fresh DMEM. Cells were subsequently loaded with fresh DMEM containing C7H with the concentrations of 0, 2, 4, 8, 12, 15 μM, and incubated for 12 h, 24h respectively. Then, the culture medium was removed, and 100 μL of MTT solution (1.0 mg mL-1) was added to each well, followed by incubation at 37 ℃ for 4 hours. After that, 100 μL of the supernatant for each well was replaced by 150 μL of DMSO to dissolve the formazan crystals. The 96-well plate was shaken for 15 min. Finally, cell viability was determined by testing the absorbance of the solution at 490 nm with a microplate reader.

Figure S9. Cytotoxicity of C7H in Chang liver cells. Error bars represent s.d..

3.2. Live cell imaging with C7H

All cell images were taken by Leica TCS SP5 II Confocal Laser Scanning Microscope and Olympus IX71 fluorescence microscope.

Chang liver cells were seeded into confocal petri dish in complete medium (90% DMEM and 10%FBS), and then incubated overnight under standard culture conditions (atmosphere of 5% CO2 and 95% air at 37°C) to allow the cells attach. The cells were washed one time with fresh DMEM medium, and then were incubated for 6 h with 2 mL of fresh DMEM to attain cell cycle synchronization by serum starvation. Replace the pure DMEM with medium containing C7H/Con-C7H/Cy7-Cl, respectively, and incubate cells for 30 min under standard culture conditions. After incubation, 2 mL of fresh DMEM culture medium was added and then observed cells under confocal microscope/Olympus IX71 microscope.

Figure S10. Fluorescence imaging of C7H, Con-C7H and Cy7-Cl in chang liver cells. Cells were incubated with C7H/Con-C7H/Cy7-Cl (10 µM), respectively, for 30 min and wished by serum-free DMEM three times, microscope images were then obtained. Scale bar = 50 µm.

Figure S11. The correlation of Lysotracker and C7H intensities. Scale bar = 5 µm. Pearson's correlation Rr=0.63; Overlap coefficient R =0.69. Image J was used for the scatterplots of overlap.

3.3. Subcellular co-localization of C7H with LysoTracker® Green DND-26

Co-localization experiment of lysosomes was conducted with commercially available lysosome probe (LysoTracker® Green DND-26). LysoTracker® Green DND-26 was first prepared as a DMSO stock solution with a concentration of 2×10-5 M and was diluted with DMEM for cell incubation. Seed Chang liver cells into confocal petri dish in complete medium, and then incubated for 12 h under standard culture conditions. After the cells had attached, the cells were washed with fresh DMEM, incubated for 6 h with 2 mL of fresh DMEM to attain cell cycle synchronization, and then add pre-warmed (37 oC) DMEM containing LysoTracker® Green DND-26 (100 nM) to incubate under standard culture conditions for 2 h. Then add 10 μL of C7H stock solution (2×10-3 M) to achieve the working concentration (10 μM) and continue to incubate for 30 min. After that, replace the loading solution with fresh medium and observe the cells using confocal microscope (Leica TCS SP5 II Confocal Laser Scanning Microscope).

4. Tracing endogenous serum albumins with C7H in larval zebrafish

4.1. Materials and instruments

The zebrafish embryos and tricaine methanesulfonate (MS-222) used to anesthetize zebrafish were provided by Shanghai Institute of Biochemistry and Cell Biology, Shanghai Institutes for Biological Sciences, Chinese Academy of Sciences. All the use of larval zebrafish in this assay was approved by the Institutional Biomedical Research Ethics Committee of the Shanghai Institutes for Biological Sciences. In the experiment, embryos were from the same female fish and fed in culture water for 3-5 days under 28 °C to provide the larval zebrafish. During the second day of cultivation, in order to facilitate our observation of the subsequent experimental phenomena, adding 3% 1-phenyl-2-thio-ure (PTU) to inhibit the formation of melanin in larval zebrafish. Target probe C7H and control probe Con-C7H were prepared into 8×10-4 M DMSO stock solution, respectively. The pretreatment of larval zebrafish was conducted under the observation of Nikon stereoscopic microscope SMZ745T. The fluorescence images were obtained under differing processing conditions using an Olympus IX71 microscope at 4 × objective.

4.2. Dynamic tracing of endogenous serum albumins with C7H in larval zebrafish

Firstly, the stock solution of C7H and Con-C7H were diluted to 10 μM with PBS buffer (0.2 M, pH 7.4) for further incubation. Then well-behaved larval zebrafishes were fed with 2 mL PBS buffer containing C7H (or Con-C7H) in six-well plate and the changes of fluorescence were recorded every 1 min, and the dynamic tracing process lasted for 2 hours (120 minutes) by the Olympus IX71 microscope at 4 × objective. The obtained time-lapse fluorescence images were merged with ImageJ software to provide the video. Meanwhile, the photos at very 10 min interval were taken and the relative fluorescence intensity was semi-quantitative analyzed with ImageJ software. ROI module in ImageJ software was used to analyze the fluorescence intensity in brain and yolk areas.

Figure S12. a) Time-dependent fluorescence imaging in larval zebrafish with Con-C7H (10 µM). b) Enlarged fluorescence images of brain and yolk section in larval zebrafish with Con-C7H (10 µM). Images were obtained by an Olympus IX71 microscope with a 4X objective. Scale bar, 250 µm.

4.3. Dynamic tracing of endogenous serum albumins with C7H in wound area of larval zebrafish

The stock solution of C7H and Con-C7H were diluted to 10 μM with PBS buffer (0.2 M, pH 7.4) for further incubation. Then the well-behaved larval zebrafishes were selected and their waists were scratched gently with a sharp surgical blade to induce an acute wound under the observation of Nikon stereoscopic microscope SMZ745T. Then the wound larval zebrafishes were immediately treated with 2 mL of prepared PBS buffer containing C7H (or Con-C7H). The changes of fluorescence were recorded for 1 h (60 minutes) by an Olympus IX71 microscope at 4 × objective. The obtained time-lapse fluorescence images were merged with ImageJ software to provide the video. Meanwhile, the photos at very 5 min interval were taken and the relative fluorescence intensity was semi-quantitative analyzed with ImageJ software. ROI module in ImageJ software was used to analyze the fluorescence intensity in brain, yolk and wound areas.

Figure S13. a) Time-dependent fluorescence imaging of brain, yolk and wound areas in wound larval zebrafish with C7H (10 µM). Scale bar, 500 µm. b) Time-dependent averaged fluorescence intensity changes of brain and yolk section in wound larval zebrafish with C7H. Data was from semi-quantitative detection of fluorescence intensity according to photos in a) with ImageJ software. The wound larval zebrafish was prepared by scratching the waist gently with a sharp surgical blade to induce an acute wound, then the wound larval zebrafish was immersed in C7H and observed immediately for continuous 60 min using an Olympus IX71 microscope with a 4X objective.

4.4. Exploration of wound healing with C7H in larval zebrafish

The stock solution of C7H was diluted to 10 μM with PBS buffer (0.2 M, pH 7.4) for further incubation. Then the well-behaved larval zebrafishes were selected and their waists were scratched gently with a sharp surgical blade to induce an acute wound under the observation of Nikon stereoscopic microscope SMZ745T. To conduct the wound healing experiments, a wound larval zebrafish was immediately treated with 2 mL of prepared PBS buffer containing C7H after the induction of wound on the waist. The changes of fluorescence were recorded for 30 min by an Olympus IX71 microscope at 4 × objective, then larval zebrafish was washed with fresh PBS (0.2 M, pH 7.4) without C7H (this procedure last about 5 min). Then the larval zebrafish was transferred to the fresh PBS (0.2 M, pH 7.4) without C7H and incubated for another 60 min. The changes of fluorescence were recorded for 30 min by an Olympus IX71 microscope at 4 × objective. Above procedure was recorded as first-round observation of wound healing. Then, the zebrafish was transferred to 2 mL of prepared PBS buffer containing C7H again, and incubation for 30 min. The changes of fluorescence were recorded by an Olympus IX71 microscope at 4 × objective. Then the larval zebrafish was washed and transferred to fresh PBS (0.2 M, pH 7.4) without C7H and incubated for another 60 min. The changes of fluorescence were recorded by an Olympus IX71 microscope at 4 × objective. We record this procedure as the second-round procedure. Finally, the third-round procedure was conducted as above two procedures. The obtained time-lapse fluorescence images were merged with ImageJ software to provide the video. Meanwhile, the photos at very 5 min interval were taken and the relative fluorescence intensity was semi-quantitative analyzed with ImageJ software.

Figure S14. Wound healing investigation with C7H in larval zebrafish. a) Wound healing investigation according to the time-dependent fluorescence changes of wound area in larval zebrafish. b) Wound healing investigation according to the time-dependent averaged fluorescence intensity changes in wound larval zebrafish. Scale bar, 100 µm. Data represent mean values of fluorescence intensity ± s.d. (n = 3 fishes).

5. Movies

Movie S1. The dynamic fluorescence changes of larval zebrafish treated with C7H.

Movie S2. The dynamic fluorescence changes of larval zebrafish treated with Con-C7H.

Movie S3. The dynamic fluorescence changes of wound larval zebrafish treated with C7H.

Movie S4. The dynamic fluorescence changes of wound larval zebrafish treated with Con-C7H.

Movie S5. Exploration of wound healing with C7H in larval zebrafish.

6. NMR spectrum and HRMS

Figure S15. 1H NMR spectrum of compound 1.

Figure S16. 13C NMR spectrum of compound 1.

Figure S17. 1H NMR spectrum of compound 2.

Figure S18. 13C NMR spectrum of compound 2.

Figure S19. 1H NMR spectrum of compound Cy7-Cl.

Figure S20. 13C NMR spectrum of compound Cy7-Cl.

Figure S21. HRMS of compound Cy7-Cl.

Figure S22. 1H NMR spectrum of compound Cy7-OH.

Figure S23. 13C NMR spectrum of compound Cy7-OH.

Figure S24. HRMS of compound Cy7-OH.

Figure S25. 1H NMR spectrum of probe C7H.

Figure S26. 13C NMR spectrum of probe C7H.

Figure S27. Maldi-Tof/Tof-MS spectrum of probe C7H.

Figure S28. 1H NMR spectrum of compound Con-C7H.

Figure S29. 13C NMR spectrum of compound Con-C7H.

Figure S30. HRMS of compound Con-C7H.

Reference

(1) Wu, J.; Jiang, X.; Xu, J.; Dai, W.-M. Tandem Wittig–intramolecular Diels–Alder cycloaddition of ester-tethered 1,3,9-decatrienes under microwave heating. Tetrahedron 2011, 67, 179-192.

(2) Cheng, G.; Fan, J.; Sun, W.; Cao, J.; Hu, C.; Peng, X. A near-infrared fluorescent probe for selective detection of HClO based on Se-sensitized aggregation of heptamethine cyanine dye. Chem. Commun. 2014, 50, 1018-1020.