Appendix 1:  Concentrations of uranium in stream sediments, stream waters

Examples show the distribution of uranium in over 100,000 samples of stream sediment and 50,000 samples of stream water at a typical resolution of 1 sample per 2.5 km2 (sediments).  These data clearly show the linkage between geological features  such as granitic rocks and elevated concentrations in stream sediment  and the lack of spatial correlation between concentrations in stream waters and stream sediments, due to differences in uranium mobility resulting from changes in uranium mineralisation and water chemistry.   Licensed discharges from Springfield and Capenhurst can also be clearly identified.

Appendix 2: Isotopic analysis of uranium from areas surrounding Springfields and Capenhurst

A bi-variate plot of uranium isotopic ratios in stream sediment samples from N.E. England. The diagram shows samples from natural and industrial sites with anomalously high uranium concentrations have a natural fixed 238/235U ratio but natural variable 238/234U ratio. But samples associated geographically with nuclear processing sites follow a mixed array between natural and fuel cycle compositions.