Magnetic fluidized bed for solid phase extraction in microfluidic systems

Iago Pereiroabc†, Sanae Tabnaouiab†, Marc Fermigierd, Olivia du Roured, Stéphanie Descroixabc, Jean-Louis Viovyabc, Laurent Malaquinabc*

aLaboratoire Physico Chimie Curie, Institut Curie, PSL Research University, CNRS UMR168, 75005, Paris, France

bSorbonne Universités, UPMC Univ Paris 06, 75005, Paris, France

cInstitut Pierre-Gilles de Gennes, 75005, Paris, France

dPhysique et Mécanique des Milieux Hétérogènes, ESPCI-ParisTech-CNRS, Paris, France

†  These authors contributed equally to this work

*Current address : CNRS, LAAS, 7 avenue du colonel Roche, F-31400 Toulouse, France

Supplementary information

SI-1. Experimental setup

Figure S1. Diagram of the experimental setup (a). Picture of the PDMS fluidized bed device (b) and fluidic connection  system (c). (scale bar 1cm)

SI-2. Finite element simulations

Finite element 3D numerical simulations of the magnetic field were performed using COMSOL Multiphysics v4.3b (COMSOL, Inc.,Burlington, MA ). The ‘Magnetic Field, no current’ module was used to investigate the magnetic field intensity in the fluidized bed and evaluate the corresponding magnetic forces exerted on the particles. The device geometry was defined according to the design shown in Figure S2. The permanent magnetization of the magnet was set to M = 1170 kA.m-1 (Br = 1,47T) according to the manufacturer’s datasheet. The 3D design of the magnet was set according to the magnet dimensions (30x20x20 mm). A fillet was applied to the structure geometry with a 2 mm value. A spherical volume (10 cm diameter) was designed around the magnet to define the magnetic insulation parameters and set the initial values of the Magnetic Scalar potential (Vm = 0) around the magnet. Magnetic properties of air were used to define the surrounding medium properties.  The mesh parameters were set to ‘Physics controlled mesh’ with an ‘extremely fine’ element size. Simulation were done in stationary conditions by compiling the following equations:

i ) ∇. (μ0μr H)= 0

ii) H = -∇Vm

with μ0 and µr the permittivity of vacuum and relative permittivity of the considered material respectively, H the magnetic field and Vm the magnetic potential. Stationary solver provided a converging solution in 60 s (31 linear iterations, 2099470 degrees of freedom solved) (Figure 2).

Figure S2.  Magnetic field streamlines  inside the microfluidic chamber (white traits) and the associated magnetic field strength (finite element COMSOL Multiphysics®) (a). 3D magnetic field line distribution around the permanent magnet  (b).

Hydrodynamic simulations were performed using COMSOL Multiphysics v4.3b (COMSOL, Inc.,Burlington, MA ). Numerical calculations were performed in stationary mode. The 3D geometry of the devices was defined according to the design shown in Figure 1. The ‘Laminar Flow’ module was used to solve the Navier–Stokes equation for incompressible flow given as :

ρ ∂v/∂t+ρ(v.∇)v=-∇P+μ∇2 v+F

in which v is the flow velocity, P is the pressure, ρ is the fluid density, μ is the dynamic viscosity and F is the volume force.  Water was used as a reference material for simulations.  No slip boundary conditions were applied to the channel walls. A uniform flow velocity was set at the channel inlet while a zero pressure condition was set to the channel outlet. A free tetrahedral mesh was used with element size ranging from 24,8µm to 4,6 µm (maximum element growth rate: 1,13, resolution of curvature : 0,5).  Stationary solver provided a converging solution in 85 s (7 linear iterations, 318724 degrees of freedom solved).

Fig. S3. Typical flow intensity distribution inside the 50 µm high microfluidic v-shaped chamber for a 3 µL/min flow rate (finite element simulation with COMSOL Multiphysics®). The profile of velocities is relatively flat across the width of the channel. Velocities decrease towards the output (right side) from 2 mm/s on left to 1 mm/s right.

SI-3. Magnetic field distribution and influence on magnetic bead interactions

Figure S4. Measured angle for an independent and straight 13-beads column inside the microfluidic chamber. Angle measurements were obtained by focusing on each terminal bead of the column. The angle of the column is compared with the angle of the magnetic field streamlines obtained from finite elements simulation, showing a strong correlation (scale bar = 10µm).

Figure S5. x- axis component of the magnetic field along the main axis of the chamber (finite element simulation, COMSOL Multiphysics®), and the magnetic force applied on the magnetic beads as a result of the gradient of this field.

Figure S6. y-axis component of the magnetic field in a section of the chamber at 4 mm from the entrance obtained by finite element simulation with COMSOL Multiphysics®.

SI-4. Pressure drop as a function of flow-rate

Figure S7. Experimentally measured pressured drop compared to pressure drop values obtained with expression (1) as a function of the imposed flow-rate for three different loadings of beads.