Supplementary material to

PCCP Special Issue on the 76th Bunsen Discussion Meeting Global Phase Diagrams, August 19–22, 2001, Walberberg, Germany

Temperature, pressure and density dependencies of the solubilities of low-volatile organic compounds in compressed gases. Part 1: Solution energies from solubility data for disperse dyestuffs up to 20 MPa

by

Dirk Tuma a, Björn Wagner b and Gerhard M. Schneider a *

a Department of Chemistry, Physical Chemistry, University of Bochum, D-44780 Bochum, Germany,

E-mail: gerhard.schneider@ruhr-uni-bochum.de

b Accumulatorenwerke Hoppecke Carl Zoellner & Sohn GmbH & Co KG, Abteilung GTB, Bontkirchener Straße 2, D-59929 Brilon, Germany

# Paper Ref.: B109521H

Table I		Numerical values for the isobaric thermal expansivity αp of pure carbon dioxide in the p,T-range relevant for the experiments discussed in the paper; calculated from: R. Span and W. Wagner, J. Phys. Chem. Ref. Data, 1996, 25, 1509.

αpº1V¶V¶Tp=-1ρ¶ρ¶Tp

| T / K | p / MPa | ρ / kg m–3 | αp / K–1 |
| --- | --- | --- | --- |
| 299 | 8.5 | 779.778 | 0.014258 |
|  | 9 | 791.285 | 0.012713 |
|  | 10 | 810.405 | 0.010651 |
|  | 11 | 826.115 | 0.009311 |
|  | 12 | 839.562 | 0.008356 |
|  | 13 | 851.387 | 0.007634 |
|  | 14 | 861.982 | 0.007065 |
|  | 15 | 871.612 | 0.006602 |
|  | 16 | 880.459 | 0.006217 |
|  | 17 | 888.658 | 0.005890 |
|  | 18 | 896.310 | 0.005608 |
|  | 18.5 | 899.956 | 0.005481 |
| 305 | 8.5 | 694.533 | 0.027873 |
|  | 9 | 718.624 | 0.020969 |
|  | 10 | 751.672 | 0.014960 |
|  | 11 | 775.360 | 0.012078 |
|  | 12 | 794.152 | 0.010324 |
|  | 13 | 809.870 | 0.009122 |
|  | 14 | 823.459 | 0.008238 |
|  | 15 | 835.478 | 0.007556 |
|  | 16 | 846.289 | 0.007010 |
|  | 17 | 856.137 | 0.006562 |
|  | 18 | 865.199 | 0.006187 |
|  | 18.5 | 869.477 | 0.006020 |
| 310 | 8.5 | 514.870 | 0.127640 |
|  | 9 | 614.873 | 0.049381 |
|  | 10 | 685.773 | 0.022954 |
|  | 11 | 723.376 | 0.016083 |
|  | 12 | 749.846 | 0.012820 |
|  | 13 | 770.587 | 0.010865 |
|  | 14 | 787.780 | 0.009540 |
|  | 15 | 802.540 | 0.008574 |
|  | 16 | 815.520 | 0.007832 |
|  | 17 | 827.137 | 0.007242 |
|  | 18 | 837.676 | 0.006760 |
|  | 18.5 | 842.606 | 0.006550 |
| 315 | 10 | 586.022 | 0.042882 |
|  | 11 | 656.533 | 0.023492 |
|  | 12 | 696.993 | 0.016733 |
|  | 13 | 725.644 | 0.013326 |
|  | 14 | 748.026 | 0.011257 |
|  | 15 | 766.506 | 0.009854 |
|  | 16 | 782.310 | 0.008832 |
|  | 17 | 796.161 | 0.008049 |
|  | 18 | 808.519 | 0.007427 |
|  | 18.5 | 814.240 | 0.007162 |
| 320 | 11 | 567.350 | 0.035359 |
|  | 12 | 632.211 | 0.022609 |
|  | 13 | 673.339 | 0.016769 |
|  | 14 | 703.266 | 0.013524 |
|  | 15 | 726.830 | 0.011467 |
|  | 16 | 746.322 | 0.010046 |
|  | 17 | 762.988 | 0.009001 |
|  | 18 | 777.581 | 0.008198 |
|  | 18.5 | 784.258 | 0.007861 |
| 325 | 11 | 466.630 | 0.039581 |
|  | 12 | 555.056 | 0.029137 |
|  | 13 | 612.816 | 0.020960 |
|  | 14 | 652.866 | 0.016276 |
|  | 15 | 683.088 | 0.013403 |
|  | 16 | 707.279 | 0.011479 |
|  | 17 | 727.444 | 0.010105 |
|  | 18 | 744.750 | 0.009076 |
|  | 18.5 | 752.569 | 0.008652 |
| 330 | 12 | 476.555 | 0.030487 |
|  | 13 | 546.534 | 0.024424 |
|  | 14 | 597.523 | 0.019089 |
|  | 15 | 635.507 | 0.015471 |
|  | 16 | 665.242 | 0.013034 |
|  | 17 | 689.531 | 0.011312 |
|  | 18 | 710.016 | 0.010037 |
|  | 18.5 | 719.166 | 0.009517 |
| 335 | 12 | 413.278 | 0.025983 |
|  | 13 | 482.606 | 0.024666 |
|  | 14 | 540.301 | 0.020860 |
|  | 15 | 585.400 | 0.017267 |
|  | 16 | 620.897 | 0.014521 |
|  | 17 | 649.640 | 0.012508 |
|  | 18 | 673.610 | 0.011009 |
|  | 18.5 | 684.228 | 0.010398 |
| 340 | 12 | 367.787 | 0.020876 |
|  | 13 | 429.140 | 0.022004 |
|  | 14 | 486.608 | 0.020651 |
|  | 15 | 535.555 | 0.018108 |
|  | 16 | 575.688 | 0.015605 |
|  | 17 | 608.620 | 0.013527 |
|  | 18 | 636.082 | 0.011892 |
|  | 18.5 | 648.204 | 0.011214 |
| 345 | 13 | 387.675 | 0.018692 |
|  | 14 | 440.617 | 0.018892 |
|  | 15 | 489.417 | 0.017726 |
|  | 16 | 531.838 | 0.015938 |
|  | 17 | 567.820 | 0.014135 |
|  | 18 | 598.292 | 0.012552 |
|  | 18.5 | 611.805 | 0.011861 |

Figure A	Plot isobaric thermal expansivity αp of CO2 vs. pressure at constant temperatures.

Figure B	Plot isobaric thermal expansivity αp of CO2 vs. density at constant temperatures.

Table II	AQ01 + CO2; data for the ln (s/s+) vs. T–1 plot at p = const. (Fig. 2). The data have been fitted using Jandel’s TableCurve™ 2D v4.03 software. Numbers in parentheses are extrapolated.

| p / MPa | s / 10–6 mol dm–3 |  | p / MPa |  | s / 10–6 mol dm–3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| T = 299 K |  |  | T = 305 K |  |  |  |
| 8.5 |  | 11.183 |  | 8.5 |  | 7.421 |
| 9 |  | 12.390 |  | 9 |  | 9.252 |
| 10 |  | 14.673 |  | 10 |  | 12.436 |
| 11 |  | 16.840 |  | 11 |  | 15.327 |
| 12 |  | 18.931 |  | 12 |  | 18.080 |
| 13 |  | 20.969 |  | 13 |  | 20.764 |
| 14 |  | 22.967 |  | 14 |  | 23.412 |
| 15 |  | 24.936 |  | 15 |  | 26.045 |
| 16 |  | 26.881 |  | 16 |  | 28.675 |
| 17 |  | 28.808 |  | 17 |  | 31.310 |
| 18 |  | 30.720 |  | 18 |  | 33.955 |
| 18.5 |  | 31.672 |  | 18.5 |  | 35.283 |
| T = 310 K |  |  | T = 315 K |  |  |  |
| 8.5 |  | 2.099 |  |  |  |  |
| 9 |  | 4.343 |  |  |  |  |
| 10 |  | 8.636 |  | 10 |  | 4.236 |
| 11 |  | 12.693 |  | 11 |  | 8.859 |
| 12 |  | 16.540 |  | 12 |  | 13.367 |
| 13 |  | 20.200 |  | 13 |  | 17.773 |
| 14 |  | 23.692 |  | 14 |  | 22.085 |
| 15 |  | 27.032 |  | 15 |  | 26.310 |
| 16 |  | 30.235 |  | 16 |  | 30.454 |
| 17 |  | 33.312 |  | 17 |  | 34.524 |
| 18 |  | 36.274 |  | 18 |  | 38.525 |
| 18.5 |  | 37.715 |  | 18.5 |  | 40.500 |
| T = 320 K |  |  | T = 325 K |  |  |  |
| 11 |  | 4.937 |  | (11) |  | (1.618) |
| 12 |  | 9.522 |  | 12 |  | 5.226 |
| 13 |  | 14.217 |  | 13 |  | 9.973 |
| 14 |  | 19.015 |  | 14 |  | 15.352 |
| 15 |  | 23.908 |  | 15 |  | 21.054 |
| 16 |  | 28.890 |  | 16 |  | 26.887 |
| 17 |  | 33.955 |  | 17 |  | 32.731 |
| 18 |  | 39.099 |  | 18 |  | 38.511 |
| 18.5 |  | 41.698 |  | 18.5 |  | 41.361 |
| T = 330 K |  |  | T = 335 K |  |  |  |
| 12 |  | 2.625 |  | (12) |  | (0.968) |
| 13 |  | 6.594 |  | 13 |  | 3.782 |
| 14 |  | 11.706 |  | 14 |  | 8.034 |
| 15 |  | 17.590 |  | 15 |  | 13.416 |
| 16 |  | 23.983 |  | 16 |  | 19.692 |
| 17 |  | 30.691 |  | 17 |  | 26.681 |
| 18 |  | 37.577 |  | 18 |  | 34.241 |
| 18.5 |  | 41.053 |  | 18.5 |  | 38.198 |
| T = 340 K |  |  | T = 345 K |  |  |  |
| (12) |  | (0.339) |  |  |  |  |
| 13 |  | 2.246 |  | (13) |  | (1.030) |
| 14 |  | 5.667 |  | 14 |  | 3.516 |
| 15 |  | 10.442 |  | 15 |  | 7.432 |
| 16 |  | 16.439 |  | 16 |  | 12.734 |
| 17 |  | 23.543 |  | 17 |  | 19.384 |
| 18 |  | 31.659 |  | 18 |  | 27.343 |
| 18.5 |  | 36.070 |  | 18.5 |  | 31.802 |

Table III	AQ01 + CO2; data for the ln (s/s+) vs. T–1 plot at ρ = const. (Fig. 3). The data have been fitted using Jandel’s TableCurve™ 2D v4.03 software. Numbers in parentheses are extrapolated.

| ρ / kg m–3 | s / 10–6 mol dm–3 |  | ρ / kg m–3 |  | s / 10–6 mol dm–3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| T = 299 K |  |  | T = 305 K |  |  |  |
|  |  |  |  | (550) |  | (1.776) |
|  |  |  |  | (575) |  | (2.289) |
|  |  |  |  | 600 |  | 2.939 |
|  |  |  |  | 625 |  | 3.759 |
|  |  |  |  | 650 |  | 4.789 |
|  |  |  |  | 675 |  | 6.080 |
| (700) |  | (5.392) |  | 700 |  | 7.693 |
| 725 |  | 6.813 |  | 725 |  | 9.703 |
| 750 |  | 8.575 |  | 750 |  | 12.202 |
| 775 |  | 10.751 |  | 775 |  | 15.298 |
| 800 |  | 13.430 |  | 800 |  | 19.128 |
| 825 |  | 16.720 |  | 825 |  | 23.852 |
| 850 |  | 20.746 |  | 850 |  | 29.666 |
| 875 |  | 25.662 |  | (875) |  | (36.807) |
| 900 |  | 31.646 |  | (900) |  | (45.559) |
| (925) |  | (38.914) |  |  |  |  |
| T = 310 K |  |  | T = 315 K |  |  |  |
| (550) |  | (2.265) |  | (550) |  | (2.911) |
| (575) |  | (2.957) |  | 575 |  | 3.826 |
| 600 |  | 3.838 |  | 600 |  | 5.000 |
| 625 |  | 4.956 |  | 625 |  | 6.498 |
| 650 |  | 6.366 |  | 650 |  | 8.401 |
| 675 |  | 8.138 |  | 675 |  | 10.808 |
| 700 |  | 10.358 |  | 700 |  | 13.841 |
| 725 |  | 13.127 |  | 725 |  | 17.647 |
| 750 |  | 16.568 |  | 750 |  | 22.406 |
| 775 |  | 20.831 |  | 775 |  | 28.337 |
| 800 |  | 26.096 |  | 800 |  | 35.703 |
| 825 |  | 32.577 |  | (825) |  | (44.824) |
| (850) |  | (40.533) |  | (850) |  | (56.082) |
| (875) |  | (50.270) |  |  |  |  |
| T = 320 K |  |  | T = 325 K |  |  |  |
| (500) |  | (2.293) |  | (500) |  | (2.566) |
| (525) |  | (3.023) |  | (525) |  | (3.542) |
| 550 |  | 3.966 |  | 550 |  | 4.829 |
| 575 |  | 5.178 |  | 575 |  | 6.507 |
| 600 |  | 6.732 |  | 600 |  | 8.675 |
| 625 |  | 8.716 |  | 625 |  | 11.450 |
| 650 |  | 11.240 |  | 650 |  | 14.975 |
| 675 |  | 14.442 |  | 675 |  | 19.416 |
| 700 |  | 18.490 |  | 700 |  | 24.974 |
| 725 |  | 23.595 |  | 725 |  | 31.883 |
| 750 |  | 30.011 |  | 750 |  | 40.417 |
| 775 |  | 38.057 |  | (775) |  | (50.898) |
| (800) |  | (48.118) |  | (800) |  | (63.699) |
| T = 330 K |  |  | T = 335 K |  |  |  |
| (400) |  | (0.856) |  | (400) |  | (1.144) |
| (425) |  | (1.255) |  | (425) |  | (1.675) |
| (450) |  | (1.808) |  | (450) |  | (2.407) |
| 475 |  | 2.561 |  | 475 |  | 3.403 |
| 500 |  | 3.574 |  | 500 |  | 4.740 |
| 525 |  | 4.920 |  | 525 |  | 6.512 |
| 550 |  | 6.688 |  | 550 |  | 8.836 |
| 575 |  | 8.986 |  | 575 |  | 11.852 |
| 600 |  | 11.948 |  | 600 |  | 15.731 |
| 625 |  | 15.729 |  | 625 |  | 20.677 |
| 650 |  | 20.519 |  | 650 |  | 26.931 |
| 675 |  | 26.541 |  | 675 |  | 34.782 |
| 700 |  | 34.058 |  | (700) |  | (44.569) |
| (725) |  | (43.381) |  | (725) |  | (56.688) |
| (750) |  | (54.871) |  |  |  |  |
| T = 340 K |  |  | T = 345 K |  |  |  |
| (375) |  | (1.075) |  | (375) |  | (1.257) |
| (400) |  | (1.600) |  | (400) |  | (1.925) |
| 425 |  | 2.335 |  | (425) |  | (2.872) |
| 450 |  | 3.346 |  | 450 |  | 4.189 |
| 475 |  | 4.718 |  | 475 |  | 5.986 |
| 500 |  | 6.554 |  | 500 |  | 8.399 |
| 525 |  | 8.981 |  | 525 |  | 11.592 |
| 550 |  | 12.157 |  | 550 |  | 15.760 |
| 575 |  | 16.270 |  | 575 |  | 21.136 |
| 600 |  | 21.547 |  | 600 |  | 27.994 |
| 625 |  | 28.260 |  | (625) |  | (36.655) |
| 650 |  | 36.732 |  | (650) |  | (47.490) |
| (675) |  | (47.344) |  |  |  |  |
| (700) |  | (60.547) |  |  |  |  |

Table IV	Molar solution energies for AQ01 in CO2 at ten different temperatures; ¶lns2s+/¶lnρρ+T is abbreviated as k (cf. Fig. 4)

| p / MPa | ΔsolEρ / kJ mol–1 a |  | ΔsolEp / kJ mol–1 b |  | Δ(ΔE) / kJ mol–1 b |  |
| --- | --- | --- | --- | --- | --- | --- |
| T = 299 K, k = 7.0604 |  |  |  |  |  |  |
| 8.5 |  | 48.6 |  | –26.2 |  | 74.8 |
| 9 |  | 48.5 |  | –18.2 |  | 66.7 |
| 10 |  | 48.3 |  | –7.6 |  | 55.9 |
| 11 |  | 48.2 |  | –0.7 |  | 48.9 |
| 12 |  | 48.1 |  | 4.2 |  | 43.9 |
| 13 |  | 48.0 |  | 7.9 |  | 40.1 |
| 14 |  | 47.9 |  | 10.8 |  | 37.1 |
| 15 |  | 47.8 |  | 13.2 |  | 34.6 |
| 16 |  | 47.8 |  | 15.1 |  | 32.6 |
| 17 |  | 47.7 |  | 16.8 |  | 30.9 |
| 18 |  | 47.6 |  | 18.2 |  | 29.4 |
| 18.5 |  | 47.6 |  | 18.8 |  | 28.8 |
| T = 305 K, k = 6.4637 |  |  |  |  |  |  |
| 8.5 |  | 49.4 |  | –89.9 |  | 139.3 |
| 9 |  | 49.2 |  | –55.7 |  | 104.8 |
| 10 |  | 48.9 |  | –25.9 |  | 74.8 |
| 11 |  | 48.6 |  | –11.7 |  | 60.4 |
| 12 |  | 48.5 |  | –3.1 |  | 51.6 |
| 13 |  | 48.3 |  | 2.7 |  | 45.6 |
| 14 |  | 48.2 |  | 7.0 |  | 41.2 |
| 15 |  | 48.1 |  | 10.4 |  | 37.8 |
| 16 |  | 48.0 |  | 13.0 |  | 35.0 |
| 17 |  | 48.0 |  | 15.2 |  | 32.8 |
| 18 |  | 47.9 |  | 17.0 |  | 30.9 |
| 18.5 |  | 47.8 |  | 17.8 |  | 30.1 |
| T = 310 K, k = 6.6809 |  |  |  |  |  |  |
| 8.5 |  | 51.4 |  | –629.9 |  | 681.4 |
| 9 |  | 50.2 |  | –213.4 |  | 263.6 |
| 10 |  | 49.5 |  | –73.0 |  | 122.5 |
| 11 |  | 49.1 |  | –36.7 |  | 85.9 |
| 12 |  | 48.9 |  | –19.6 |  | 68.4 |
| 13 |  | 48.7 |  | –9.3 |  | 58.0 |
| 14 |  | 48.5 |  | –2.4 |  | 50.9 |
| 15 |  | 48.4 |  | 2.6 |  | 45.8 |
| 16 |  | 48.3 |  | 6.5 |  | 41.8 |
| 17 |  | 48.2 |  | 9.5 |  | 38.7 |
| 18 |  | 48.1 |  | 12.0 |  | 36.1 |
| 18.5 |  | 48.1 |  | 13.1 |  | 35.0 |
| T = 315 K, k = 6.8205 |  |  |  |  |  |  |
| 10 |  | 50.6 |  | –190.7 |  | 241.3 |
| 11 |  | 49.8 |  | –82.4 |  | 132.2 |
| 12 |  | 49.4 |  | –44.8 |  | 94.2 |
| 13 |  | 49.1 |  | –25.9 |  | 75.0 |
| 14 |  | 48.9 |  | –14.4 |  | 63.3 |
| 15 |  | 48.7 |  | –6.7 |  | 55.4 |
| 16 |  | 48.6 |  | –1.1 |  | 49.7 |
| 17 |  | 48.5 |  | 3.2 |  | 45.3 |
| 18 |  | 48.4 |  | 6.6 |  | 41.8 |
| 18.5 |  | 48.3 |  | 8.0 |  | 40.3 |
| T = 320 K, k = 6.6360 |  |  |  |  |  |  |
| 11 |  | 50.8 |  | –149.0 |  | 199.8 |
| 12 |  | 50.1 |  | –77.7 |  | 127.7 |
| 13 |  | 49.6 |  | –45.1 |  | 94.7 |
| 14 |  | 49.3 |  | –27.1 |  | 76.4 |
| 15 |  | 49.1 |  | –15.7 |  | 64.8 |
| 16 |  | 48.9 |  | –7.8 |  | 56.8 |
| 17 |  | 48.8 |  | –2.1 |  | 50.9 |
| 18 |  | 48.6 |  | 2.3 |  | 46.3 |
| 18.5 |  | 48.6 |  | 4.2 |  | 44.4 |
| T = 325 K, k = 6.7703 |  |  |  |  |  |  |
| 11 |  | 52.1 |  | –183.2 |  | 235.3 |
| 12 |  | 50.9 |  | –122.3 |  | 173.2 |
| 13 |  | 50.3 |  | –74.4 |  | 124.6 |
| 14 |  | 49.8 |  | –46.9 |  | 96.8 |
| 15 |  | 49.5 |  | –30.2 |  | 79.7 |
| 16 |  | 49.3 |  | –19.0 |  | 68.3 |
| 17 |  | 49.1 |  | –11.0 |  | 60.1 |
| 18 |  | 48.9 |  | –5.0 |  | 54.0 |
| 18.5 |  | 48.9 |  | –2.6 |  | 51.4 |
| T = 330 K, k = 6.5769 |  |  |  |  |  |  |
| 12 |  | 52.0 |  | –129.6 |  | 181.5 |
| 13 |  | 51.0 |  | –94.4 |  | 145.4 |
| 14 |  | 50.4 |  | –63.2 |  | 113.7 |
| 15 |  | 50.0 |  | –42.1 |  | 92.1 |
| 16 |  | 49.7 |  | –27.9 |  | 77.6 |
| 17 |  | 49.5 |  | –17.9 |  | 67.4 |
| 18 |  | 49.3 |  | –10.5 |  | 59.8 |
| 18.5 |  | 49.2 |  | –7.5 |  | 56.7 |
| T = 335 K, k = 6.6058 |  |  |  |  |  |  |
| 12 |  | 52.9 |  | –107.2 |  | 160.2 |
| 13 |  | 51.9 |  | –100.2 |  | 152.0 |
| 14 |  | 51.1 |  | –77.5 |  | 128.6 |
| 15 |  | 50.6 |  | –55.9 |  | 106.4 |
| 16 |  | 50.2 |  | –39.3 |  | 89.5 |
| 17 |  | 49.9 |  | –27.2 |  | 77.1 |
| 18 |  | 49.6 |  | –18.2 |  | 67.9 |
| 18.5 |  | 49.5 |  | –14.6 |  | 64.1 |
| T = 340 K, k = 6.5069 |  |  |  |  |  |  |
| 12 |  | 53.7 |  | –84.9 |  | 130.6 |
| 13 |  | 52.7 |  | –77.3 |  | 137.6 |
| 14 |  | 51.8 |  | –76.9 |  | 129.2 |
| 15 |  | 51.2 |  | –62.1 |  | 113.3 |
| 16 |  | 50.7 |  | –46.9 |  | 97.6 |
| 17 |  | 50.3 |  | –34.3 |  | 84.6 |
| 18 |  | 50.0 |  | –24.4 |  | 74.4 |
| 18.5 |  | 49.9 |  | –20.3 |  | 70.1 |
| T = 345 K, k = 6.6868 |  |  |  |  |  |  |
| 13 |  | 53.4 |  | –72.5 |  | 123.7 |
| 14 |  | 52.5 |  | –70.3 |  | 125.0 |
| 15 |  | 51.8 |  | –65.5 |  | 117.3 |
| 16 |  | 51.2 |  | –54.2 |  | 105.5 |
| 17 |  | 50.8 |  | –42.8 |  | 93.5 |
| 18 |  | 50.4 |  | –32.6 |  | 83.1 |
| 18.5 |  | 50.3 |  | –28.2 |  | 78.5 |
| a ΔsolEρ-values are adjusted by a best-fit function and data belonging to p calculated therefrom. The corresponding density values are obtained using the EOS by Span and Wagner (ref. 8). b Calculated using the right side of eq. (6). |  |  |  |  |  |  |

Table V	AQiso03 + CO2; data for the ln (s/s+) vs. T–1 plot at p = const. (Fig. 9). The data have been fitted using Jandel’s TableCurve™ 2D v4.03 software. Numbers in parentheses are extrapolated.

| p / MPa | s / 10–6 mol dm–3 |  | p / MPa |  | s / 10–6 mol dm–3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| T = 305 K |  |  | T = 310 K |  |  |  |
| 9 |  | 103.445 |  | 9 |  | 51.051 |
| 10 |  | 144.845 |  | 10 |  | 104.468 |
| 12 |  | 221.067 |  | 12 |  | 203.818 |
| 14 |  | 290.040 |  | 14 |  | 295.180 |
| 16 |  | 353.142 |  | 16 |  | 380.218 |
| T = 315 K |  |  | T = 320 K |  |  |  |
| (9) |  | (1.345) |  |  |  |  |
| 10 |  | 53.400 |  | 10 |  | 13.585 |
| 12 |  | 159.432 |  | 12 |  | 113.556 |
| 14 |  | 268.077 |  | 14 |  | 244.458 |
| 16 |  | 379.340 |  | 16 |  | 375.172 |
| T = 330 K |  |  | T = 340 K |  |  |  |
| 12 |  | 31.720 |  | 12 |  | 8.804 |
| 14 |  | 143.681 |  | 14 |  | 66.972 |
| 16 |  | 305.947 |  | 16 |  | 201.718 |

Table VI	AQiso03 + CO2; data for the ln (s/s+) vs. T–1 plot at ρ = const. (Fig. 10). The data have been fitted using Jandel’s TableCurve™ 2D v4.03 software. Numbers in parentheses are extrapolated.

| ρ / kg m–3 | s / 10–6 mol dm–3 |  | ρ / kg m–3 |  | s / 10–6 mol dm–3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| T = 305 K |  |  | T = 310 K |  |  |  |
|  |  |  |  | (425) |  | (6.265) |
|  |  |  |  | (450) |  | (8.436) |
|  |  |  |  | 475 |  | 11.289 |
|  |  |  |  | 500 |  | 15.020 |
|  |  |  |  | 525 |  | 19.878 |
|  |  |  |  | 550 |  | 26.176 |
|  |  |  |  | 575 |  | 34.308 |
|  |  |  |  | 600 |  | 44.766 |
| (625) |  | (41.669) |  | 625 |  | 58.167 |
| 650 |  | 53.956 |  | 650 |  | 75.281 |
| 675 |  | 69.523 |  | 675 |  | 97.065 |
| 700 |  | 89.165 |  | 700 |  | 124.705 |
| 725 |  | 113.855 |  | 725 |  | 159.672 |
| 750 |  | 144.775 |  | 750 |  | 203.780 |
| 775 |  | 183.362 |  | 775 |  | 259.267 |
| 800 |  | 231.357 |  | 800 |  | 328.885 |
| 825 |  | 290.864 |  | 825 |  | 416.011 |
| 850 |  | 364.420 |  | (850) |  | (524.785) |
| (875) |  | (455.078) |  |  |  |  |
| (900) |  | (566.500) |  |  |  |  |
| T = 315 K |  |  | T = 320 K |  |  |  |
|  |  |  |  | (375) |  | (4.909) |
|  |  |  |  | (400) |  | (6.940) |
| (425) |  | (7.282) |  | 425 |  | 9.709 |
| 450 |  | 10.039 |  | 450 |  | 13.449 |
| 475 |  | 13.717 |  | 475 |  | 18.466 |
| 500 |  | 18.591 |  | 500 |  | 25.146 |
| 525 |  | 25.009 |  | 525 |  | 33.982 |
| 550 |  | 33.408 |  | 550 |  | 45.598 |
| 575 |  | 44.339 |  | 575 |  | 60.782 |
| 600 |  | 58.489 |  | 600 |  | 80.523 |
| 625 |  | 76.716 |  | 625 |  | 106.059 |
| 650 |  | 100.082 |  | 650 |  | 138.933 |
| 675 |  | 129.907 |  | 675 |  | 181.062 |
| 700 |  | 167.813 |  | 700 |  | 234.821 |
| 725 |  | 215.800 |  | 725 |  | 303.144 |
| 750 |  | 276.319 |  | 750 |  | 389.642 |
| 775 |  | 352.367 |  | (775) |  | (498.745) |
| (800) |  | (447.599) |  |  |  |  |
| (825) |  | (566.464) |  |  |  |  |
| T = 330 K |  |  | T = 340 K |  |  |  |
| (350) |  | (3.849) |  | (350) |  | (6.328) |
| (375) |  | (6.029) |  | 375 |  | 11.276 |
| 400 |  | 9.220 |  | 400 |  | 17.957 |
| 425 |  | 13.798 |  | 425 |  | 26.977 |
| 450 |  | 20.253 |  | 450 |  | 39.156 |
| 475 |  | 29.216 |  | 475 |  | 55.599 |
| 500 |  | 41.483 |  | 500 |  | 77.801 |
| 525 |  | 58.058 |  | 525 |  | 107.778 |
| 550 |  | 80.191 |  | 550 |  | 148.251 |
| 575 |  | 109.426 |  | 575 |  | 202.897 |
| 600 |  | 147.659 |  | 600 |  | 276.678 |
| 625 |  | 197.197 |  | (625) |  | (376.296) |
| 650 |  | 260.836 |  | (650) |  | (510.797) |
| 675 |  | 341.936 |  |  |  |  |
| (700) |  | (444.520) |  |  |  |  |
| (725) |  | (573.379) |  |  |  |  |

Table VII	Molar solution energies for AQiso03 in CO2 at six different temperatures; ¶lns2s+/¶lnρρ+T is abbreviated as k (cf. Fig. 11)

| p / MPa | ΔsolEρ / kJ mol–1 a |  | ΔsolEp / kJ mol–1 b |  | Δ(ΔE) / kJ mol–1 b |  |
| --- | --- | --- | --- | --- | --- | --- |
| T = 305 K, k = 6.9830 |  |  |  |  |  |  |
| 9 |  | 53.7 |  | –59.6 |  | 113.3 |
| 10 |  | 53.3 |  | –27.5 |  | 80.8 |
| 12 |  | 53.4 |  | –2.4 |  | 55.8 |
| 14 |  | 54.3 |  | 9.9 |  | 44.5 |
| 16 |  | 55.2 |  | 17.3 |  | 37.9 |
| T = 310 K, k = 6.3439 |  |  |  |  |  |  |
| 9 |  | 53.8 |  | –196.5 |  | 250.3 |
| 10 |  | 53.9 |  | –62.4 |  | 116.4 |
| 12 |  | 53.3 |  | –11.7 |  | 65.0 |
| 14 |  | 53.3 |  | 4.9 |  | 48.4 |
| 16 |  | 54.0 |  | 14.3 |  | 39.7 |
| T = 315 K, k = 6.5791 |  |  |  |  |  |  |
| 9 |  | 53.1 |  | –392.6 |  | 445.7 |
| 10 |  | 52.9 |  | –179.9 |  | 232.8 |
| 12 |  | 53.8 |  | –37.0 |  | 90.8 |
| 14 |  | 53.3 |  | –7.8 |  | 61.1 |
| 16 |  | 53.2 |  | 5.2 |  | 47.9 |
| T = 320 K, k = 6.6550 |  |  |  |  |  |  |
| 10 |  | 47.1 |  | –265.9 |  | 313.0 |
| 12 |  | 54.1 |  | –74.0 |  | 128.1 |
| 14 |  | 53.8 |  | –22.8 |  | 76.6 |
| 16 |  | 53.3 |  | –3.6 |  | 56.9 |
| T = 330 K, k = 6.9391 |  |  |  |  |  |  |
| 12 |  | 45.6 |  | –145.9 |  | 191.5 |
| 14 |  | 53.3 |  | –66.6 |  | 119.9 |
| 16 |  | 54.1 |  | –27.8 |  | 81.9 |
| T = 340 K, k = 6.9306 |  |  |  |  |  |  |
| 12 |  | 64.4 |  | –91.5 |  | 139.1 |
| 14 |  | 46.0 |  | –74.7 |  | 137.6 |
| 16 |  | 52.5 |  | –51.5 |  | 103.9 |
| a, b Cf. Table IV. |  |  |  |  |  |  |