Electronic Supplementary Information

http://www.rsc.org/suppdata/cp/b0/B200711H

High pressure investigations of (n-alkanes + an ether) mixtures

Urszula Domańska1 and Piotr Morawski

Warsaw University of Technology, Faculty of Chemistry, Physical Chemistry Division, Noakowskiego 3, 00-664 Warsaw, Poland

TABLE 1  Experimental liquid-solid equilibrium pressures for hydrocarbon (1) + MTBE (2) systems

| x1 | p /MPa |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | T = 313.15 K | T = 323.15 K | T = 333.15 K | T = 343.15 K | T = 353.15 K |
| n-Octadecane (1) + MTBE (2) |  |  |  |  |  |
| 1.00000 | 50.3 | 97.9 | 147.7 | 201.3 | 261.2 |
| 0.89105 | 63.6 | 112.2 | 164.5 | 222.7 | 285.8 |
| 0.78001 | 70.0 | 119.2 | 172.6 | 232.1 | 299.5 |
| 0.69926 | 75.2 | 125.1 | 178.8 | 238.9 | 306.1 |
| 0.63262 | 80.4 | 133.8 | 186.8 | 246.1 | 313.7 |
| 0.54123 | 90.1 | 143.8 | 199.3 | 259.6 | 327.6 |
| 0.45351 | 103.6 | 159.7 | 217.8 | 279.0 | 344.3 |
| 0.35956 | 120.3 | 176.4 | 240.5 | 303.8 | 368.3 |
| 0.26104 | 138.9 | 195.1 |  |  |  |
| 0.26723 |  |  | 265.1 | 330.8 | 394.7 |
| 0.19025 | 159.1 | 219.1 | 289.6 | 358.6 | 429.8 |
| 0.11509 | 187.9 | 253.0 | 322.7 | 396.4 | 474.3 |
| 0.05182 | 248.8 | 330.8 | 415.0 | 503.0 | 605.1 |
| 0.02576 | 344.3 | 442.7 | 542.6 | 651.5 | 777.3 |
| 0.00966 | 540.3 | 632.3 | 748.3 | 892.8 | 1082.0 |
| 0.00000 | 973.8 | 1436.9 | 1900.0 | 2363.0 | 2826.0 |
| n-Eicosane (1) + MTBE (2) |  |  |  |  |  |
| 1.00000 | 12.2 | 51.2 | 101.9 | 148.6 | 203.1 |
| 0.89923 | 23.2 | 62.8 | 116.4 | 167.1 | 221.1 |
| 0.79156 | 31.6 | 72.8 | 128.9 | 179.2 | 237.2 |
| 0.69566 | 39.6 | 80.4 | 138.7 | 190.4 | 249.4 |
| 0.59730 | 47.4 | 91.3 | 147.8 | 201.5 | 260.4 |
| 0.49562 | 57.2 | 100.9 | 156.9 | 213.3 | 272.1 |
| 0.39138 | 68.4 | 115.0 | 170.0 | 231.2 | 294.0 |
| 0.29429 | 82.2 | 133.8 | 191.8 | 251.1 | 316.5 |
| 0.19637 | 104.1 | 161.6 | 225.3 | 289.7 | 353.7 |
| 0.10598 | 137.6 | 201.9 | 267.0 | 342.5 | 413.8 |
| 0.05381 | 182.9 | 265.2 | 339.7 | 431.1 | 501.1 |
| 0.04687 | 189.3 | 272.4 | 347.1 | 438.0 | 508.1 |
| 0.02926 | 250.1 | 330.3 | 415.2 | 507.3 | 597.7 |
| 0.01413 | 303.4 | 398.3 | 474.3 | 575.6 | 702.2 |
| 0.00694 | 407.8 | 478.4 | 591.3 | 744.7 | 922.2 |
| 0.00000 | 973.8 | 1436.9 | 1900.0 | 2363.0 | 2826.0 |

TABLE 2 Interpolated solid - liquid equilibrium temperatures for hydrocarbon (1) + MTBE (2) systems

| x1 | T /K | x1 | T /K | x1 | T /K |
| --- | --- | --- | --- | --- | --- |
|  | p = 200 MPa |  | p = 400 MPa |  | p = 600 MPa |
| n-Octadecane (1) + MTBE (2) |  |  |  |  |  |
| 0.9702 | 341.15 | 0.9753 | 379.15 | 0.9803 | 417.15 |
| 0.9101 | 339.15 | 0.9398 | 377.15 | 0.9551 | 415.15 |
| 0.7904 | 337.15 | 0.8997 | 375.15 | 0.9250 | 413.15 |
| 0.6699 | 335.15 | 0.8303 | 373.15 | 0.8948 | 411.15 |
| 0.5750 | 333.15 | 0.7400 | 371.15 | 0.8501 | 409.15 |
| 0.4998 | 331.15 | 0.6401 | 369.15 | 0.7897 | 407.15 |
| 0.4403 | 329.15 | 0.5747 | 367.15 | 0.6996 | 405.15 |
| 0.3751 | 327.15 | 0.5146 | 365.15 | 0.6202 | 403.15 |
| 0.3246 | 325.15 | 0.4604 | 363.15 | 0.5704 | 401.15 |
| 0.2702 | 323.15 | 0.4195 | 361.15 | 0.5147 | 399.15 |
| 0.2253 | 321.15 | 0.3731 | 359.15 | 0.4747 | 397.15 |
| 0.1798 | 319.15 | 0.3322 | 357.15 | 0.4350 | 395.15 |
| 0.1450 | 317.15 | 0.2910 | 355.15 | 0.4004 | 393.15 |
| 0.1151 | 315.15 | 0.2553 | 353.15 | 0.3680 | 391.15 |
| 0.0954 | 313.15 | 0.2229 | 351.15 | 0.3363 | 389.15 |
|  |  | 0.1911 | 349.15 | 0.3051 | 387.15 |
|  |  | 0.1412 | 345.15 | 0.2736 | 385.15 |
|  |  | 0.1052 | 341.15 | 0.2456 | 383.15 |
|  |  | 0.0773 | 335.15 | 0.1995 | 379.15 |
|  |  | 0.0545 | 331.15 | 0.1773 | 377.15 |
|  |  | 0.0205 | 313.15 | 0.1360 | 373.15 |
|  |  |  |  | 0.0909 | 363.15 |
|  |  |  |  | 0.0455 | 351.15 |
|  |  |  |  | 0.0182 | 329.15 |

TABLE 3 (cont.)

| n-Eicosane (1) + MTBE (2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.0000 | 353.15 | 0.9603 | 393.15 | 0.9851 | 435.15 |
| 0.9450 | 351.15 | 0.9299 | 391.15 | 0.9553 | 433.15 |
| 0.8803 | 349.15 | 0.8944 | 389.15 | 0.9247 | 431.15 |
| 0.8103 | 347.15 | 0.8451 | 387.15 | 0.8900 | 429.15 |
| 0.7298 | 345.15 | 0.7900 | 385.15 | 0.8550 | 427.15 |
| 0.6250 | 343.15 | 0.7250 | 383.15 | 0.8202 | 425.15 |
| 0.5254 | 341.15 | 0.6549 | 381.15 | 0.7749 | 423.15 |
| 0.4449 | 339.15 | 0.5751 | 379.15 | 0.7245 | 421.15 |
| 0.3746 | 337.15 | 0.4993 | 377.15 | 0.6254 | 419.15 |
| 0.3253 | 335.15 | 0.4503 | 375.15 | 0.5954 | 417.15 |
| 0.2750 | 333.15 | 0.4100 | 373.15 | 0.5300 | 415.15 |
| 0.2395 | 331.15 | 0.3596 | 371.15 | 0.4805 | 413.15 |
| 0.2054 | 329.15 | 0.3098 | 369.15 | 0.4504 | 411.15 |
| 0.1745 | 327.15 | 0.2799 | 367.15 | 0.4197 | 409.15 |
| 0.1450 | 325.15 | 0.2550 | 365.15 | 0.3850 | 407.15 |
| 0.1197 | 323.15 | 0.2301 | 363.15 | 0.3399 | 405.15 |
| 0.0951 | 321.15 | 0.2046 | 361.15 | 0.3001 | 403.15 |
| 0.0820 | 319.15 | 0.1649 | 357.15 | 0.2751 | 401.15 |
| 0.0698 | 317.15 | 0.1445 | 355.15 | 0.2600 | 399.15 |
| 0.0551 | 315.15 | 0.1253 | 353.15 | 0.2405 | 397.15 |
| 0.0470 | 313.15 | 0.0800 | 345.15 | 0.2249 | 395.15 |
|  |  | 0.0431 | 337.15 | 0.2051 | 393.15 |
|  |  | 0.0108 | 319.15 | 0.1852 | 391.15 |
|  |  |  |  | 0.1603 | 387.15 |
|  |  |  |  | 0.1446 | 385.15 |
|  |  |  |  | 0.1098 | 381.15 |
|  |  |  |  | 0.0703 | 369.15 |
|  |  |  |  | 0.0397 | 361.15 |
|  |  |  |  | 0.0222 | 349.15 |
|  |  |  |  | 0.0044 | 313.15 |