Supplementary Information

Autonomous capillary microfluidic devices with constant flow rate and temperature-controlled valving

Lanhui Li,*a,b Eiko Y. Westerbeek,b,c Jeroen C. Vollenbroek,b,d Sissi de Beer,e Lingling Shui,*a Mathieu Odijk,b and

Jan C.T. Eijkelb

a National Center for International Research on Green Optoelectronics & South China Academy of Advanced Optoelectronics, South China Normal University, Guangzhou 510006, China.

b BIOS/Lab on a Chip Group, MESA+ Institute for Nanotechnology, Max Planck Centre for Complex Fluid Dynamics, University of Twente, Netherlands.

c μFlow group, Department of Chemical Engineering, Vrije Universiteit Brussels, Belgium.

d Dept. Nephrology & Hypertension, University Medical Center Utrecht, Netherlands.

e Materials Science and Technology of Polymers and MESA+ Institute for Nanotechnology, University of Twente, Netherlands.

* Correspondence: l.li-3@utwente.nl; shuill@m.scnu.edu.cn;

SI.1 Temperature sensor characterization

The temperature sensor characterization is done as described in the experimental part. Fig. S1 shows the measured resistance and linear fit of the sensor. It shows a sensitivity of 0.0495 ꭥ/°C and an offset of 20.948 ꭥ. The linearity, R2 is 0.9994.

Fig. S1 Temperature characteristic of the temperature sensor.

SI.2 UV-induced surface grafting of PNIPAm on PDMS mediated by benzophenone

Fig. S2 Schematic drawing of UV-induced surface grafting of PNIPAm on PDMS.

SI.3 Fluorescent dye concentration as indicator for Benzophenone concentration in the different channel walls

We used the fluorescent Rhodamine 6 G at an equal concentration with Benzophenone photoinitiator in acetone as a concentration indicator1. The channel was filled with Rhodamine 6 G-acetone for 5 min followed by a deep wash of the channels with DI water. After washing, a fluorescence microscope image was captured using Olympus IX51 with a FLIR Grasshopper®3 color camera. As shown in Fig. S3, results indicate that the concentration of benzophenone in the bottom PDMS layer is much higher than in the top wall and side walls.

Fig. S3 Fluorescent Rhodamine 6 G in the PDMS channel walls.

SI.4 Material properties used in the numerical simulation

The material properties from the COMSOL 5.5 library were used;

PDMS;

Density , ρ : 970 kg/m3

Thermal conductivity, k: 0.16 W/(m·K)

Heat capacity, Cp: 1460 J/(kg·K)

Borosilicate (Glass):

Density, ρ : 2230 kg/m3

Thermal conductivity, k: 1.13 W/(m·K)

Heat capacity, Cp: 754 J/(kg·K)

H2O (Water):

Density, ρ : 345.28+5.749816*T^1-0.0157244*T^2+1.264375E-5*T^3 kg/m3

Thermal conductivity, k: -0.9003748+0.008387698*T^1-1.118205E-5*T^2 W/(m·K)

Heat capacity, Cp: 4035.841+0.492312*T^1 J/(kg·K)

Air (gas):

Density, ρ : 352.716*T^-1 kg/m3

Thermal conductivity, k: -8.404165E-4+1.107418E-4*T^1-8.635537E-8*T^2+6.31411E-11*T^3-1.88168E-14*T^4 W/(m·K)

Heat capacity, Cp: 1010.97+0.0439479*T^1-2.922398E-4*T^2+6.503467E-7*T^3 J/(kg·K)

SI.5 Mesh details numerical simulation

Fig. S4 a) overview geometry numerical simulation with mesh. b,c,d) details of mesh structure. The mesh has 4 levels i,ii,iii,iv indicated in the figure.

| Mesh (i) | Settings Description; Value. Minimum element size; 0.1 μm. Maximum element size; 2 μm. Curvature factor; 0.2. Curvature factor; Off. Resolution of narrow regions; Off. Maximum element growth rate; 1.05. Maximum element growth rate; Off. |
| --- | --- |
| Mesh (ii) | Settings Description; Value. Maximum element size; 5 μm. Minimum element size; 1 μm. Curvature factor; 0.2. Curvature factor; Off. Resolution of narrow regions; Off. Maximum element growth rate; 1.3. Maximum element growth rate; Off. |
| Mesh (iii) | Settings Description; Value. Maximum element size; 1600. Maximum element size; Off. Minimum element size; 10 μm. Curvature factor; 0.5. Curvature factor; Off. Resolution of narrow regions; 0.6. Resolution of narrow regions; Off. Maximum element growth rate; 1.45. Maximum element growth rate; Off. |
| Mesh (iv) (remaining) | Settings Description; Value. Maximum element size; 1600 μm. Minimum element size; 1. Curvature factor; 0.5. Curvature factor; Off. Resolution of narrow regions; 0.6. Resolution of narrow regions; Off. Maximum element growth rate; 1.45. Maximum element growth rate; Off. |

SI.6 Time stepping and solver settings

Fig. S5 Reciprocall time-step size in seconds vs the amount of time-steps.

An algebraic multigrid solver was used, with the BDF solver. Below an export of the solver settings from COMSOL.

Time-Dependent Solver 1 (t1)

General

| Description | Value |
| --- | --- |
| Defined by study step | Time Dependent |
| Output times | {1.0E-6, 1.0E-5, 1.0E-4, 0.001, 0.01, 0.1, 1, 10} |

Time stepping

| Description | Value |
| --- | --- |
| Maximum BDF order | 2 |
| Error estimation | Exclude algebraic |

Algebraic multigrid Solver

Method and termination

| Description | Value |
| --- | --- |
| Damping factor | 0.9 |
| Jacobian update | Once per time step |
| Stabilization and acceleration | Anderson acceleration |
| Dimension of iteration space | 5 |
| Mixing parameter | 0.9 |
| Iteration delay | 1 |

Error

| Description | Value |
| --- | --- |
| Factor in error estimate | 20 |

Multigrid 1 (mg1)

General

| Description | Value |
| --- | --- |
| Solver | Smoothed aggregation AMG |
| Maximum number of DOFs at coarsest level | 50000 |
| Construct prolongators componentwise | On |
| Prolongator smoothing | Off |

Coarse Solver (cs)

Direct 1 (d1)

General

| Description | Value |
| --- | --- |
| Solver | PARDISO |
| Pivoting perturbation | 1.0E-13 |

SI.7 Videos

Capillary filling in channels with a width of 300 µm and height of 34, 60, 100 µm (Video S1, S2, S3); Stop valve during capillary filling (Video S4).

References

1	D. Ma, H. Chen, D. Shi, Z. Li and J. Wang, J. Colloid Interface Sci., 2009, 332, 85–90.