Electronic Supplementary Material

Insights on the Sustainability of Liquid Chromatographic Methods for Favipiravir Bioanalysis: A Comparative Study

Ahmed Mostafa

E-mail: ammostafa@iau.edu.sa

ammostaf@uwaterloo.ca

Scoring Rules

Red Criteria

- Application scope: In some methods, favipiravir has been analyzed alongside other analytes, thereby expanding the range of substances that a particular method can assess. The broader the spectrum of analytes a method can accommodate, the wider its analytical scope.

| Analytes | Scores |
| --- | --- |
| < 2 | 80 |
| 2 - 4 | 90 |
| 5 - 7 | 100 |
| > 7 | 110 |

- (Lower limit of quantification (LLOQ): In most of the evaluated methods, Limit of Detection (LOD) values were absent, with only Lower Limit of Quantification (LLOQ) values provided. Consequently, LLOQs were utilized for the evaluation of methods according to the following scoring criteria.

| LLOQ (ng mL-1) | Scores |
| --- | --- |
| < 1 | 120 |
| 1 – 9.9 | 110 |
| 10 - 49 | 100 |
| 50 - 99 | 95 |
| 100 - 499 | 90 |
| 500 - 999 | 85 |
| 1000 - 1999 | 80 |
| 2000 - 2999 | 75 |
| 3000 - 3999 | 70 |
| 4000 - 5000 | 65 |

- Precision: Precision is represented as the percent relative standard deviation (%RSD). Higher precision is indicated by a lower RSD. Consequently, a score of 100 is allocated to methods exhibiting RSD values below 5%. International standards typically permit RSD values of up to 20%, thus the scoring scheme sets methods into four distinct categories to differentiate their precision levels.

| Precision | Scores |
| --- | --- |
| <5% | 100 |
| <10% | 95 |
| <15% | 90 |
| <20% | 85 |

- Accuracy

Accuracy is represented as the % recovery and scores were assigned as per the following scoring criteria.

| % Recovery | Scores |
| --- | --- |
| 95 – 105% | 100 |
| 90 – 110% | 95 |
| 85 – 115% | 90 |
| 80 – 120% | 85 |
| 75 – 125% | 80 |
| 70 – 130% | 75 |
| < 70 or > 130% | 70 |

Green Criteria

- Toxicity of reagents: is scored based on the total number of pictograms of the utilized solvents and reagents. A score of 100 marks was assigned for zero pictograms, then followed by 5 marks deduction per pictogram.

- Amount of reagents and waste: A score of 100 marks was assigned for zero waste. Subsequently, there was a deduction of 2 marks for each additional 1 mL increase in waste volume.

- Consumption of energy: All 19 investigated methods utilized liquid chromatographic techniques, including HPLC-UV, UPLC-UV, and LC-MS/MS. Consequently, for the purpose of uniformity and simplicity, instruments were classified into three groups: group with energy consumption ≤0.1 kWh, group with energy consumption ≤1.5 kWh and group with energy consumption ≥1.5 kWh and were subsequently scored as follows.

| Energy Consumption | Scores |
| --- | --- |
| ≤0.1 kWh | 100 |
| ≤1.5 kWh | 95 |
| ≥1.5 kWh | 90 |

- Direct impact and occupational hazards: The occupational hazards present in all 19 investigated methods can be categorized as either organic vapors, noise/high voltage, or reflux temperature. Consequently, a numerical scale ranging from 0 to 3 was devised, where 0 signifies the absence of any occupational hazards, 1 denotes the presence of one of the aforementioned hazards, 2 indicates the presence of any two hazards, and 3 indicates the simultaneous presence of all three hazards. Subsequently, the methods were consistently scored as follows.

| Occupational Hazards | Scores |
| --- | --- |
| 0 | 100 |
| 1 | 95 |
| 2 | 90 |
| 3 | 85 |

Blue Criteria

- Cost efficiency

| Estimated Cost | Scores |
| --- | --- |
| Very low (HPLC-UV with no sample preparation) | 100 |
| Low ((HPLC-UV with sample preparation that does not require commercialized devices) | 95 |
| Moderate (HPLC/UPLC-UV with sample preparation that requires commercialized tools such as SPE manifold and cartridges, ionic liquids, fibres, etc.) | 90 |
| High (LC-MS/MS) | 85 |

- Time efficiency

| Speed of analysis per sample (extraction and analysis times) | Scores |
| --- | --- |
| Fast (< 25 min) | 100 |
| Moderate (25 – 50 min) | 90 |
| Low (50 – 75 min) | 80 |
| Very low (> 75 min) | 70 |

- Requirements

  - Sample consumption

| Sample Consumption per run (mL/mg) | Scores |
| --- | --- |
| < 0.25 | 100 |
| < 1.5 | 90 |
| < 3 | 80 |
| >3 | 70 |

  - Other needs: advanced instruments, skills, facilities

“Other needs” were categorized on a scale of 1 to 5, depending on the complexity of the tools and instruments required for the method. Subsequently, the methods were consistently scored as follows.

| Other needs | Scores |
| --- | --- |
| 1 (Only glassware and simple equipment) | 100 |
| 2 (Basic instrumentation (pH-meter, conductivity meter), glassware and basic equipment for manual preparation | 95 |
| 3 (Medium complexity instrumentation (HPLC-UV, spectrophotometer), equipment for basic preparation | 90 |
| 4 (Complex instrumentation (HPLC-MS), equipment for preparation | 85 |
| 5 (High complexity instrumentation (LC-MS/MS), complex equipment | 80 |

- Operational simplicity (Miniaturization and integration & automation)

| Miniaturization | Scores |
| --- | --- |
| Miniaturized | 100 |
| Not miniaturized | 90 |

| Integration and Automation | Scores |
| --- | --- |
| Fully integrated and automated sample preparation and analysis | 100 |
| None integrated sample preparation but automated analysis | 90 |
| None integrated sample preparation and manual analysis | 80 |