Supplementary Information N°3 for

Harmonizing the assessment of (Green) Hydrogen Supply Chain: a modular and parametrized Life Cycle Assessment Framework.

Gabriel Magnaval 1,2,*, Maël Mouhoub 2, Anne-Marie Boulay1, Manuele Margni2

1 CIRAIG, Department of Chemical Engineering, Polytechnique Montreal, 2500 Chem. De Polytechnique, Montreal, Canada

2 HES-SO Valais-Wallis, Rue de l’Industrie 21, Sion, Switzerland

* corresponding author details: gabriel.magnaval@polymtl.ca

Aim of this document

This document gathers all detailed calculations that structure the parametrized life cycle inventories of green hydrogen supply chain. The equations are organized per module and per elementary process, following the modular structure presented in Figure 3 of the article. In the following document, the flows discussed are recalled, then the equations and parameters are presented. All the processes are scaled to the processing of 1kg of hydrogen: it means that we are going to produce this hydrogen, prepare it, supply it, and reconvert it into electricity. The quantification of parameters is detailed in another SI-2.

Modular structure of the green hydrogen product system. The system is organized into modules—coherent groups of one or more elementary processes that delivers a specific intermediary flow throughout the hydrogen life–cycle.

## 1 – Energy Mixes Preparation

The electricity and heat generation feed all the cradle-to-gate processes in pink called ‘Electricity Consumed’ or ‘Heat consumed’. Energy mixes are parameterized and user-configurable. Users can specify the share (ratio) of the modeled energy provider (wind, PV, hydro, nuclear and european grid for electricity; electric heater, natural gas heater, fatal heat for heat) to construct a custom supply mix.

## ielec=rwind.iwind+rhydro.ihydro+rPV.iPV+rNUC.iNUC+rRER.iRER

## iheat=relec.ielec+ravailable.iavailable+rNG.iNG

With rprovider the ratio of the specific provider in the mix defined by the user.

The iprovider are the provider impact intensities (e.g., CO2eq/kWh)

## 2 – Electricity Generation (Stored in Hydrogen)

Hydrogen contains 33.3kWh/kgH2 of chemical energy, which is provided by electricity in water electrolysis process. This impact is accounted separately in the impact of the supply chain to illustrate that this intrinsic electricity demand cannot be reduced by eco-design. The impact associated to this electricity content is given as followed:

Igen=LHV.ielec

## 3- Electrolyser: Electricity-to-Hydrogen Conversion

In this section, the conversion of electricity into 1kg of hydrogen is assessed.

## 3.1 – Stack Manufacturing

System Scaling to the Functional Unit

First, the electrolyser is scaled to the functional unit (i.e. how much electrolyser stack per kgH2 produced)

| Flow | Electrolyser (FU=1kgH2) |
| --- | --- |
| Reference Flows |  |
| Electrolyser per FU | QEL=1msystem.LTstack |
| BoP system per FU | QBoP_EL=1msystem.LTBoP |
| List of parameters |  |
| Input Parameters | msystem [kgH2/h] hydrogen flowrate; LTi [h] Lifetime of component i. |
| Calculated Parameters | QEL [item/FU] number of electrolyser system per FU; QBoP_EL [item/FU] number of stack per FU; Qstack [item/FU] number of stack waste to manage per FU; |

Stack Design

For the stack, the number of stack and cells is calculated depending on the system size. These amounts are not scaled yet to the functional unit, but given per system.

| Stack Design |  |
| --- | --- |
| Active Surface in the system (*) | Ssystem=PsystemUcell.Jcell=msystem.LHVUideal.Jcell |
| Number Cell in the system | Ncell=Ssystem/Scell |
| Number Stack in the system | Nstack=Ncell/Ncell_per_stack |
| Number of End Plate in the system | Nend_plate=2*Nstack |
| List of parameters |  |
| Input Parameters | Scell [m2] active surface of a cell; Ncell_per_stack [item] number of cell per stack; Jcell [A/m2] current density of the cell; Ucell [V] voltage of the cell. |
| Calculated Parameters | Ssystem [m2/system] total active surface of the system; Nstack [item/system] number of stacks per system; Ncell [item/system] number of cells per system; Nend_plate [item/system] number of end plate per system. |
| Physical Constant | Uideal=1.23V ideal voltage of the cell (no loss); LHV = 33.3 kWh/kgH2 Lower heating Value of hydrogen. |

(*) the power of the system is Psystem=msystem.LHVUcellUideal, explaining the calculation.

Stack and Cells Weight

For the stack, the weight of each layer is calculated. This value is given for the system, and not yet scaled to the functional unit.

| End Plates |  |
| --- | --- |
| Weight of End Plate per system | mEP=sEP.Scell.tEP.Nend_plate .ρEP |
| Stack Components (Frame, Collector, Sealant) |  |
| Weight of stack component per system | mcomp=scomp.Ssystem.qcomp |
| Cell layer (to be calculated for each layer) |  |
| Weight of Support per system | msupport=ssupport.tsupport.Scell.Ncell .ρsupport |
| Weight of Additive per system | madditive=Scell.Ncell.qload |
| Weight of Coating per system | mcoating=ssupport.tcoating.Scell.Ncell .ρcoating |
| List of parameters |  |
| Input Parameters | slayer [-] surface ratio of the layer (surface of the layer divided by the cell active area); ti [kWh] thickness of the layer; qi [kg/m2] load of element i per surface of active area. |
| Calculated Parameters | Ssystem [m2] total active surface of the system; Nstack [item] number of stacks per system; Ncell [item] number of cells per system; mEPkgsystem quantity of end plate per system; mframekgsystem quantity of frame per system; msupport, madditive, mcoating [kgsystem] respective mass of the support, and potential additivie and coating for a layer. |
| Physical Constant | ρmat [kg/m3] density of the material; |

Stack and Cells Environmental impact

For the stack, the intensity of material is invariant with the size (Battelle et al. 2017).

Consequently, the impact of the layers (support, plus potential additive and/or coating) can be written as:

Ilayer=Qlayer.ilayer.Qstack

With ilayer the intensity of the material production (e.g., kgCO2/kg of material)

This can be simplified as:

Ilayer=slayer.msystem.LHVUideal.Jcell.tlayer.ρmat.ilayermsystem.LTstack= slayer.LHV.tlayer.ρmatUideal.Jcell.LTstack.ilayer

The impact of the layers is invariant with the size of the system (including the number of cells or the active surface of a cell).

And for stack components,

Icomp=Qcomp.icomp.Qstack

IEP= slayer.msystem.LHVUideal.Jcell.Ncell_per_stack.tlayer.ρmat.iEPmsystem.LTstack= slayer.LHV.tlayer.ρmatUideal.Jcell.Ncell_per_stack.LTstack.iEP

| Impact of the end plates |  |
| --- | --- |
| Impact End plates | IEP=mEP.Qstack.iEP |
| Impact of Stack Components per FU |  |
| Impact comp | Icomp=mcomp.Qstack.iframe |
| Impact of a layer per FU |  |
| Impact support | Isupport=msupport.Qstack.isupport |
| Impact Load | Iload=mload.Qstack.iload |
| Impact Coating | Icoating=mcoating.Qstack.icoat |

The more cells per stack, the lower is the impact of the end plate per functional unit.

## 3.2 – BoP Manufacturing

System Scaling to the Functional Unit

First, the BoP is scaled to the functional unit (i.e. how much equipment per kgH2 produced)

| Flow | Electrolyser (FU=1kgH2) |
| --- | --- |
| Reference Flows |  |
| BoP system per FU | QBoP_EL=1msystem.LTBoP |
| List of parameters |  |
| Input Parameters | msystem [kgH2/h] hydrogen flowrate; LTi [h] Lifetime of component i. |
| Calculated Parameters | QBoP_EL [item/FU] number of equipment per FU. |

BoP Design and Sizing

The size of each component of the balance of plant is determined as a function of the size of the system. The metrics to characterize the component of the BoP have been chosen so that they linearly scaled with the flowrate.

| BoP Sizing |  |
| --- | --- |
| Compressor Size [kW/system] | Pcomponent=pcomponent.msystem |
| HEX Size [m2/system] |  |
| Chiller Size [kW/system] |  |
| Rectifier Size [kW/system] |  |
| Piping Size [kg/system] |  |
| Pump Size [kW/system] |  |
| Instrumentation Size [kW/system] |  |
| Container Size [m3/system] |  |
| List of parameters |  |
| Input Parameters | pcomponent [metric/(kgH2/h)] size of the component per unit of flowrate; msystem [ kgH2/h] flowrate of the system. |
| Calculated Parameters | Pcomponent [metric/(kgH2/h)] size of the component in the final system. |

BoP environmental impacts

For the balance of plant, the impact intensity of equipment (e, g. kgCO2/kW for the compressor) varies with the size (economies of scale as seen in ecoinvent1).

This economy of scale is represented by a power law, with the scaling factor empirically determined from ecoinvent (see SI4) using the following equation:

ieq=irefPeqpreffeq=irefpref.msystempreffeq=irefmsystemfeq

Consequently, the impact of equipment can be calculated as followed:

Ieq=pref.msystemmsystem.LTeqieq= prefLTeq.irefmsystemfeq

The flow of equipment (capacity of equipment per FU) is independent of the size, but the impact of equipment varies with upsizing due to economies of scale.

## 3.3 – EoL of the system

This elementary process consists in the waste management of the stack; the end-of-life of equipment components is already included in cradle-to-gate inventories (in ecoinvent database).

The amount of waste stack to manage (QEoL) is equal to the amount of stack to manufacture per FU:

QEoL= QEL

In this paper, the recycling rate of metal content in the stack (rrecycling) is considered an input parameter. The rest of the stack is treated as inert waste. As a cut-off approach is considered, no benefit are allocated to the recycling material (but benefits are allocated to recycling content in the material production inventories). The remaining mass of metals (which usually support the end plates, the frame, the current collector and the bipolar plate) and the mass of non metal layers are sent to inert waste management. Consequently, the impact of the end-of-life of the system is:

IEoL=1-rrecycling*mEP+mframe+mcollector+mBP+motherlayer*QEoL*ieol,inert

With ieol,inert the impact intensity of inert waste treatment.

| Flow | Formula for Electrolysers (FU=1kgH2) |
| --- | --- |
| Electricity Demand |  |
| Average Voltage Calculation 2 | Ucell,av=Ucell,BoL1+dmax2 |
| Stack Electricity Losses per FU | Estack=Ucell,av.z.F3.6e6 MH2-LHV=(Ucell,av-Uideal).LHVUideal |
| BoP Electricity Consumed per FU | EBoP=(Pcomp.+Pchiller +Ppump+Pinstrum+ηrect.Estack)/msystem |
| Total Electricity Losses | Esystem= (Ucell,av-Uideal).LHVUideal+EBoP |
| Other Consumable Demand |  |
| Heat Demand per FU | Eheat |
| Water Demand per FU | Qwater |
| KOH Demand per FU | QKOH |
| Hydrogen Leakage |  |
| H2 Leakage per FU | rleakage |
| List of parameters |  |
| Input Parameters | Ucell,BoL [V] voltage at beginning of life of a cell; dmax [%] maximum degradation of the system; Pi [kW] power of the component i; ηrect [-] efficiency of the rectifier; Eheat [kWh/kgH2] stack heat demand per FU; Qwater [kWh/kgH2] stack water demand per FU; QKOH [kWh/kgH2] stack KOH demand per FU |
| Calculated Parameters | Ucell,av [V] average voltage of a cell; Estack [kWh/kgH2] stack electricity losses; EBoP [kWh/kgH2] BoP electricity consumption; |
| Physical Constant | Uideal=1.23V ideal voltage of the cell (no loss); LHV = 33.3 kWh/kgH2 Lower heating Value of hydrogen; z=2 number of electrons exchanged in the reaction; F = 96 485 [C/m] Faraday constant; MH2 = 0.002 [kg/mol] molar mass of H2; |

## 3.4 – Operation of the electrolyser

Consumables Flow per FU

In this section, the quantity of consumables (electricity, heat, KOH and water) and the H2 leakage are directly calculated per functional unit (1kgH2 produced).

The total electricity supply to the stack, including the electricity that will be stored as chemical energy in the hydrogen, is equal to 3,5:

EEL= Ucell,av.z.F3.6e6 MH2

The ideal conversion (without stack losses) corresponds therefore to:

Eideal= LHV= Uideal.z.F3.6e6 MH2

Consequently, electricity lost in the stack corresponds to the calculated parameter Estack. The lower voltage, the lower the losses are in electrolyser mode. Additionally, the equipment also consumes electricity. Notably, the compressors, chillers, pumps, instrumentations, and rectifier induce electricity consumption to operate. This electricity depends on the power of this components (in kW) which depends on the size of the system. The rectifier is the exception, as losses are due to inefficiency of the conversion from AC to DC.

Regarding the H2 leakage, they do not conduct to additional hydrogen to produce, but only to elementary flow emitted in the environment4.

Operation environmental impacts

The following impacts are determined from the flow calculated above. The impact intensity of energy mixes (ielec and iheat) has already been determined (see 1).

| Electrolyser |  |
| --- | --- |
| Impact Electricity | Ielec=Esystem.ielec |
| Impact Water | Iwater= Qwater.iwater |
| Impact KOH | IKOH= QKOH.iKOH |
| Impact Heat | Iheat=Eheat.iheat |
| Impact H2 Leakage | Ileakage=rleakage.ileak |

## 3.5 – Conclusion: Total impact of the electrolysis (elec-to-H2 conversion)

IEL[per kgH2]=Ielec+Iwater+IKOH+Iheat+Ileakage+(eq in BOPIeq)+(IEP+comp in stackIcomp+layer in cellIlayer)

## 4. Post Processing

Post Processing corresponds to adjustement of pressure and purity of the hydrogen produced by the electrolyser to fit the functional unit requirements, or the storage, distribution and fuel cell requirements.

A compressor is operated when the pressure of the hydrogen produced by the electrolyser is lower than the pressure required. The model of Escamilla et al. (2022) is used to size the compressor 3 depending on the hydrogen pressure before and after compression:

With hr=1.41 the capacity ratio, n=msystemMH23600 the molar flow, Rgas=8.314 the gas constant, Tin the temperature of the hydrogen out of the stack (depends on the technology), Zgas=1.024 compressibility factor of the hydrogen, ηcomp=90% efficiency of the compression, Pout the required pressure, Pin the pressure of the hydrogen delivered by the stack and:

With rate per stage = 2.1 is the per stage compression ratio.

This determines the size of the compressor, that should be scaled to the functional unit as the other BoP equipment (with QBoP_EL, see 3.2). It also determines the electricity consumption of the electrolyser (see 3.2).

A Pressure Swing Adsorption (PSA) process is operated when the hydrogen purity supplied by the electrolysis does not reach the required purity 6,7. The size of the PSA depends on the difference between the required purity and the purity of hydrogen supplied by the FC&EL system 8,9. These papers demonstrates that a PSA of 0.5kW is required to purify hydrogen from Grade A (98%) to Grade D (99.97%) with around 3% of hydrogen leakage. Additionally, a PSA of 4kW is required to purify hydrogen from Grade A to Ultrapure 5.0 Grade (99.999%), with 7% of hydrogen leakage (see SI2 for H2 leakage).

Consequently, the environmental impacts of the post-processing is:

IPPper kgH2=(Pcomp.icomp+PPSA.iPSA).QBoP,EL

+(Pcomp+PPSA)/msystem.ielec+rleak,distrib.ileak+(rleak,distrib).(Igen+IEL)

The hydrogen leakage has two consequences: the direct emission of hydrogen in the environment as an elementary flow; the need for surproduction of hydrogen to compensate this loss. Igen+IEL corresponds to the impacts of upstream processes for producing and preparing the surplus of hydrogen that leaks in the system.

## 5. Distribution

Distribution corresponds to delivery of hydrogen from the site of production to the size of end-use application. In this section, the impact of delivering 1kg of hydrogen is calculated. The distribution is characterized by:

- The type of equipment used. The equipment has a specific impact intensity (background), already scaled to the functional unit of 1kg H2 delivered (Ieq,distrib).

- The electricity consumed for delivering the service (Edistrib in kWh/kgH2).

- The hydrogen leakage rate during the delivery (rleak,distrib in %)

The hydrogen leakage has two consequences: the direct emission of hydrogen in the environment as an elementary flow; the need for surproduction of hydrogen to compensate this loss. 

Consequently, the environmental impacts:

Idistribper kgH2=Ieq,distrib+Edistrib.ielec+rleak,distrib.ileak+(rleak,distrib).(Igen+IEL+IPP)

Igen+IEL+IPP corresponds to the impacts of upstream processes for producing and preparing the surplus of hydrogen that leaks in the system.

## 6. Storage

Hydrogen may be stored on the site of end-use application before being used. In this section, the impact of storing 1kg of hydrogen is calculated. The storage is characterized by:

- The type of equipment used. The equipment has a specific impact intensity (background), already scaled to the functional unit of 1kg H2 delivered (Ieq,stored).

- The electricity consumed for delivering the service (Estored in kWH/kgH2).

- The hydrogen leakage rate during the delivery (rleak,storage in %)

The hydrogen leakage has two consequences: the direct emission of hydrogen in the environment as an elementary flow; the need for surproduction of hydrogen to compensate this loss. 

Consequently, the environmental impacts:

Istorageper kgH2=Ieq,storage+Estorage.ielec+rleak,storage.ileak+(rleak,storage).(Igen+IEL+IPP+Idistrib)

Igen+IEL+IPP+Idistrib corresponds to the impacts of upstream processes for producing and preparing the surplus of hydrogen that leaks in the system.

## 7. Fuel Cell System: Hydrogen-to-Electricity Reconversion

In this section, for ensuring continuity with the previous work, we propose to compute the impacts of the fuel cell module, scaled to the reconversion of 33kWh of electricity (which corresponds to the energy stored in 1kg of hydrogen).

## 7.1 – Stack Manufacturing

The modeling of the stack and BoP manufacturing for the fuel cell are almost similar to the electrolyser calculation. The only difference is that the size of the system is expressed in terms of electrical power (Psystem), and not in flowrate. All the flows for stack manufacturing are summed up in next table, following the same steps as for the electrolyser

| Flow | Fuel Cell (FU=33kWh) |
| --- | --- |
| Reference Flows |  |
| Stack per FU | Qstack_FC=LHVPsystem.LTstack |
| BoP per FU | QBoP_FC=LHVPsystem.LTBoP |
| Stack Design |  |
| Active Surface per system | Ssystem=PsystemUcell,av.Jcell |
| Number Cell per system | Ncell=Ssystem/Scell |
| Number Stack per system | Nstack=Ncell/Ncell_per_stack |
| Number of End Plate in the system | Nend_plate=2*Nstack |
| End Plate |  |
| Weight of End Plate per system | mEP=sEP.Scell.tEP.Nend_plate .ρEP |
| Stack Components |  |
| Weight of stack component per system | mcomp=scomp.Ssystem.qcomp |
| Cell Layers |  |
| Weight of Support per system | msupport=ssupport.tsupport.Scell.Ncell .ρsupport |
| Weight of Additive per system | madditive=Scell.Ncell.qload |
| Weight of Coating per system | mcoating=ssupport.tcoating.Scell.Ncell .ρcoating |
| List of parameters |  |
| Input Parameters | Psystem [kWh] Electrical Power; LTi [h] Lifetime of component I; Scell [m2] active surface of a cell; Ncell_per_stack [item] number of cell per stack; Jcell [A/m2] current density of the cell; Ucell,av [V] average voltage of the cell; slayer [-] surface ratio of the layer (surface of the layer divided by the cell active area); ti [kWh] thickness of the layer; qi [kg/m2] load of element i per surface of active area; λ [-] fuel utilization. |
| Calculated Parameters | Ssystem [m2] total active surface of the system; Nstack [item] number of stacks per system; Ncell [item] number of cells per system; mEPkgsystem quantity of end plate per system; mframekgsystem quantity of frame per system; msupport, madditive, mcoating [kgsystem] respective mass of the support, and potential additivie and coating for a layer. |
| Physical Constant | LHV = 33.3 kWh/kgH2 Lower heating Value of hydrogen; ρmat [kg/m3] density of a material |

Fuel Cell Stack environmental impacts

For the stack, the intensity of material is invariant with the size (Battelle et al. 2017).

Consequently, the impact of the layers (support, plus potential additive and/or coating) can be written as:

Ilayer=Qlayer.ilayer.Qstack

With ilayer the intensity of the material production (e.g., kgCO2/kg of material)

This can be simplified as:

Ilayer=slayer.PsystemUcell.Jcell.tlayer.ρmat.ilayer.LHVPsystem.LTstack= slayer.LHV.tlayer.ρmatUcell.Jcell.LTstack.ilayer

The impact of the layers is invariant with the size of the system (including the number of cells or the active surface of a cell).

And for stack components,

Icomp=Qcomp.icomp.Qstack

IEP= slayer.PsystemUcell.Jcell.Ncell_per_stack.tlayer.ρmat.iEPmsystem.LTstack= slayer.LHV.tlayer.ρmatUcell.Jcell.Ncell_per_stack.LTstack.iEP

The more cells per stack, the lower is the impact of the end plate per functional unit.

| Impact of the end plates |  |
| --- | --- |
| Impact End plates | IEP=mEP.Qstack.iEP |
| Impact of Stack Components per FU |  |
| Impact comp | Icomp=mcomp.Qstack.iframe |
| Impact of a layer per FU |  |
| Impact support | Isupport=msupport.Qstack.isupport |
| Impact Load | Iload=mload.Qstack.iload |
| Impact Coating | Icoating=mcoating.Qstack.icoat |

Fuel Cell Stack end of life

As for electrolyser,

IEoL=1-rrecycling*mEP+mframe+mcollector+mBP+motherlayer*Qstack_FC*ieol,inert

| BoP Sizing (*) |  |
| --- | --- |
| Compressor Size [kW/system] | Pcomponent=pcomponent.(PsystemLHV.UidealUcell,av.λ) |
| HEX Size [m2/system] |  |
| Chiller Size [kW/system] |  |
| Rectifier Size [kW/system] |  |
| Piping Size [kg/system] |  |
| Pump Size [kW/system] |  |
| Instrumentation Size [kW/system] |  |
| Container Size [m3/system] |  |
| List of parameters |  |
| Input Parameters | pcomponent [metric/(kgH2/h)] size of the component per unit of flowrate; Psystem [ kW] power of the system; λ [-] fuel utilization; ηFC [-] fuel utilization. |
| Calculated Parameters | Pcomponent [metric/(kgH2/h)] size of the component in the final system |
| Physical Constant | LHV = 33.3 kWh/kgH2 Lower heating Value of hydrogen; Uideal= 1.23V ideal voltage. |

## 7.2 – BoP Manufacturing

## * msystem=(PsystemLHV.UidealUcell,av.λ) the flowrate of hydrogen in the system.

BoP environmental impacts

For the balance of plant, the impact intensity of equipment (e, g. kgCO2/kW for the compressor) varies with the size (economies of scale as seen in ecoinvent1).

This economy of scale is represented by a power law, with the scaling factor empirically determined from ecoinvent (see SI4) using the following equation 10,11:

ieq=irefPeqpreffeq=irefpref.msystempreffeq=irefmsystemfeq

Consequently, the impact of equipment can be calculated as followed:

Ieq=pref.PsystemLHV.UidealUcell,av.λ.LHVPsystem.LTeqieq= pref.UidealUcell,av.λLHV.LTeq.irefmsystemfeq

The flow of equipment (capacity of equipment per FU) is independent of the size, but the impact of equipment varies with upsizing due to economies of scale.

## 7.3 – Operation

Consumables Flow per FU

In this section, the quantity of consumables (electricity, heat, KOH and water) and the H2 leakage are directly calculated per functional unit (1kgH2 produced).

| Hydrogen Demand (Fuel Cell) |  |
| --- | --- |
| Hydrogen Losses |  |
| Cell Voltage Degradation | Ucell,av=Ucell,BoL1-dmax2 |
| Electrical Energy recovered from 1kgH2 | Eelec,FC=Ucell,avUideal.λ.LHVH2 |
| Energy Recovered from Heat (Energy Allocation) | Eheat,FC |
| Share of Elec. Lost by the stack (Energy Allocation) | rstack=1-(Eelec,FC+Eheat,FC)LHV |
| Exergy Recovered from heat (Exergy Allocation) (**) | Exheat,FC=Eheat,FC.1-T0lnTstackT0Tstack-T0 |
| Share of Elec. Lost by the stack (Exergy Allocation) | rstack=1-(Eelec,FC+Exheat,FC)LHV |
| Share of electricity consumed by BoP (*) | rBoP=Pcomp.+Pchiller +Ppump+Pinstrum+ηrect.EstackPsystem |
| H2 leakage | rleakage |
| Energy Recovered from Heat (Energy Allocation) | Eheat,FC |
| Exergy Recovered from heat (Exergy Allocation) | Exheat,FC=Eheat,FC.(1-T0lnTstackT0Tstack-T0) |
|  | Eelec,FC=1-UidealUcell,av.λ |
| Total Hydrogen Losses per FU | Hsystem=1(1-rstack)(1-rBoP)(1-rleakage)-1 |
| Heat Recovering (Fuel Cell) |  |
| Heat Recovered : Exergy Allocation |  |
| Other Consumable Demand (Fuel Cell) |  |
| KOH Demand per FU(**) | QKOH.(1+Hsystem) |
| List of parameters |  |
| Input Parameters | Ucell,BoL [V] voltage at beginning of life of a cell; dmax [%] maximum degradation of the system; Pi [kW] power of the component i; ηrect [-] efficiency of the rectifier; stack water demand; QKOH [kWh/kgH2] stack KOH demand; Eheat,rec [kWh/kgH2] stack heat recovered; λ [-] fuel utilization; rleakage [-] hydrogen leakage rate; Tstack [K] stack temperature; T0 [K] ambiant temperature. |
| Calculated Parameters | Ucell,av [V] average voltage of a cell; Estack [kWh/kgH2] stack electricity losses; EBoP [kWh/kgH2] BoP electricity consumption; rstack [-] Share of energy lost in stack; rBoP [-] Share of power lost in BoP; HFC [kgH2FU] hydrogen lost in the system during operation; Eheat,rec [kWh/kgH2 processed] quantity of heat recovered per kg of hydrogen processed. |
| Physical Constant | Uideal=1.23V ideal voltage of the cell (no loss); LHV = 33.3 kWh/kgH2 Lower heating Value of hydrogen; z=2 number of electrons exchanged in the reaction; F = 96 485 [C/m] Faraday constant; MH2 = 0.002 [kg/mol] molar mass of H2; |

(*) the electricity to power the BoP is taken directly from the electricity produced by the fuel cell system.

(**) Equation from https://klimapedia.nl/wp-content/uploads/2014/10/Exergy-Guidebook-for-Building-Professionals_version-2.1_Nov2014.pdf

The fuel cell consumed KOH and hydrogen that are lost during operation phase. Hydrogen losses are due to stack conversion losses, BoP electricity consumption and hydrogen leakage in the system. This hydrogen loss lead to a surplus of hydrogen production to fulfill the functional unit of the system: 1kg of hydrogen is produced for the final electricity demand, plus Hsystem that is produced (to compensate the losses in the system). The impacts of this hydrogen corresponds to the impacts of the upstream processes of the supply chain.

Consequently, the operation phase impacts is:

IKOH=QKOH.iKOH

Ileakage=rleak,storage.ileak

IsurplusH2=Hsystem.(Igen+IEL+IPP+Idistrib+Istorage)

Iope_FCper FU=IKOH+Ileakage+IsurplusH2

## 7.4 – Conclusion: Total impact of the fuel cell module (H2-to-elec conversion)

IFC[per kgH2]=IKOH+Ileakage+IsurplusH2+(eq in BOPIeq)+(IEP+comp in stackIcomp+layer in cellIlayer)

## 8. End-Use Processes

The burner for the generation of 1MJ of industrial heat from hydrogen is characterized by the impact of the equipment, scaled to 1kg of hydrogen burnt, and the hydrogen leakage rate. As the efficiency of hydrogen burner are around 90%, this application requires 0.009 kgH2/MJ industrial heat.

Iburner[per MJ]=Ieq,burner+rleak,burner.ileak

The fuel cell vehicle (FCEV) for the generation of mechanical energy for 1vehicle.km is characterized by an aggregated impact given per v.km. This includes the whole life cycle stage of the vehicle, including vehicle and engine manufacturing and maintenance, end-of-life, use phase processes other than energy consumption (including road infrastructure and brake, wear, type emission).

IFCEV[per v.km]=Ieq,FCEV

Based on PETRAUL6, an average fuel cell vehicle needs to produce 0.22 kWh/(v.km), which correspond to 6.7e-3 kgH2/(v.km)

## 9. Total supply-chain impact per application

Finally, all the impacts calculated in this work should be scaled to the final end-use functional unit. The quantity of hydrogen required to fulfill this functional unit depends on the application, and the efficiency of end-use processes.  The following table

| Application | FU | Total Impact |
| --- | --- | --- |
| Hydrogen Production Application |  |  |
| Chemical Platform | 1kgH2 | Igen+IEL+IPP+Istorage+Idistrib |
| Industrial Heat | 1MJ | Igen+IEL+IPP+Istorage+Idistrib120 [MJkgH2]+Iburner |
| Hydrogen Production and Reconversion Applications |  |  |
| Domestic Heat | 1MJ | Igen+IEL+IPP+Istorage+Idistrib+IFC.1LHV*3.6 |
| Elec. Storage | 1kWh | IEL+IPP+Istorage+Idistrib+IFC.1LHV |
| Controllable Elec. | 1kWh | Igen+IEL+IPP+Istorage+Idistrib+IFC.1LHV |
| Transportation | 1v.km | Igen+IEL+IPP+Istorage+Idistrib+IFC.6.7 10-3kgH2v.km+IFCEV |

1.	Wernet, G., Bauer, C., Steubing, B., Reinhard, J., Moreno-Ruiz, E., and Weidema, B. (2016). The ecoinvent database version 3 (part I): overview and methodology. International Journal of Life Cycle Assessment 21, 1218–1230. https://doi.org/10.1007/s11367-016-1087-8.

2.	CLEAN HYDROGEN JU (2022). Strategic Research and Innovation Agenda 2021 – 2027 .

3.	Escamilla, A., Sánchez, D., and García-Rodríguez, L. (2022). Assessment of power-to-power renewable energy storage based on the smart integration of hydrogen and micro gas turbine technologies. Int J Hydrogen Energy 47, 17505–17525. https://doi.org/10.1016/j.ijhydene.2022.03.238.

4.	European Commission, J.R.C., Arrigoni, A., Dolci, F., Ortiz Cebolla, R., Weidner, E., D’Agostini, T., Eynard, U., Santucci, V., and Mathieux, F. (2024). Environmental life cycle assessment (LCA) comparison of hydrogen delivery options within Europe.

5.	Zhao, Y., Xue, H., Jin, X., Xiong, B., Liu, R., Peng, Y., Jiang, L., and Tian, G. (2021). System level heat integration and efficiency analysis of hydrogen production process based on solid oxide electrolysis cells. Int J Hydrogen Energy 46, 38163–38174. https://doi.org/10.1016/j.ijhydene.2021.09.105.

6.	Magnaval, G., Debonnet, T., and Margni, M. (2025). Integrated LCA and Eco-design Process for Hydrogen Technologies: Case Study of the Solid Oxide Electrolyser. Preprint, https://doi.org/LAPSE:2025.0030v2 https://doi.org/LAPSE:2025.0030v2.

7.	Szczurowski, J., Lubecki, A., Hasal, M., Fudalewski, J., and Zarębska, K. (2025). Application of LCA in eco-design of the laboratory PSA unit. Environ Impact Assess Rev 112, 107839. https://doi.org/10.1016/j.eiar.2025.107839.

8.	Schwarze, K., Posdziech, O., Kroop, S., Lapeña-Rey, N., and Mermelstein, J. (2017). Green Industrial Hydrogen via Reversible High-Temperature Electrolysis. ECS Trans 78, 2943–2952. https://doi.org/10.1149/07801.2943ecst.

9.	Ligen, Y., Vrubel, H., and Girault, H. (2020). Energy efficient hydrogen drying and purification for fuel cell vehicles. Int J Hydrogen Energy 45, 10639–10647. https://doi.org/10.1016/j.ijhydene.2020.02.035.

10.	Remer, D., and Chai, L. (1993). Process Equipment, Cost Scale-Up. In Encyclopedia of Chemical Processing and Design, pp. 306–317.

11.	Zhang, X., Bauer, C., Mutel, C.L., and Volkart, K. (2017). Life Cycle Assessment of Power-to-Gas: Approaches, system variations and their environmental implications. Appl Energy 190, 326–338. https://doi.org/10.1016/j.apenergy.2016.12.098.