# Supplementary Information

Microkinetic characterization of ROS-driven methane production through quantum and classical simulations

Ronghan Zhanga,b,*, Zexuan Wanga,b,*, Kin Tung Michael Hoa, Nigel J.D. Grahama, Giin-Yu Amy Tanb,**, Po-Heng Henry Leea,**

a Department of Civil and Environmental Engineering, Imperial College London, London, SW7 2AZ, UK

b Department of Architecture and Civil Engineering, City University of Hong Kong, Hong Kong SAR, China

* Co-first authors email address: rzhang343-c@my.cityu.edu.hk; zexuawang4-c@my.cityu.edu.hk

** Corresponding author email address: amytan@cityu.edu.hk; po-heng.lee@imperial.ac.uk

43 pages

6 supplementary notes

3 supplementary tables

1 supplementary figure

Table of Contents

Supplementary Note 1: Model development chemical parameters calculation	3

Supplementary Note 2: Model development for determination of the catalytic and/or redox reaction of ASC and Fe3+	4

Supplementary Note 3: Model development for determination of the ROS-induced methane production pathway	6

Supplementary Note 4: Model development for determination of the kinetic reaction rate constant	9

Supplementary Note 5: ROS-induced methane production and conversion of MET to methane	10

Supplementary Note 6: Code available	11

Supplementary Table 1. Summary of the estimated kinetic reaction rate constants and Gibbs free energy for Eqs. 8-10 in Table 2.	40

Supplementary Table 2. Estimated Chemical Equilibrium constant (K)	40

Supplementary Table 3. Changes in reactant/product concentration with temperature increase from 25°C to 37°C	41

Supplementary Figure 1. The variation of catalyst and redox reactions with oxygen concentration from 0 μM to 1000 μM	42

References	43

## Supplementary Note 1: Model development chemical parameters calculation

The inverse reaction rate constant (kinetic parameter) k-i is obtained using (Supplementary Eq.1):

|  | k-i=ki Ki | (1) |
| --- | --- | --- |

where ki is the forward reaction rate constant (kinetic parameter), and K is the chemical equilibrium constant that can be derived from Supplementary Eq. 2:

|  | ∆G0=-RTln(K) | (2) |
| --- | --- | --- |

where R represents the gas constant, which is 8.314 J.K-1.mol-1, T is the temperature, set to 298.15K, and ∆G0 is the Gibbs free energy.

## Supplementary Note 2: Model development for determination of the catalytic and/or redox reaction of ASC and Fe3+

To investigate whether the reaction mechanism of ASC with Fe3+ is redox and/or catalytic under the conditions described by Althoff et al., 1 the reaction rates are described as follows:

|  | r1=k1ASC-k-1[DHA][H2O2] | (3) |
| --- | --- | --- |
|  | r8=k3ASC-k-3H+2[O2∙-][A∙-] | (4) |
|  | r4=k4A·- [O2∙-]-k-4[DHA][OH-][HO2.-] | (5) |

The inverse reaction rates k-1,k-2 and k-3 are obtained by using Supplementary Eq. 1 and Supplementary Eq. 2. Additionally, according to the conservation of materials: [ASCreacted]+[DHA(I)]+[DHA(II)]+[A∙-]= ASCtotal = 50×10-3mol/L, where [DHA(I)] is [H2O2] and [DHA(II)] is [HO2.-]. Thus, a nonlinear matrix can be built and is given by Supplementary Eq. 6.

| 1111k10-k-1[DHA]00-k-4OH-[DHA]0k4[O2∙-]k300-k-3[H+]2[O2∙-]×ASCDHA(II)DHA(I)A∙-=50×10-3000 | (6) |
| --- | --- |

To investigate the mechanism for methane production microkinetics in non-methanogenic microbes, a nonlinear matrix (Supplementary Eq. 6) was used. By making use of assumptions 1-4 and applying steady-state conditions, the unknown parameters in matrix A were estimated. Next, the non-linear matrix was linearized for HHL algorithm implementation.

| A∙x=b |  |
| --- | --- |
| 150×10-3150×10-3150×10-3150×10-3k10-k-1[DHA]00-k-4OH-[DHA]0k4[O2∙-]k300-k-3[H+]2[O2∙-]×ASCHO2.-H2O2A∙-=1000 | (7) |

where Supplementary Eq. 7 is the microkinetic linear matrix model (Eq. 14) in the Methodology section of the paper.

The HHL algorithm requires the matrix to be Hermitian, necessitating the transformation of matrix A into matrix AH (Supplementary Eq. 8).

|  | AH=0AA†0 | (8) |
| --- | --- | --- |

where A† denotes the complex conjugate transpose of A.

After the conversion of the "4×4" matrix using Supplementary Eq. 8, the matrix is expanded to an "8×8" matrix AH. The b and x matrices are also in "8×1" dimensions. By deploying these transformations, the unknown term x could be calculated using Supplementary Eq. 9 in the HHL algorithm:

|  | x=b×AH-1 | (9) |
| --- | --- | --- |

## Supplementary Note 3: Model development for determination of the ROS-induced methane production pathway

Simulating the methane production pathway using the HHL algorithm requires two steps. Firstly, it is necessary to determine the amount of [Fe(IV)=O]2+ generated after the reaction of ASC, H2O2 and Fe3+ in the Fenton reaction. The second step is to simulate the process of the reaction between MET and [Fe(IV)=O]2+ to generate methane.

To investigate the ratio of different iron ions under steady state after the Fenton reaction, the reaction rates can be described as follow:

| r2=k2ASC[Fe3+]-k-2H+2[Fe2+][A∙-] | (10) |
| --- | --- |
| r10=k5ASC[Fe2+]H2O2-k-5[A∙-][Fe3+] | (11) |
| r6=k6[Fe2+]H2O2-k-6[FeIV=O2+] | (12) |

The inverse kinetic reaction rate constants of k-2, k-5 and k-6 are obtained using Supplementary Eqs. 1 and 2.

Additionally, according to the conservation of materials: [Fe3+]+[Fe2+]+[FeIV=O2+]= Fetotal = 40×10-3mol/L (Table 2), and a nonlinear matrix can be formulated as Supplementary Eq. 13.

| 1110k2ASC-k-2H+2[A∙-]01-k-5[A∙-]k5ASCH2O2010k6H2O2-k-60×Fe3+Fe2+FeIV=O2+c=40×10-3000 | (13) |
| --- | --- |

* Elements (2,4) and (3,4) in the 4×4 matrix are included to ensure that the 4×4 matrix is not a singular matrix, and element c in the 4×1 matrix is an arbitrary term used to balance the matrix.

To simulate methane production under steady state, the reaction rate is described as follows:

| r9=k9[FeIV=O2+]2[MET]H2O2[H+]2-k-9[Fe3+]2[HCyA] | (14) |
| --- | --- |

The stoichiometric relationship between HCyA and methane is assumed to be 1:1 in Eq. 9 in Table 2. Therefore, the production of HCyA represents the production of methane. Additionally, according to the conservation of materials, the total amount of methionine (MET) is equal to the sum of its concentration and HCyA concentration, expressed as [MET]+[HCyA]= METtotal = 0.5×10-3mol/L (refer to Table 2). Thus, the nonlinear matrix could be constructed as Supplementary Eq. 15.

| 11k9[FeIV=O2+]2H2O2[H+]2-k-9[Fe3+]2×METHCyA=0.5×10-30 | (15) |
| --- | --- |

To simulate the ROS-induced methane production model, the conversion rate of different iron ions in this model was first determined under steady state conditions using the non-linear matrix Supplementary Eq. 13. Accordingly, the unknown parameters in matrix A can be estimated. The non-linear matrix Supplementary Eq. 13 is then linearized (Supplementary Eq. 16) for computation using the HHL algorithm.

| 140×10-3140×10-3140×10-30k2ASC-k-2H+2[A∙-]01-k-5[A∙-]k5ASCH2O2010k6H2O2-k-60×Fe3+Fe2+FeIV=O2+c=1000 | (16) |
| --- | --- |

where Supplementary Eq. 16 is the microkinetic linear matrix model (Eq. 15) in the Methodology section of the paper.

To simulate the ROS-induced methane production microkinetic model under steady state conditions, the nonlinear matrix Supplementary Eq. 15 was used. By use of the outcomes of Objective 2 (Supplementary Table 2), the unknown parameters in matrix A were estimated. Subsequently, the HHL algorithm simulation can be used in the linearized matrix Supplementary Eq. 17.

| 10.5×10-310.5×10-3k9[FeIV=O2+]2H2O2[H+]2-k-9[Fe3+]2×METHCyA=10 | (17) |
| --- | --- |

where Supplementary Eq. 17 is the microkinetic linear matrix model Eq. 16 in the Methodology section of the paper.

## Supplementary Note 4: Model development for determination of the kinetic reaction rate constant

The mass balance of reactants can be expressed as:

|  | NInitial – NFinal = rVt | (18) |
| --- | --- | --- |

The mass balance of products can be expressed as:

|  | NFinal – NInitial = rVt | (19) |
| --- | --- | --- |

where NInitial and NFinal, represent the number of moles of substances before and after the reaction, respectively; t represents the reaction time (s); V represents the volume of the reaction device (L); and r represents the reaction rate (M-1s-1).

To determine the kinetic reaction rate constant for Ferryl ion and MET, the nonlinear matrix Supplementary Eq. 20 (refer to Eq.17 in the Methodology section) can be formulated using the mass balances in Eqs.2, 5, 6, 7 ,9 (Table 2).

| NFe2+, FinalNH2O2, FinalNFe3+, FinalN∙OH, FinalNCH4, FinalNASC, FinalNFeIVO2+, FinalNMET, Final--k5-k6 k2-k5-k6 0 0 0 0 0 k5 k2 k5 0 0 0 -k7 0 0 0 0 -k20 0 0 0 0 k9 -k70 0 -k9 0 -k9∙[Fe2+]∙[H2O2][Fe3+]∙[ASC][Fe3+]∙[∙OH][FeIV=O2+]2[MET]H2O2[H+]2∙V∙t-NFe2+, InitialNH2O2, InitialNFe3+, InitialN∙OH, InitialNCH4, InitialNASC, InitialNFeIVO2+, InitialNMET, Initial=0 | (20) |
| --- | --- |

## Supplementary Note 5: ROS-induced methane production and conversion of MET to methane

To verify the HHL algorithm simulation results in the matrix above (Supplementary Eq. 20), the methane production (Supplementary Eq. 21) and the methane conversion rate (η) from MET (Supplementary Eq. 22) were calculated.

| NMethane = [HCyA]∙V | (21) |
| --- | --- |
| η= NMethaneNMET × 100% | (22) |

## Supplementary Note 6: Code available

- Mechanistic determination by Google Quantum AI & Classical MATLAB

Google Quantum AI

import cirq

import numpy as np

import scipy.linalg

# Data input:  k1, k3, and k4 are taken from Table 2 in the main text (MS).

# The corresponding reverse/reconstructed constants k_1, k_3, and k_4

# are obtained based on Supplementary Eq. 1 in Supplementary Note 1.

def get_chemical_matrix():

k1 = 4 * 10**5

k_1 = 14

k3 = 1 * 10**-5

k_3 = 4.5 * 10**-72

k4 = 2.6 * 10**8

k_4 = 3.66 * 10**36

# The following rows are constructed according to Eq. 14  in Section 2.2 (Methodology) of the MS.

row0 = [20.0, 20.0, 20.0, 20.0]

row1 = [k1, 0.0, -k_1 * 1.31 * 10**-4, 0.0]

row2 = [0.0, -k_4 * 10**-11 * 1.31 * 10**-4, 0.0, k4 * 3.9 * 10**-6]

row3 = [k3, 0.0, 0.0, -k_3 * 10**-6 * 3.9 * 10**-6]

# The coefficient matrix A and vector b are assembled in accordance with Eq. 11 in Section 2.2 (Methodology) of the MS.

A = np.array([row0, row1, row2, row3], dtype=np.float64)

b = np.array([1.0, 0.0, 0.0, 0.0], dtype=np.float64)

return A, b

def strong_preconditioning(A, b, iters=2000):

print("--- [Step 1] Running Sinkhorn-Knopp Preconditioning ---")

r = np.ones(A.shape[0], dtype=np.float64)

c = np.ones(A.shape[1], dtype=np.float64)

A_new = A.astype(np.float64).copy()

for _ in range(iters):

r_norms = np.linalg.norm(A_new, axis=1)

r_norms[r_norms == 0] = 1.0

A_new = np.diag(1.0 / r_norms) @ A_new

r *= (1.0 / r_norms)

c_norms = np.linalg.norm(A_new, axis=0)

c_norms[c_norms == 0] = 1.0

A_new = A_new @ np.diag(1.0 / c_norms)

c *= (1.0 / c_norms)

b_new = np.diag(r) @ b

b_norm = np.linalg.norm(b_new)

if b_norm < 1e-30:

raise ValueError("b' norm too small after scaling.")

b_quantum = (b_new / b_norm).astype(np.complex128)

print(f"    Balanced Matrix Condition Number: {np.linalg.cond(A_new):.4f}")

return A_new, b_quantum, r, c, float(b_norm)

def build_block_matrix(A):

n = A.shape[0]

Z = np.zeros((n, n), dtype=np.float64)

return np.vstack((np.hstack((Z, A)), np.hstack((A.T, Z)))).astype(np.float64)

def embed_b_to_8d(b4, embed="top"):

b4 = np.asarray(b4, dtype=np.complex128).reshape(4)

b8 = np.zeros(8, dtype=np.complex128)

if embed.lower() == "top":

flag = 0

elif embed.lower() == "bottom":

flag = 1

else:

raise ValueError("embed must be 'top' or 'bottom'")

for vec in range(4):

b8[(flag << 2) | vec] = b4[vec]

nrm = np.linalg.norm(b8)

if nrm < 1e-30:

raise ValueError("Embedded b8 has ~0 norm.")

return b8 / nrm

def amplitude_embed_unitary(b8):

b8 = np.asarray(b8, dtype=np.complex128).reshape(8)

b8 = b8 / np.linalg.norm(b8)

M = np.eye(8, dtype=np.complex128)

M[:, 0] = b8

Q, _ = np.linalg.qr(M)

phase = np.vdot(Q[:, 0], b8)

if abs(phase) > 1e-12:

Q[:, 0] *= (phase / abs(phase)).conjugate()

return Q

#HHL circuit construction

def create_hhl_circuit(H, b4_quantum, *, n_clock=8, embed="top",

scale_t=0.99, scale_C=0.95, eig_cut=1e-9):

n_sys = 3

qubits_clock = [cirq.LineQubit(i) for i in range(n_clock)]

q_flag = cirq.LineQubit(n_clock)

q_v0   = cirq.LineQubit(n_clock + 1)

q_v1   = cirq.LineQubit(n_clock + 2)

qubits_sys = [q_flag, q_v0, q_v1]

qubit_anc = cirq.LineQubit(n_clock + 3)

eigvals = np.linalg.eigvalsh(H)

valid = eigvals[np.abs(eigvals) > eig_cut]

lam_min = float(np.min(np.abs(valid)))

lam_max = float(np.max(np.abs(valid)))

t = float(2 * np.pi / lam_max * scale_t)

C = float(lam_min * scale_C)

print(f"--- [Step 2] HHL Parameters: t={t:.6f}, C={C:.6e}, "

f"λ_min={lam_min:.6e}, λ_max={lam_max:.6e}, n_clock={n_clock} ---")

b8 = embed_b_to_8d(b4_quantum, embed=embed)

Uprep = amplitude_embed_unitary(b8)

circuit = cirq.Circuit()

circuit.append(cirq.MatrixGate(Uprep).on(*qubits_sys))

# QPE

circuit.append(cirq.H.on_each(*qubits_clock))

for k in range(n_clock):

dt = t * (2**k) / (2**n_clock)

U = scipy.linalg.expm(1j * H * dt).astype(np.complex128)

cU = cirq.MatrixGate(U).controlled()

ctrl = qubits_clock[n_clock - 1 - k]

circuit.append(cU(ctrl, *qubits_sys))

circuit.append(cirq.qft(*qubits_clock, inverse=True))

# Inversion rotation

N = 2**n_clock

two_pi_over_t = 2 * np.pi / t

for i in range(1, N):

phi = i / N

phi_wrapped = (phi - 1.0) if (phi >= 0.5) else phi

lam_est = two_pi_over_t * phi_wrapped

if abs(lam_est) < 1e-12:

continue

ratio = float(np.clip(C / lam_est, -1.0, 1.0))

theta = float(2 * np.arcsin(ratio))

bits = format(i, f"0{n_clock}b")

controls = []

for idx, bit in enumerate(bits):

q = qubits_clock[idx]

if bit == "0":

circuit.append(cirq.X(q))

controls.append(q)

circuit.append(cirq.ry(theta).on(qubit_anc).controlled_by(*controls))

for idx, bit in enumerate(bits):

if bit == "0":

circuit.append(cirq.X(qubits_clock[idx]))

# Inverse QPE

circuit.append(cirq.qft(*qubits_clock))

for k in range(n_clock - 1, -1, -1):

dt = -t * (2**k) / (2**n_clock)

U = scipy.linalg.expm(1j * H * dt).astype(np.complex128)

cU = cirq.MatrixGate(U).controlled()

ctrl = qubits_clock[n_clock - 1 - k]

circuit.append(cU(ctrl, *qubits_sys))

circuit.append(cirq.H.on_each(*qubits_clock))

meta = {

"qubits_clock": qubits_clock,

"qubits_sys": qubits_sys,

"qubit_anc": qubit_anc,

"n_clock": n_clock,

}

return circuit, meta

#Postselection & analysis

def extract_postselection(final_state, n_clock, n_sys):

dims = [2] * (n_clock + n_sys + 1)

tensor = final_state.reshape(dims)

amp_anc1 = tensor[..., 1]

P_anc1 = float(np.vdot(amp_anc1, amp_anc1).real)

if P_anc1 < 1e-30:

return P_anc1, None, None, None, None

slicer = tuple([0] * n_clock + [slice(None)] * n_sys)

amp_clk0_sys = amp_anc1[slicer].flatten()

P_clk0_and_anc1 = float(np.vdot(amp_clk0_sys, amp_clk0_sys).real)

P_clk0_given_anc1 = P_clk0_and_anc1 / P_anc1

return P_anc1, P_clk0_given_anc1, P_clk0_and_anc1, amp_anc1, amp_clk0_sys

def recover_solution_block(sys_amp_8, want="bottom"):

sys_amp_8 = np.asarray(sys_amp_8, dtype=np.complex128).reshape(8)

return sys_amp_8[4:8] if want.lower() == "bottom" else sys_amp_8[0:4]

def residuals(A, b, x):

r = A @ x - b

return float(np.linalg.norm(r)), r

#Projection onto constraints

def project_to_simplex_with_mass(v, mass):

"""

Euclidean projection onto {x >= 0, sum x = mass}.

"""

v = np.asarray(v, dtype=np.float64).reshape(-1)

n = v.size

if mass < 0:

raise ValueError("mass must be nonnegative")

if mass == 0:

return np.zeros(n, dtype=np.float64)

u = np.sort(v)[::-1]

cssv = np.cumsum(u)

rho = -1

theta = 0.0

for j in range(n):

t = (cssv[j] - mass) / (j + 1)

if u[j] - t > 0:

rho = j

theta = t

if rho == -1:

return np.full(n, mass / n, dtype=np.float64)

w = np.maximum(v - theta, 0.0)

# Lightweight numerical cleanup of the sum:

s = w.sum()

if abs(s - mass) > 1e-12 and s > 1e-30:

w *= (mass / s)

return w

# Scheme 1: Projected GD

def projected_gradient_descent(

A, b, x_init, *,

target_mass=0.05,

eps_norm=1e-6,

max_iters=2000,

alpha=None,           # None => auto 0.9/||A||_2^2

alpha_scale=0.9,

verbose=True,

print_every=50,

):

"""

Minimize f(x)=0.5||Ax-b||^2 over C={x>=0, sum=mass} using projected GD.

"""

A = np.asarray(A, dtype=np.float64)

b = np.asarray(b, dtype=np.float64).reshape(-1)

x = project_to_simplex_with_mass(x_init, target_mass)

if alpha is None:

smax = float(np.linalg.svd(A, compute_uv=False)[0])  # sigma_max

L = smax * smax

alpha = alpha_scale / max(L, 1e-30)

hist = []

for k in range(max_iters):

norm_r, rvec = residuals(A, b, x)

hist.append((k, norm_r, x.copy(), rvec.copy()))

if verbose and (k % print_every == 0 or k < 5):

print(f"    [PGD] iter={k:04d}  ||Ax-b||_2={norm_r:.6e}  alpha={alpha:.3e}  x={x}")

if norm_r <= eps_norm:

break

g = A.T @ (A @ x - b)

x = project_to_simplex_with_mass(x - alpha * g, target_mass)

return x, hist, alpha

def main():

print("=== HHL + Scheme1(PGD) Integrated (n_clock=8) ===")

n_clock = 8

target_mass = 0.05

eps_norm = 1e-6

max_outer_iters = 2000

A, b = get_chemical_matrix()

# Classical reference

x_classical = np.linalg.solve(A, b)

norm_class, r_class = residuals(A, b, x_classical)

print("--- [Self-check] Classical residual on original system ---")

print(f"    ||A x_classical - b||_2 = {norm_class:.6e}")

print(f"    Ax_classical - b = {r_class}")

print(f"    sum(x_classical) = {x_classical.sum():.12f}")

print(f"\n    Target Total Concentration: {target_mass:.6f} M")

# Preconditioning for HHL initial solve

A_prime, b_prime_quantum, r_scale, c_scale, b_norm = strong_preconditioning(A, b)

H = build_block_matrix(A_prime)

# HHL to get x_raw -> x0

print("--- [Step 2/3] Building & Simulating HHL Circuit ---")

embed = "top"

want_solution_in = "bottom"

circuit, meta = create_hhl_circuit(H, b_prime_quantum, n_clock=n_clock, embed=embed)

all_qubits = meta["qubits_clock"] + meta["qubits_sys"] + [meta["qubit_anc"]]

circuit = circuit + cirq.Circuit(cirq.I.on_each(*all_qubits))

sim = cirq.Simulator()

result = sim.simulate(circuit, qubit_order=all_qubits)

final_state = result.final_state_vector.astype(np.complex128)

# Postselection

n_sys = 3

P_anc1, P_clk0_given_anc1, P_clk0_and_anc1, amp_anc1, amp_clk0_sys = extract_postselection(

final_state, n_clock, n_sys

)

print("--- [Step 4] Postselection Diagnostics ---")

print(f"    P(anc=1)                = {P_anc1:.6e}")

print(f"    P(clk=0 | anc=1)        = {P_clk0_given_anc1:.6e}")

print(f"    P(clk=0, anc=1)         = {P_clk0_and_anc1:.6e}")

if P_clk0_and_anc1 < 1e-30:

raise RuntimeError("P(clk=0, anc=1) too small; cannot extract system amplitudes.")

sys_amp_8 = amp_clk0_sys / np.sqrt(P_clk0_and_anc1)

x4_raw = recover_solution_block(sys_amp_8, want=want_solution_in)

# Unscale back from preconditioning

x_raw_unscaled = x4_raw * c_scale

# Build physics-informed x0: nonneg + mass

x0 = np.real(x_raw_unscaled)

x0 = project_to_simplex_with_mass(x0, target_mass)

norm0, r0 = residuals(A, b, x0)

print("\n--- [Baseline] x0 (from HHL + projection) ---")

print(f"    x0 = {x0}")

print(f"    ||A x0 - b||_2 = {norm0:.6e}")

print(f"    Ax0 - b = {r0}")

print(f"    sum(x0) = {x0.sum():.12f}, min(x0) = {x0.min():.3e}")

# Outer PGD

print("\n--- [Scheme 1] Outer iteration (PGD) ---")

print(f"    eps_norm={eps_norm:.3e}, max_iters={max_outer_iters}")

x_star, hist, alpha_used = projected_gradient_descent(

A, b, x0,

target_mass=target_mass,

eps_norm=eps_norm,

max_iters=max_outer_iters,

alpha=None,

alpha_scale=0.9,

verbose=True,

print_every=50,

)

print(f"\n    [PGD] alpha_used = {alpha_used:.6e}")

x_star_clean = x_star.copy()

x_star_clean[np.abs(x_star_clean) < 1e-30] = 0.0

norm_star, r_star = residuals(A, b, x_star_clean)

print("\n--- [Result] x* after Scheme 1 (PGD) ---")

print(f"    x* = {x_star_clean}")

print(f"    ||A x* - b||_2 = {norm_star:.6e}")

print(f"    Ax* - b = {r_star}")

print(f"    sum(x*) = {x_star_clean.sum():.12f}, min(x*) = {x_star_clean.min():.3e}")

# Compare to classical

err0 = float(np.linalg.norm(x0 - x_classical))

err_star = float(np.linalg.norm(x_star_clean - x_classical))

print("\n--- [Comparison vs classical] ---")

print(f"    ||x0 - x_class||_2 = {err0:.6e}")

print(f"    ||x* - x_class||_2 = {err_star:.6e}")

names = ["A", "B", "C", "D"]

print("\n" + "=" * 92)

print(f"{'Species':<8} | {'Classical':<18} | {'x0 (HHL+proj)':<18} | {'x* (PGD, cleaned)':<18}")

print("-" * 92)

for i in range(4):

print(f"{names[i]:<8} | {x_classical[i]:<18.8e} | {x0[i]:<18.8e} | {x_star_clean[i]:<18.8e}")

print("-" * 92)

if __name__ == "__main__":

main()

Matlab

clc; clear;

% --- 1 ---

% Data input:  k1, k3, and k4 are taken from Table 2 in the main text (MS).

% The corresponding reverse/reconstructed constants k_1, k_3, and k_4

% are obtained based on Supplementary Eq. 1 in Supplementary Note 1.

k1  = 4e5;      k_1 = 14;

k2  = 4.5e3;    k_2 = 2.88e-4;

k3  = 1e-5;     k_3 = 4.5e-72;

k4  = 2.6e8;    k_4 = 3.66e36;

% --- 2 ---

% The following rows are constructed according to Eq. 14 in Section 2.2 (Methodology).

%  k1*x1 - c1*x3 = 0

c1 = k_1 * 1.31e-4;

%  -c2*x2 + c3*x4 = 0

c2 = k_4 * 1e-11 * 1.31e-4;

c3 = k4 * 3.9e-6;

% k3*x1 - c4*x4 = 0

c4 = k_3 * 1e-6 * 3.9e-6;

% --- 3 ---

% Step A

% x2 = (c3 / c2) * x4

ratio_2 = c3 / c2;

% Step B

% x1 = (c4 / k3) * x4

ratio_1 = c4 / k3;

% Step C

% x3 = (k1 / c1) * x1

ratio_3 = (k1 / c1) * ratio_1;

% Step D

% 20 * (x1 + x2 + x3 + x4) = 1  =>  Sum(x) = 0.05

total_ratio = ratio_1 + ratio_2 + ratio_3 + 1;

x4_exact = 0.05 / total_ratio;

% Step E:

x1_exact = ratio_1 * x4_exact;

x2_exact = ratio_2 * x4_exact;

x3_exact = ratio_3 * x4_exact;

X_raw = [x1_exact; x2_exact; x3_exact; x4_exact];

% --- 4 ---

decimals = 30;

X_final = round(X_raw, decimals);

% --- 5---

disp('--------------------------------');

disp('--------------------------------');

vars = {'x1', 'x2', 'x3', 'x4'};

for i = 1:4

if X_final(i) == 0

fprintf('%s = 0\n', vars{i});

else

fprintf('%s = %.4e\n', vars{i}, X_final(i));

end

end

disp('--------------------------------');

Kinetic reaction rate constant determination by Classical MATLAB

clc

clear all

options = optimoptions('fsolve','Display','none','PlotFcn',@optimplotfirstorderopt);

options.Algorithm = "levenberg-marquardt";

fun = @matrix;

x0 = [0,35*10^-3,7.52*10^-3,7.5*10^-3,1.83*10^-4,0,32.48*10^-3,0.25949*10^-3];

x = fsolve(fun,x0,options);

fprintf('the final concentration of Fe(II) is %f', x(1,1));

fprintf('the final concentration of H2O2 is %f', x(1,2));

fprintf('the final concentration of Fe(III) is %f', x(1,3));

fprintf('the final concentration of OH radical is %f', x(1,4));

fprintf('the reaction rate constante between Fe(IV)=O and MET is %f', x(1,5));

fprintf('the final concentration of ASC is %f', x(1,6));

fprintf('the final concentration of Fe(IV)=O is %f', x(1,7));

fprintf('the final concentration of MET is %f', x(1,8));

function F = matrix(x)

k1 = 5.7*10^2; % kinetic constant of equation (5) in Table 2

k2 = 1.0*10^6; % kinetic constant of equation (6) in Table 2

k3 = 4.5*10^3; % kinetic constant of equation (2) in Table 2

k4 = 7.9*10^9; % kinetic constant of equation (7) in Table 2

% The following rows are constructed according to Eq. 17 in Section 2.2 (Methodology).

T = 30*3600;  % reaction time

F(1) = x(1)-((-k1-k2)*x(1)*x(2)+k3*x(6)*x(3))*T-40*10^-3;

F(2) = x(2)-((-k1-k2)*x(1)*x(2))*T-100*10^-3;

F(3) = x(3)-((k1)*x(1)*x(2)-(k3)*x(6)*x(3))*T-0*10^-3;

F(4) = x(4)-((k1)*x(1)*x(2)-(k4)*x(6)*x(4))*T-0*10^-3;

F(5) = (0.05*10^-3)-(x(5)*x(8)*x(2)*(10^-3)*(10^-3)*(x(7)^2))*T-0*10^-3;

F(6) = x(6)-(-(k3)*x(6)*x(3)-(k4)*x(6)*x(4))*T-50*10^-3;

F(7) = x(7)-(k2*x(1)*x(2)-2*x(5)*x(8)*x(2)*(10^-3)*(10^-3)*(x(7)^2))*T-0*10^-3;

F(8) = x(8)-(-x(5)*x(8)*x(2)*(10^-3)*(10^-3)*(x(7)^2))*T-0.5*10^-3;

F(9) = x(1)+x(3)+x(7)-40*10^-3;

Microkinetic simulation by Google Quantum AI & Classical MATLAB

Google Quantum AI:

Products of Fenton reaction

import cirq

import numpy as np

import scipy.linalg

#  Problem setup

def get_chemical_matrix():

# Data input:  k2, k5, and k6 are taken from Table 2 in the main text (MS).

# The corresponding reverse/reconstructed constants k_2, k_5, and k_6

# are obtained based on Supplementary Eq. 1 in Supplementary Note 1.

# Kinetic constants

k2 = 4.5 * 10**3

k_2 = 2.88 * 10**4

k5 = 5.7 * 10**2

k_5 = 6.22 * 10**-23

k6 = 1 * 10**6

k_6 = 1.19 * 10**-5

# The following rows are constructed according to Eq. 15  in Section 2.2 (Methodology) of  MS.

row0 = [25.0, 25.0, 25.0, 0.0]

row1 = [k2 * 1.31 * 10**-11, -k_2 * 10**-6 * 0.05, 0.0, 1.0]

row2 = [-k_5 * 0.05, k5 * 1.31 * 10**-11 * 39.7 * 10**-3, 0.0, 1.0]

row3 = [0.0, k6 * 39.7 * 10**-3, -k_6, 0.0]

# The coefficient matrix A and vector b are assembled in accordance with Eq. 11 in Section 2.2 # (Methodology) of the MS.

A = np.array([row0, row1, row2, row3], dtype=np.float64)

b = np.array([1.0, 0.0, 0.0, 0.0], dtype=np.float64)

return A, b

def strong_preconditioning(A, b, iters=2000):

print("--- [Step 1] Running Sinkhorn-Knopp Preconditioning ---")

r = np.ones(A.shape[0], dtype=np.float64)

c = np.ones(A.shape[1], dtype=np.float64)

A_new = A.astype(np.float64).copy()

for _ in range(iters):

r_norms = np.linalg.norm(A_new, axis=1)

r_norms[r_norms == 0] = 1.0

A_new = np.diag(1.0 / r_norms) @ A_new

r *= (1.0 / r_norms)

c_norms = np.linalg.norm(A_new, axis=0)

c_norms[c_norms == 0] = 1.0

A_new = A_new @ np.diag(1.0 / c_norms)

c *= (1.0 / c_norms)

b_new = np.diag(r) @ b

b_norm = np.linalg.norm(b_new)

if b_norm < 1e-30:

raise ValueError("b' norm too small after scaling.")

b_quantum = (b_new / b_norm).astype(np.complex128)

print(f"    Balanced Matrix Condition Number: {np.linalg.cond(A_new):.4f}")

return A_new, b_quantum, r, c, float(b_norm)

def build_block_matrix(A):

n = A.shape[0]

Z = np.zeros((n, n), dtype=np.float64)

return np.vstack((np.hstack((Z, A)), np.hstack((A.T, Z)))).astype(np.float64)

# State preparation helpers

def embed_b_to_8d(b4, embed="top"):

b4 = np.asarray(b4, dtype=np.complex128).reshape(4)

b8 = np.zeros(8, dtype=np.complex128)

if embed.lower() == "top":

flag = 0

elif embed.lower() == "bottom":

flag = 1

else:

raise ValueError("embed must be 'top' or 'bottom'")

for vec in range(4):

b8[(flag << 2) | vec] = b4[vec]

nrm = np.linalg.norm(b8)

if nrm < 1e-30:

raise ValueError("Embedded b8 has ~0 norm.")

return b8 / nrm

def amplitude_embed_unitary(b8):

b8 = np.asarray(b8, dtype=np.complex128).reshape(8)

b8 = b8 / np.linalg.norm(b8)

M = np.eye(8, dtype=np.complex128)

M[:, 0] = b8

Q, _ = np.linalg.qr(M)

phase = np.vdot(Q[:, 0], b8)

if abs(phase) > 1e-12:

Q[:, 0] *= (phase / abs(phase)).conjugate()

return Q

#HHL circuit construction

def create_hhl_circuit(H, b4_quantum, *, n_clock=8, embed="top",

scale_t=0.99, scale_C=0.95, eig_cut=1e-9):

n_sys = 3

qubits_clock = [cirq.LineQubit(i) for i in range(n_clock)]

q_flag = cirq.LineQubit(n_clock)

q_v0   = cirq.LineQubit(n_clock + 1)

q_v1   = cirq.LineQubit(n_clock + 2)

qubits_sys = [q_flag, q_v0, q_v1]

qubit_anc = cirq.LineQubit(n_clock + 3)

eigvals = np.linalg.eigvalsh(H)

valid = eigvals[np.abs(eigvals) > eig_cut]

lam_min = float(np.min(np.abs(valid)))

lam_max = float(np.max(np.abs(valid)))

t = float(2 * np.pi / lam_max * scale_t)

C = float(lam_min * scale_C)

print(f"--- [Step 2] HHL Parameters: t={t:.6f}, C={C:.6e}, "

f"λ_min={lam_min:.6e}, λ_max={lam_max:.6e}, n_clock={n_clock} ---")

b8 = embed_b_to_8d(b4_quantum, embed=embed)

Uprep = amplitude_embed_unitary(b8)

circuit = cirq.Circuit()

circuit.append(cirq.MatrixGate(Uprep).on(*qubits_sys))

# QPE

circuit.append(cirq.H.on_each(*qubits_clock))

for k in range(n_clock):

dt = t * (2**k) / (2**n_clock)

U = scipy.linalg.expm(1j * H * dt).astype(np.complex128)

cU = cirq.MatrixGate(U).controlled()

ctrl = qubits_clock[n_clock - 1 - k]

circuit.append(cU(ctrl, *qubits_sys))

circuit.append(cirq.qft(*qubits_clock, inverse=True))

# Inversion rotation

N = 2**n_clock

two_pi_over_t = 2 * np.pi / t

for i in range(1, N):

phi = i / N

phi_wrapped = (phi - 1.0) if (phi >= 0.5) else phi

lam_est = two_pi_over_t * phi_wrapped

if abs(lam_est) < 1e-12:

continue

ratio = float(np.clip(C / lam_est, -1.0, 1.0))

theta = float(2 * np.arcsin(ratio))

bits = format(i, f"0{n_clock}b")

controls = []

for idx, bit in enumerate(bits):

q = qubits_clock[idx]

if bit == "0":

circuit.append(cirq.X(q))

controls.append(q)

circuit.append(cirq.ry(theta).on(qubit_anc).controlled_by(*controls))

for idx, bit in enumerate(bits):

if bit == "0":

circuit.append(cirq.X(qubits_clock[idx]))

# Inverse QPE

circuit.append(cirq.qft(*qubits_clock))

for k in range(n_clock - 1, -1, -1):

dt = -t * (2**k) / (2**n_clock)

U = scipy.linalg.expm(1j * H * dt).astype(np.complex128)

cU = cirq.MatrixGate(U).controlled()

ctrl = qubits_clock[n_clock - 1 - k]

circuit.append(cU(ctrl, *qubits_sys))

circuit.append(cirq.H.on_each(*qubits_clock))

meta = {

"qubits_clock": qubits_clock,

"qubits_sys": qubits_sys,

"qubit_anc": qubit_anc,

"n_clock": n_clock,

}

return circuit, meta

#  Postselection & analysis

def extract_postselection(final_state, n_clock, n_sys):

dims = [2] * (n_clock + n_sys + 1)

tensor = final_state.reshape(dims)

amp_anc1 = tensor[..., 1]

P_anc1 = float(np.vdot(amp_anc1, amp_anc1).real)

if P_anc1 < 1e-30:

return P_anc1, None, None, None, None

slicer = tuple([0] * n_clock + [slice(None)] * n_sys)

amp_clk0_sys = amp_anc1[slicer].flatten()

P_clk0_and_anc1 = float(np.vdot(amp_clk0_sys, amp_clk0_sys).real)

P_clk0_given_anc1 = P_clk0_and_anc1 / P_anc1

return P_anc1, P_clk0_given_anc1, P_clk0_and_anc1, amp_anc1, amp_clk0_sys

def recover_solution_block(sys_amp_8, want="bottom"):

sys_amp_8 = np.asarray(sys_amp_8, dtype=np.complex128).reshape(8)

return sys_amp_8[4:8] if want.lower() == "bottom" else sys_amp_8[0:4]

def residuals(A, b, x):

r = A @ x - b

return float(np.linalg.norm(r)), r

#  Projection onto constraints

def project_to_simplex_with_mass(v, mass):

"""

Euclidean projection onto {x >= 0, sum x = mass}.

"""

v = np.asarray(v, dtype=np.float64).reshape(-1)

n = v.size

if mass < 0:

raise ValueError("mass must be nonnegative")

if mass == 0:

return np.zeros(n, dtype=np.float64)

u = np.sort(v)[::-1]

cssv = np.cumsum(u)

rho = -1

theta = 0.0

for j in range(n):

t = (cssv[j] - mass) / (j + 1)

if u[j] - t > 0:

rho = j

theta = t

if rho == -1:

return np.full(n, mass / n, dtype=np.float64)

w = np.maximum(v - theta, 0.0)

s = w.sum()

if abs(s - mass) > 1e-12 and s > 1e-30:

w *= (mass / s)

return w

#  Scheme 1: Projected GD

def projected_gradient_descent(

A, b, x_init, *,

target_mass=0.05,

eps_norm=1e-6,

max_iters=2000,

alpha=None,

alpha_scale=0.9,

verbose=True,

print_every=50,

):

A = np.asarray(A, dtype=np.float64)

b = np.asarray(b, dtype=np.float64).reshape(-1)

x = project_to_simplex_with_mass(x_init, target_mass)

if alpha is None:

smax = float(np.linalg.svd(A, compute_uv=False)[0])

L = smax * smax

alpha = alpha_scale / max(L, 1e-30)

hist = []

for k in range(max_iters):

norm_r, rvec = residuals(A, b, x)

hist.append((k, norm_r, x.copy(), rvec.copy()))

if verbose and (k % print_every == 0 or k < 5):

print(f"    [PGD] iter={k:04d}  ||Ax-b||_2={norm_r:.6e}  alpha={alpha:.3e}  x={x}")

if norm_r <= eps_norm:

break

g = A.T @ (A @ x - b)

x = project_to_simplex_with_mass(x - alpha * g, target_mass)

return x, hist, alpha

def main():

print("=== HHL + Scheme1(PGD) Integrated (n_clock=8) [Fenton data] ===")

n_clock = 8

target_mass = 0.04

eps_norm = 1e-6

max_outer_iters = 2000

A, b = get_chemical_matrix()

# Classical reference

x_classical = np.linalg.solve(A, b)

norm_class, r_class = residuals(A, b, x_classical)

print("--- [Self-check] Classical residual on original system ---")

print(f"    ||A x_classical - b||_2 = {norm_class:.6e}")

print(f"    Ax_classical - b = {r_class}")

print(f"    sum(x_classical) = {x_classical.sum():.12f}")

print(f"\n    Target Total Concentration: {target_mass:.6f} M")

# Preconditioning for HHL initial solve

A_prime, b_prime_quantum, r_scale, c_scale, b_norm = strong_preconditioning(A, b)

H = build_block_matrix(A_prime)

# HHL to get x_raw -> x0

print("--- [Step 2/3] Building & Simulating HHL Circuit ---")

embed = "top"

want_solution_in = "bottom"

circuit, meta = create_hhl_circuit(H, b_prime_quantum, n_clock=n_clock, embed=embed)

all_qubits = meta["qubits_clock"] + meta["qubits_sys"] + [meta["qubit_anc"]]

circuit = circuit + cirq.Circuit(cirq.I.on_each(*all_qubits))

sim = cirq.Simulator()

result = sim.simulate(circuit, qubit_order=all_qubits)

final_state = result.final_state_vector.astype(np.complex128)

# Postselection

n_sys = 3

P_anc1, P_clk0_given_anc1, P_clk0_and_anc1, amp_anc1, amp_clk0_sys = extract_postselection(

final_state, n_clock, n_sys

)

print("--- [Step 4] Postselection Diagnostics ---")

print(f"    P(anc=1)                = {P_anc1:.6e}")

print(f"    P(clk=0 | anc=1)        = {P_clk0_given_anc1:.6e}")

print(f"    P(clk=0, anc=1)         = {P_clk0_and_anc1:.6e}")

if P_clk0_and_anc1 < 1e-30:

raise RuntimeError("P(clk=0, anc=1) too small; cannot extract system amplitudes.")

sys_amp_8 = amp_clk0_sys / np.sqrt(P_clk0_and_anc1)

x4_raw = recover_solution_block(sys_amp_8, want=want_solution_in)

# Unscale back from preconditioning

x_raw_unscaled = x4_raw * c_scale

# Build physics-informed x0: nonneg + mass

x0 = np.real(x_raw_unscaled)

x0 = project_to_simplex_with_mass(x0, target_mass)

norm0, r0 = residuals(A, b, x0)

print("\n--- [Baseline] x0 (from HHL + projection) ---")

print(f"    x0 = {x0}")

print(f"    ||A x0 - b||_2 = {norm0:.6e}")

print(f"    Ax0 - b = {r0}")

print(f"    sum(x0) = {x0.sum():.12f}, min(x0) = {x0.min():.3e}")

# Outer PGD

print("\n--- [Scheme 1] Outer iteration (PGD) ---")

print(f"    eps_norm={eps_norm:.3e}, max_iters={max_outer_iters}")

x_star, hist, alpha_used = projected_gradient_descent(

A, b, x0,

target_mass=target_mass,

eps_norm=eps_norm,

max_iters=max_outer_iters,

alpha=None,

alpha_scale=0.9,

verbose=True,

print_every=50,

)

print(f"\n    [PGD] alpha_used = {alpha_used:.6e}")

# Cosmetic cleanup for display

x_star_clean = x_star.copy()

x_star_clean[np.abs(x_star_clean) < 1e-30] = 0.0

norm_star, r_star = residuals(A, b, x_star_clean)

print("\n--- [Result] x* after Scheme 1 (PGD) ---")

print(f"    x* = {x_star_clean}")

print(f"    ||A x* - b||_2 = {norm_star:.6e}")

print(f"    Ax* - b = {r_star}")

print(f"    sum(x*) = {x_star_clean.sum():.12f}, min(x*) = {x_star_clean.min():.3e}")

# Compare to classical

err0 = float(np.linalg.norm(x0 - x_classical))

err_star = float(np.linalg.norm(x_star_clean - x_classical))

print("\n--- [Comparison vs classical] ---")

print(f"    ||x0 - x_class||_2 = {err0:.6e}")

print(f"    ||x* - x_class||_2 = {err_star:.6e}")

names = ["A", "B", "C", "D"]

print("\n" + "=" * 92)

print(f"{'Species':<8} | {'Classical':<18} | {'x0 (HHL+proj)':<18} | {'x* (PGD, cleaned)':<18}")

print("-" * 92)

for i in range(4):

print(f"{names[i]:<8} | {x_classical[i]:<18.8e} | {x0[i]:<18.8e} | {x_star_clean[i]:<18.8e}")

print("-" * 92)

if __name__ == "__main__":

main()

Conversion rate

import cirq

import numpy as np

import scipy.linalg

class StiffSystemHHLSimulation:

def __init__(self):

# Quantum Register Configuration

# n_clock: 14 qubits selected to maximize spectral resolution for small eigenvalues.

# n_sys: 2 qubits required for the 2x2 system (embedded in 4x4 block matrix).

self.n_clock = 14

self.n_sys = 2

self.n_ancilla = 1

# Qubit Allocation

self.qubits_clock = cirq.LineQubit.range(self.n_clock)

self.qubits_sys = cirq.LineQubit.range(self.n_clock, self.n_clock + self.n_sys)

self.qubit_ancilla = cirq.LineQubit(self.n_clock + self.n_sys)

def get_system_matrix(self):

# Data input:  k9 are taken from the result of Eq.17.

# The corresponding reverse/reconstructed constants k_9

# are obtained based on Supplementary Eq. 1 in Supplementary Note 1. .

# Kinetic constants

k9 = 1.83 * 10**-4

k_9 = 4.8 * 10**-9

# The following rows are constructed according to Eq. 16  in Section 2.2 (Methodology)

# Matrix Construction

# Row 0: Mass balance constraint [2000, 2000]

row0 = [2000.0, 2000.0]

# The coefficient matrix A and vector b are assembled in accordance with Eq. 11 in Section 2.2 # (Methodology) of the MS.

# Row 1: Equilibrium/Rate expression

# Term 1: k9 * [Reactants]

term1 = k9 * (35.4 * 10**-3)**2 * (39.7 * 10**-3) * (10**-3)**2

# Term 2: -k_9 * [Products]

term2 = -k_9 * (5.56 * 10**-3)**2

row1 = [term1, term2]

A = np.array([row0, row1])

b = np.array([1.0, 0.0])

return A, b

def preprocess_matrix(self, A, b):

#Applies aggressive Sinkhorn-Knopp algorithm to rescale matrix A.

r = np.ones(A.shape[0])

c = np.ones(A.shape[1])

A_new = A.copy()

for _ in range(5000):

r_norms = np.linalg.norm(A_new, axis=1)

r_norms[r_norms == 0] = 1.0

D_r = np.diag(1.0 / r_norms)

A_new = D_r @ A_new

r = r * (1.0 / r_norms)

c_norms = np.linalg.norm(A_new, axis=0)

c_norms[c_norms == 0] = 1.0

D_c = np.diag(1.0 / c_norms)

A_new = A_new @ D_c

c = c * (1.0 / c_norms)

b_new = np.diag(r) @ b

b_norm = np.linalg.norm(b_new)

b_normalized = b_new / b_norm

return A_new, b_normalized, r, c, b_norm

def block_encoding(self, A):

"""

Embeds the non-Hermitian matrix A into a Hermitian matrix H.

Structure: H = [[0, A], [A^T, 0]]

"""

n = A.shape[0]

zeros = np.zeros((n, n))

H = np.vstack((np.hstack((zeros, A)), np.hstack((A.T, zeros))))

return H

def build_circuit(self, H, t, C):

"""Constructs the quantum circuit for the HHL algorithm."""

circuit = cirq.Circuit()

# --- Stage 1: Quantum Phase Estimation (QPE) ---

circuit.append(cirq.H.on_each(self.qubits_clock))

# Controlled-Unitary evolution: U = e^{iHt}

for i in range(self.n_clock):

exponent = i

power = 2**exponent

time_step = t * power / (2**self.n_clock)

U_mat = scipy.linalg.expm(1j * H * time_step)

c_U = cirq.MatrixGate(U_mat).controlled()

circuit.append(c_U(self.qubits_clock[self.n_clock - 1 - i], *self.qubits_sys))

# Inverse Quantum Fourier Transform (IQFT)

circuit.append(cirq.qft(*self.qubits_clock, inverse=True))

# --- Stage 2: Eigenvalue Inversion (Controlled Rotation) ---

for i in range(1, 2**self.n_clock):

lambda_est = (i * 2 * np.pi) / (t * (2**self.n_clock))

# Phase wrapping correction

if lambda_est > np.pi / t:

lambda_est -= (2 * np.pi / t)

# Filter near-zero eigenvalues to avoid singularities

if abs(lambda_est) < 1e-9:

continue

ratio = C / lambda_est

ratio = np.clip(ratio, -1.0, 1.0)

theta = 2 * np.arcsin(ratio)

if abs(theta) < 1e-6:

continue

# Bitwise control logic

bin_str = format(i, f'0{self.n_clock}b')

controls = []

x_gates = []

for idx, bit in enumerate(bin_str):

q = self.qubits_clock[idx]

if bit == '1':

controls.append(q)

else:

x_gates.append(cirq.X(q))

controls.append(q)

circuit.append(x_gates)

circuit.append(cirq.ry(theta).on(self.qubit_ancilla).controlled_by(*controls))

circuit.append(x_gates)

# --- Stage 3: Inverse QPE---

circuit.append(cirq.qft(*self.qubits_clock))

for i in range(self.n_clock - 1, -1, -1):

exponent = i

power = 2**exponent

time_step = -1 * t * power / (2**self.n_clock)

U_mat = scipy.linalg.expm(1j * H * time_step)

c_U = cirq.MatrixGate(U_mat).controlled()

circuit.append(c_U(self.qubits_clock[self.n_clock - 1 - i], *self.qubits_sys))

circuit.append(cirq.H.on_each(self.qubits_clock))

return circuit

def run(self):

print("=== HHL Quantum Simulation: High-Stiffness System ===")

# 1. Classical Solution

A, b = self.get_system_matrix()

max_el = np.max(np.abs(A))

min_el = np.min(np.abs(A[np.nonzero(A)]))

stiffness_ratio = max_el / min_el

print("  [Analysis] Matrix Stiffness Metrics:")

print(f"    > Max Element: {max_el:.4e}")

print(f"    > Min Element: {min_el:.4e}")

print(f"    > Stiffness Ratio: ~1e{int(np.log10(stiffness_ratio))}")

x_classical = np.linalg.solve(A, b)

target_mass = np.sum(x_classical)

# 2. Preprocessing

print("  [Status] Applying Strong Bilateral Preconditioning...")

A_prime, b_prime, r_scale, c_scale, _ = self.preprocess_matrix(A, b)

H = self.block_encoding(A_prime)

# 3. Parameter Optimization

eigvals = np.linalg.eigvalsh(H)

valid_eigs = eigvals[np.abs(eigvals) > 1e-12]

lambda_max = np.max(np.abs(valid_eigs))

lambda_min = np.min(np.abs(valid_eigs))

# Heuristic tuning

t = 2 * np.pi / lambda_max * 0.99

C = lambda_min

print(f"  [Parameters] Evolution time t: {t:.4f}, Constant C: {C:.4e}")

# 4. Circuit Construction

print("  [Status] Constructing Quantum Circuit...")

circuit = self.build_circuit(H, t, C)

# --- Visualization ---

print("\n" + "="*40)

print("      QUANTUM CIRCUIT TOPOLOGY")

print("="*40)

print(f"  > Depth (Moments): {len(circuit)}")

print(f"  > Total Operations: {len(list(circuit.all_operations()))}")

print("-" * 40)

try:

if len(circuit) > 50:

print("(Display truncated: Showing first 50 moments)")

print(circuit[:50].to_text_diagram(transpose=True))

print("\n... [Circuit continues] ...\n")

else:

print(circuit.to_text_diagram(transpose=True))

except Exception as e:

print(f"Visualization skipped: {e}")

print("="*40 + "\n")

# 5. State Initialization

full_state_size = 2**(self.n_clock + self.n_sys + self.n_ancilla)

initial_state = np.zeros(full_state_size, dtype=np.complex64)

# Map normalized vector b_prime to system qubits

for i, val in enumerate(b_prime):

target_idx = i * 2

initial_state[target_idx] = val

# 6. Execution (Statevector Simulation)

print("  [Status] Executing Statevector Simulation...")

simulator = cirq.Simulator()

result = simulator.simulate(circuit, initial_state=initial_state)

# 7. Post-Processing

final_state = result.final_state_vector

dims = [2] * (self.n_clock + self.n_sys + self.n_ancilla)

state_tensor = final_state.reshape(dims)

# Post-selection: Measure Clock=0, Ancilla=1

selector = [0]*self.n_clock + [slice(None)]*self.n_sys + [1]

target_vec_4d = state_tensor[tuple(selector)].flatten()

# Extract solution from block encoding (lower half of 4x4 block)

x_quantum_raw = target_vec_4d[2:]

# Rescaling and Normalization

x_unscaled = np.abs(np.real(x_quantum_raw)) * c_scale

current_sum = np.sum(x_unscaled)

if current_sum > 1e-20:

x_final = x_unscaled * (target_mass / current_sum)

else:

x_final = np.zeros_like(x_unscaled)

return x_classical, x_final

def report_results(self, x_classical, x_quantum):

print("\n" + "="*85)

print(f"{'Variable':<10} | {'Classical (Truth)':<22} | {'Quantum (HHL)':<22} | {'Status'}")

print("-" * 85)

labels = ["c(1)", "c(2)"]

for i in range(2):

c_val = x_classical[i]

q_val = x_quantum[i]

# Determine dominance status

if q_val > np.max(x_quantum) * 0.1:

status = "Dominant Species"

else:

status = "Trace Component"

print(f"{labels[i]:<10} | {c_val:<22.8e} | {q_val:<22.8e} | {status}")

print("-" * 85)

# Derived Parameter 'n'

# n represents the mole fraction of Species 1 relative to a reference capacity (0.005)

n_classical = x_classical[0] / 0.005

n_quantum = x_quantum[0] / 0.005

print(f"{'Param n':<10} | {n_classical:<22.4f} | {n_quantum:<22.4f} | {'Derived Metric'}")

print("-" * 85)

# Validation Metrics

l2_err = np.linalg.norm(x_quantum - x_classical)

print(f"Validation Metrics:")

print(f"  > L2 Error Norm:       {l2_err:.4e}")

# Strict validation threshold due to high stiffness

if abs(n_quantum - n_classical) < 1e-3:

print("\nResult: VALIDATION SUCCESSFUL. Quantum simulation accurately recovered parameter 'n'.")

else:

print("\nResult: DEVIATION DETECTED.")

if __name__ == "__main__":

sim = StiffSystemHHLSimulation()

x_c, x_q = sim.run()

sim.report_results(x_c, x_q)

Matlab:

Products of Fenton reaction

clear all

k2 = 4.5*10^3;

k_2 = 2.88*10^4;

k5 = 5.7*10^2;

k_5 = 6.22*10^-23;

k6 = 1 *10^6;

k_6 = 1.19*10^-5;

A = [ 25,  25,  25,  0;

k2*1.31*10^-11, -k_2*10^-6*0.05,  0,  1;

-k_5*0.05,  k5*1.31*10^-11*39.7*10^-3, 0, 1;

0,   k6*39.7*10^-3,  -k_6, 0;];

b = [1;

0;

0;

0];

X = A \ b;

Conversion rate

clear all

k2 = 4.5*10^3;

k_2 = 2.88*10^4;

k5 = 5.7*10^2;

k_5 = 6.22*10^-23;

k6 = 1 *10^6;

k_6 = 1.19*10^-5;

A = [ 25,  25,  25,  0;

k2*1.31*10^-11, -k_2*10^-6*0.05,  0,  1;

-k_5*0.05,  k5*1.31*10^-11*39.7*10^-3, 0, 1;

0,   k6*39.7*10^-3,  -k_6, 0;];

b = [1;

0;

0;

0];

X = A \ b;

## Supplementary Table 1. Summary of the estimated kinetic reaction rate constants and Gibbs free energy for Eqs. 8-10 in Table 2.

| Supplementary Equation | Chemical equations | Kinetic reaction rate constants (k) (M-1 s-1) | Gibbs free energy (∆G) (kcal/mol) |
| --- | --- | --- | --- |
| (1) | ASC+O2=2H++O2∙-+A·- | 1×10-5 | -90.5 |
| (2) | 2[FeIV=O]2+ + MET+ H2O2 + 2H+ = 2Fe3+ + HCyA + CH4 + H2O | 1.83×10-4 | -6.8 |
| (3) | ASC + Fe2+ + H2O2 = Fe3++A·- + 2H2O | 5.7×102 | -1.7 |

## Supplementary Table 2. Estimated Chemical Equilibrium constant (K)

| Parameter | Concentration (mmol/L) | Chemical Equilibrium constant (K) |
| --- | --- | --- |
| Fe2+ | 0.0 | 3.8×104 |
| H2O2 | 39.7 |  |
| Fe3+ | 5.5 |  |
| OH radical | 2.0×10-3 |  |
| ASC | 0.0 |  |
| [FeIV=O]2+ | 35.4 |  |
| MET | 0.317 |  |

- Note: These results were obtained from “Kinetic reaction rate constant determination by Classical MATLAB” in Supplementary Note 6.

The simulation conditions used assumed a temperature = 25°C, as indicated in Supplementary Table 1. To investigate the effect of temperature within the range from 25 to 37°C, we simulated objective 1 in this temperature range using the Google Quantum AI platform. The results indicated that different temperatures did not affect the dominant reaction mechanism between ASC and Fe3+, which is the redox reaction. The simulation results are presented in Supplementary Table 3.

## Supplementary Table 3. Changes in reactant/product concentration with temperature increase from 25°C to 37°C

|  | ASC (mol/L) | A·-(mol/L) | H2O2 (mol/L) | HO2- (mol/L) |
| --- | --- | --- | --- | --- |
| 25℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 1.06E-20 |
| 26℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 1.31E-20 |
| 27℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 1.63E-20 |
| 28℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 2.02E-20 |
| 29℃ | 8.65E-56 | 5.00E-02 | 5.55E-18 | 2.50E-20 |
| 30℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 3.07E-20 |
| 31℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 3.79E-20 |
| 32℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 4.67E-20 |
| 33℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 5.74E-20 |
| 34℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 7.33E-20 |
| 35℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 8.66E-20 |
| 36℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 1.06E-19 |
| 37℃ | 0.00E+00 | 5.00E-02 | 0.00E+00 | 1.30E-19 |

Supplementary Figure 1 summarizes the impact of oxygen concentration, varying from 0μM to 1000μM, on the relative importance of the redox and catalytic reactions described in the Methodology section of the paper. The results were obtained by conducting the simulations on the Google Quantum AI platform. A·- is the principal product of the redox reaction, and the results indicated that [A·-] remained constant regardless of variations in the oxygen concentration. Additionally, while the oxygen concentration fluctuations caused an increase in the concentration of the catalytic reaction product [HO2∙-], they did not affect the dominant reaction. Hence, variations in the oxygen concentration spanning from 0 to 1000 μM did not alter the conclusion that the predominant reaction was oxidation.

## Supplementary Figure 1. The variation of catalyst and redox reactions with oxygen concentration from 0 μM to 1000 μM

## References

1.	F. Althoff, A. Jugold and F. Keppler, Methane formation by oxidation of ascorbic acid using iron minerals and hydrogen peroxide, Chemosphere, 2010, 80, 286-292.