This Journal is ©The Royal Society of Chemistry 2003

Table S1. Certified reference materials related to the priority list

| Name of priority substance | CRM-Identifier | Producer | Description | Certified value | Uncertainty | Reference | Non PS certified |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alachlor | SRM-3070 | NIST | Calibration solution in acetone | 24.0mg/kg | 0.7mg/kg | http//ts.nist.gov/ts | bromacil; butachlor; metolachlor; metribuzin; prometon |
| Anthracene | SRM-1941a | NIST | Marine sediment | 184.0ppb | 8% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in acetonitrile | 0.000102% | 4% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 0.69µg/kg | 0.2µg/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 0.38ppm | 40% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 1.1ppm | 40% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | DPAC-1 | NRC-CNRC, Canada | deuterated calibration solution toluene | 13.0ppm | N/A | www.cm.inms.nrc.ca | 1-methylnaphthalene-D10; biphenyl-D10; acenaphtylene-D8; acenaphthene-D10; dibenzofuran-D8; fluorene-D10; dibenzo(b,d)thiophene-D8, phenanthrene-D10; pyrene-D10; benz(a)anthracene-D12; chrysene-D12; 9-methylanthracene-D12, coronene-D12, dibenzo(a,i)pyrene-D14 |
|  | HS-3B | NRC-CNRC, Canada | Marine sediment | 2.76ppm | 2% | www.cm.inms.nrc.ca | 1-methylnaphthalene, biphenyl, acenaphtylene, acenaphthene, dibenzofuran, fluorene, dibenzothiophene, phenanthrene, pyrene, benz(a)anthracene, chrysene, benzo(b,k)fluoranthene, dibenz(a, h)anthracene, coronene, dibenzo(a,i)pyrene |
|  | HS-4B | NRC-CNRC, Canada | Marine sediment | 0.46ppm | 20% | Fernandez et al.54 | 1-methylnaphthalene; biphenyl; acenaphtylene; acenaphthene; dibenzofuran; fluorene; dibenzothiophene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(b,k)fluoranthene; dibenz(a, h)anthracene; coronene; dibenzo(a,i)pyrene |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 1.2.0ppm | 30% | www.cm.inms.nrc.ca | phenanthrene, pyrene, benz(a)anthracene, benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 0.11ppm | 20% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5- trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5- tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 41.0ppb | 20% | www.cm.inms.nrc.ca | acenaphtylene; acenaphtene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| Atrazine | LGC-1603 | LGC-Promochem | Pure compound | 99.3% | N/A | www.lgc.co.uk | non |
|  | LGC-1004 | LGC-Promochem | Calibration solution in water | 26.7% | 8% | www.lgc.co.uk | trietazine; terbuthylazine |
|  | SRM-3070 | NIST | Calibration solution in acetone | 39.2mg/kg | 0.8mg/kg | http//ts.nist.gov/ts | bromacil; butachlor; metolachlor; metribuzin; prometon |
| Benzene | DMR-19a | CENAM, Mexico | Calibration solution in hexane | 1.0pmm | N/A | http://www.comar.bam.de/ | toluene; o-xylene |
|  | SRM-3069 | NIST | Calibration solution in methanol | 0.01001g/g | 0.00007g/g | http//ts.nist.gov/ts | non |
| Brominated diphenylethers | non |  |  |  |  |  |  |
| Cadmium and its compounds | BCR-320 | European Commission | River sediment | 0.533mg/kg | 0.026mg/kg | www.irmm.jrc.be | As; Cr; Cu; Sc; Se; Zn |
|  | BCR-060 | European Commission | Aquatic plant | 2.2mg/kg | 0.1mg/kg | www.irmm.jrc.be | Al; Cu; Mn; Zn |
|  | BCR-279 | European Commission | Sea lettuce | 0.274mg/kg | 0.022mg/kg | Griepink et al.55 Muntau 56 | As; Cu; Se; Zn |
|  | BCR-278R | European Commission | Mussel Tissue | 0.348mg/kg | 0.007mg/kg | Papadakis et al.57 | As; Cr; Cu; Mn; Se; Zn |
|  | BCR-414 | European Commission | Plankton | 0.383mg/kg | 0.014mg/kg | Quevauviller et al.58 Muntau 56 | As; Cr; Cu; Mn; Se; V; Zn |
|  | BCR-422 | European Commission | Cod muscle | 0.017mg/kg | 0.002mg/kg | www.irmm.jrc.be | As; Cu; Fe; I; Mn; Se; Zn |
|  | BCR-609 | European Commission | Ground water | 0.164µg/kg | 0.012µg/kg | www.irmm.jrc.be | Al; As; Cu |
|  | BCR-610 | European Commission | Ground water | 2.94µg/kg | 0.08µg/kg | www.irmm.jrc.be | Al; As; Cu |
|  | BCR-403 | European Commission | Sea water | 0.175nmol/kg | 0.018nmol/kg | www.irmm.jrc.be | Al; As; Cu; Mo; V; Zn |
|  | BCR-505 | European Commission | Estuarine water | 0.8nmol/kg | 0.04nmol/kg | www.irmm.jrc.be | Cu; Zn |
|  | SRM-2704 | NIST | River sediment | 0.000345% | 7% | http//ts.nist.gov/ts | Al; Ca; C; Fe; Mg; P; K; Si; Na; S; Ti; Sb; As; Cr; Co; Cu; Li; Mn; Se; Tl; U; V; Zn |
|  | SRM-1646a | NIST | Estuarine sediment | 0.0000148% | 5% | http//ts.nist.gov/ts | Al; Ca; Fe; Mg; P; As; Mn; V; Zn; K; Si; Na; S; Ti; Cr; Cu; Se |
|  | 6690-93 | Research and Control Centre of Water, Russia | Spectrometric standard in water | 0.0993% | 1% | http://www.comar.bam.de/ | non |
|  | 7325-96 | Research and Control Centre of Water, Russia | Spectrometric multi-element standard in water | 0.0101% | 2% | http://www.comar.bam.de/ | Al; As; Co; Cr; Cu |
|  | GSO-5222-90 | SCTB-EP, Ukraine | Spectrometric standard in HNO3/water | 0.097% | 1% | http://www.comar.bam.de/ | non |
|  | NIES-2 | NIES, Japan | Pond sediment | 0.82ppm | 8% | www.nies.go.jp | Al; Fe; Ca; K; Na; Zn; Cu; Cr; Co; As |
|  | NIES-9 | NIES, Japan | Sargasso sediment | 0.15ppm | 20% | www.nies.go.jp | K; Na; Ca; Mg; Sr; Fe; As; Rb; Mn; Zn; Cu; V; Ag; Co |
|  | JK-0012 Cd 1000 | CITI, Japan | Calibration solution in aqueous HNO3 | 0.1% | 0.3% | http://www.comar.bam.de/ | non |
|  | GBW-08602 | NRCCRM, China | Water | 0.1ppm | 0.002% | http://www.comar.bam.de/ | non |
|  | GBW-08607 | NRCCRM, China | Water | 0.1ppm | 2% | http://www.comar.bam.de/ | Cu; Cr; Zn |
|  | GBW-08608 | NRCCRM, China | Water | 10.0ppb | 4% | http://www.comar.bam.de/ | Cu; Cr; Zn |
|  | GBW-08301 | NRCCRM, China | River sediment | 2.45ppm | 10% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-08571 | NRCCRM, China | Mussel Tissue | 4.5ppm | 10% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-07314 | NRCCRM, China | Offshore marine sediment | 0.2ppm | 20% | http://www.comar.bam.de/ | Cu; Zn; Cr; As; Se; Co; Sr; Ba |
|  | SLRM-12-2-02 | pb-anal, Slovakia | Green algae | 44.8ppb | 7% | http://www.comar.bam.de/ | As; Ca; Co; Cr; Cu; Fe; K; Mg; Mn; Na; P; Zn |
|  | SLRM-12-3-10 | Research Institute for Irrigation, Slovakia | Water (acidified) | 0.005ppm | 10% | www.cm.inms.nrc.ca | Ag; Al; As; Co; Cr; Cu; Fe; K; Mn; Na; Zn |
|  | MESS-2 | NRC-CNRC, Canada | Marine sediment | 0.24ppm | 4% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; C; S |
|  | CASS-3 | NRC-CNRC, Canada | Nearshore seawater | 0.03ppb | 20% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Mo; Zn |
|  | SLEW-2 | NRC-CNRC, Canada | Estuarine water | 0.019ppb | 10% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Zn |
|  | DORM-2 | NRC-CNRC, Canada | Dogfish muscle | 0.043ppm | 20% | Padberg et al.59 Goessler et al.60 | Al; As; Co; Cr; Cu; Fe; Mn; Se; Ag |
|  | LUTS-1 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 2.12ppm | 7% | www.cm.inms.nrc.ca | As; Ca; Cr; Cu; Co; Fe; Mg; Mn; K; Ag; Se; Sr; Zn |
|  | TORT-2 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 26.7ppm | 2% | Padberg et al.59 | As; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | NASS-5 | NRC-CNRC, Canada | Ocean seawater | 0.023ppb | 20% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Mo; Zn |
|  | SLRS-4 | NRC-CNRC, Canada | Riverine water | 0.012ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Sr; U; V; Zn; Ca; Mg; K; Na |
|  | HISS-1 | NRC-CNRC, Canada | Marine sediment | 0.024ppm | 40% | www.cm.inms.nrc.ca | As; Be; Cr; Cu; Li; Mn; Se; Ag; Sr; V; Zn |
|  | PACS-2 | NRC-CNRC, Canada | Harbour sediment | 2.11ppm | 7% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; S |
|  | TM-23.2 | NRC-CNRC, Canada | Lake water | 2.6ppb | 30% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | TM-24 | NRC-CNRC, Canada | Water | 12.5ppb | 30% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be |
|  | TM-26.2 | NRC-CNRC, Canada | Lake water | 6.8ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; Tl; V; Zn |
|  | TM-27 | NRC-CNRC, Canada | Water | 1.0ppb | 90% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Li; Be |
|  | TM-27.2 | NRC-CNRC, Canada | Water | 1.2ppb | 40% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Ag; Sr; V; Zn |
|  | TM-28 | NRC-CNRC, Canada | Lake water | 1.2ppb | 40% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Li; Mn; Mo; Se; Sr; V; Zn |
|  | TMRAIN-95 | NRC-CNRC, Canada | Rain water | 0.48ppb | 30% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Bi; Li; Be; U; Tl |
|  | TMDA-51.2 | NRC-CNRC, Canada | Lake water | 25.1ppb | 100% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; Tl; U; V; Zn |
|  | TMDA-52.2 | NRC-CNRC, Canada | Lake water | 91.4ppb | 100% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be; U |
|  | TMDA-53.2 | NRC-CNRC, Canada | Lake water | 122.0ppb | 100% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Cu; Fe; Mn; Mo; Se; Sr; U; V; Zn |
|  | TMDA-54.2 | NRC-CNRC, Canada | Lake water | 185.0ppb | 9% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be; U |
| C10-13 chloroalkanes | non |  |  |  |  |  |  |
| Chlorfenvinphos | LGC-1003 | LGC-Promochem | Calibration solution in acetonitrile | 9.7ppm | 3% | www.lgc.co.uk | chlorpyrifos; diazinon; ethion; fenthion; mecarbam; parathion-methyl; triazophos |
| Chlorpyrifos | LGC-1003 | LGC-Promochem | Calibration solution in acetonitrile | 9.3ppm | 3% | www.lgc.co.uk | chlorpyrifos; diazinon; ethion; fenthion; mecarbam; parathion-methyl; triazophos |
| 1,2-Dichloroethane | 7332-96 | Research and Control Centre of Water, Russia | Pure compound | 99.93% | 0.01% | http://www.comar.bam.de/ | non |
|  | SLRM-10-2-08 | Slovak Institute of Metrology | Calibration solution in methanol | 984.0ppm | 6% | http://www.comar.bam.de/ | non |
|  | SRM-3012 | NIST | Calibration solution in methanol | 0.010039g/g | 0.000071g/g | http//ts.nist.gov/ts | non |
| Dichloromethane | non |  |  |  |  |  |  |
| Di(2-ethylhexyl)phthalate (DEHP) | non |  |  |  |  |  |  |
| Diuron | LGC-1403 | LGC-Promochem | Pure compound | 99.95% | N/A | www.lgc.co.uk | non |
|  | LGC-1005 | LGC-Promochem | Calibration solution in water | 26.1ppb | 8% | www.lgc.co.uk | chlortoluron |
| Endosulfan | LGC-1119 | LGC-Promochem | Pure compound | 99.70% | N/A | www.lgc.co.uk | non |
|  | SRM-3069 | NIST | Calibration solution in acetone | 4.66mg/kg | 0.1mg/kg | http//ts.nist.gov/ts | aldrin; cis-chlordane; trans-chlordane; 4,4'-DDE; 4,4'-DDD; 4,4'-DDT; dieldrin; endrin; heptachlor; heptachlorepoxide; hexacholorocyclopentadiene; methoxichlor; cis-nonachlor; propachlor |
| Hexachlorobenzene | SRM-1941a | NIST | Marine sediment | 70.0ppb | 40% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-3069 | NIST | Calibration solution in acetone | 4.22mg/kg | 0.12mg/kg | http//ts.nist.gov/ts | aldrin; cis-chlordane; trans-chlordane; 4,4'-DDE; 4,4'-DDD; 4,4'-DDT; dieldrin; endrin; heptachlor; heptachlorepoxide; hexacholorocyclopentadiene; methoxichlor; cis-nonachlor; propachlor |
|  | LGC-1131 | LGC-Promochem | Pure compound | 99.9% | N/A | www.lgc.co.uk | non |
|  | LGC-1000 | LGC-Promochem | Calibration solution in 2,2,4-Trimethylpentane | 10.7 ppm | N/A | www.lgc.co.uk | p,p'-DDT; o,p'-DDT; p,p'-TDE; alpha-BHC; gamma-BHC; dieldrin; p,p'-DDE; beta-BHC |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 5.4ppb | 40% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene;benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 200.6ppb | 7% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-3 | NRC-CNRC, Canada | River sediment | 279.0ppb | 10% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; total PCB |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 98.0ppb | 9% | www.cm.inms.nrc.ca | acenaphtylene; acenaphtene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | DMR-20a | CENAM, Mexico | Calibration solution in hexane | 0.04ppm | N/A | http://www.comar.bam.de/ | heptachlor, aldrin, dechlorane |
| Hexachlorobutadiene | EC-1 | NRC-CNRC, Canada | Harbour sediment | 0.66ppb | 90% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 21.3ppb | 8% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-3 | NRC-CNRC, Canada | River sediment | 61.0ppb | 10% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; total PCB |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 21.0ppb | 100% | www.cm.inms.nrc.ca | acenaphtylene; acenaphtene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| Hexachlorocyclohexane | LGC-1104 | LGC-Promochem | Pure compound | 99.70% | N/A | www.lgc.co.uk | non |
| gamma-isomer | SLRM-10-2-08 | Slovak Institute of Metrology | Calibration solution in methanol | 998.0ppm | 6% | http://www.comar.bam.de/ | non |
|  | DMR-20a | CENAM, Mexico | Calibration solution in hexane | 0.04ppm | N/A | http://www.comar.bam.de/ | heptachlor; aldrin; dechlorane |
|  | BCR-598 | European Commission | Cod liver oil | 23.0µg/kg | 4µg.kg | www.irmm.jrc.be | alpha-HCH; beta-HCH; HCB; gamma-chlordane; alpha-chlordane; oxychlordane; transnonachlor; dieldrin; p,p'-DDE; o,p'-DDD; p,p'-DDD; p,p'-DDT |
|  | SRM-3069 | NIST | Calibration solution in acetone | 4.22mg/kg | 0.12mg/kg | http//ts.nist.gov/ts | non |
| Isoproturon | LGC-1406 | LGC-Promochem | Pure compound | 99.5% | N/A | www.lgc.co.uk | non |
|  | LGC-1005 | LGC-Promochem | Calibration solution in water | 30.5ppb | 7% | www.lgc.co.uk | chlortoluron |
| Lead and its compounds | BCR-320 | European Commission | River sediment | 42.3mg/kg | 1.6mg/kg | www.irmm.jrc.be | As; Cr; Cu; Sc; Se; Zn |
|  | BCR-060 | European Commission | Aquatic plant | 63.8mg/kg | 3.2mg/kg | www.irmm.jrc.be | Al; Cu; Mn; Zn |
|  | BCR-279 | European Commission | Sea lettuce | 13.48mg/kg | 0.36mg/kg | Griepink et al.55 Muntau 56 | As; Cu; Se; Zn |
|  | BCR-278R | European Commission | Mussel Tissue | 2.0mg/kg | 0.04mg/kg | Papadakis et al.57 | As; Cr; Cu; Mn; Se; Zn |
|  | BCR-414 | European Commission | Plankton | 3.97mg/kg | 0.19mg/kg | Quevauviller et al.58 Muntau 56 | As; Cr; Cu; Mn; Se; V; Zn |
|  | BCR-422 | European Commission | Cod muscle | 0.085mg/kg | 0.015mg/kg | www.irmm.jrc.be | As; Cu; Fe; I; Mn; Se; Zn |
|  | BCR-609 | European Commission | Ground water | 1.63ug/kg | 0.04ug/kg | www.irmm.jrc.be | Al; As; Cu |
|  | BCR-610 | European Commission | Ground water | 7.78µg/kg | 0.13µg/kg | www.irmm.jrc.be | Al; As; Cu |
|  | BCR-403 | European Commission | Sea water | 0.117nmol/kg | 0.025nmol/kg | www.irmm.jrc.be | Al; As; Cu; Mo; V; Zn |
|  | SRM-2704 | NIST | River sediment | 0.0161% | 10% | http//ts.nist.gov/ts | Al; Ca; C; Fe; Mg; P; K; Si; Na; S; Ti; Sb; As; Cr; Co; Cu; Li; Mn; Se; Tl; U; V; Zn |
|  | SRM-1646a | NIST | Estuarine sediment | 0.00117% | 10% | Mao et al.61 | Al; Ca; Fe; Mg; P; As; Mn; V; Zn; K; Si; Na; S; Ti; Cr; Cu; Se |
|  | LGC-6010 | LGC-Promochem | Hard drinking water | 97.0ppb | 2% | www.lgc.co.uk | Al; Ag; As; Ba; Cr; Fe; Mn; Se; Sb; Zn; Ca; K; Mg; Na |
|  | LGC-6011 | LGC-Promochem | Soft drining water | 50.0ppb | 4% | www.lgc.co.uk | Al; Ag; Ba; Cr; Fe; Mn; Zn; Ca; K; Mg; Na |
|  | 7012-93 | Research and Control Centre of Water, Russia | Spectrometric standard in water | 0.0996% | 1% | http://www.comar.bam.de/ | non |
|  | 7272-96 | Research and Control Centre of Water, Russia | Spectrometric multi-element standard in water | 0.0098% | 2% | http://www.comar.bam.de/ | Fe; Mn; Zn |
|  | NIES-2 | NIES, Japan | Pond sediment | 105.0ppm | 6% | www.nies.go.jp | Al; Fe; Ca; K; Na; Zn; Cu; Cr; Co; As |
|  | NIES-9 | NIES, Japan | Sargasso sediment | 1.35ppm | 4% | www.nies.go.jp | K; Na; Ca; Mg; Sr; Fe; As; Rb; Mn; Zn; Cu; V; Ag; Co |
|  | JK-0015-Pb-1000 | CITI, Japan | Calibration solution in aqueous HNO3 | 0.1% | 0.3% | http://www.comar.bam.de/ | non |
|  | GBW-08601 | NRCCRM, China | Water | 1.0ppm | 0.02% | http://www.comar.bam.de/ | non |
|  | GBW-08607 | NRCCRM, China | Water | 1.0ppm | 2% | http://www.comar.bam.de/ | Cu; Cr; Zn |
|  | GBW-08608 | NRCCRM, China | Water | 50.0ppb | 4% | http://www.comar.bam.de/ | Cu; Cr; Zn |
|  | GBW-08301 | NRCCRM, China | River sediment | 79.0ppm | 20% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-08571 | NRCCRM, China | Mussel Tissue | 1.96ppm | 6% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-07314 | NRCCRM, China | Offshore marine sediment | 25.0ppm | 20% | http://www.comar.bam.de/ | Cu; Zn; Cr; As; Se; Co; Sr; Ba |
|  | SLRM-12-2-02 | pb-anal, Slovakia | Green algae | 1.23ppm | 6% | http://www.comar.bam.de/ | As; Ca; Co; Cr; Cu; Fe; K; Mg; Mn; Na; P; Zn |
|  | SLRM-12-3-10 | Research Institute for Irrigation, Slovakia | Water (acidified) | 0.029ppm | 6% | www.cm.inms.nrc.ca | Ag; Al; As; Co; Cr; Cu; Fe; K; Mn; Na; Zn |
|  | MESS-2 | NRC-CNRC, Canada | Marine sediment | 21.9ppm | 6% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; C; S |
|  | CASS-3 | NRC-CNRC, Canada | Nearshore seawater | 0.012ppb | 40% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Mo; Zn |
|  | SLEW-2 | NRC-CNRC, Canada | Estuarine water | 0.027ppb | 20% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Zn |
|  | DORM-2 | NRC-CNRC, Canada | Dogfish muscle | 0.065ppm | 10% | Padberg et al.59 Goessler et al.60 | Al; As; Co; Cr; Cu; Fe; Mn; Se; Ag |
|  | LUTS-1 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 0.01ppm | 20% | www.cm.inms.nrc.ca | As; Ca; Cr; Cu; Co; Fe; Mg; Mn; K; Ag; Se; Sr; Zn |
|  | TORT-2 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 0.35ppm | 40% | Padberg et al.59 | As; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | NASS-5 | NRC-CNRC, Canada | Ocean seawater | 0.008ppb | 60% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Mo; Zn |
|  | SLRS-4 | NRC-CNRC, Canada | Riverine water | 0.086ppb | 8% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Sr; U; V; Zn; Ca; Mg; K; Na |
|  | HISS-1 | NRC-CNRC, Canada | Marine sediment | 3.13ppm | 10% | www.cm.inms.nrc.ca | As; Be; Cr; Cu; Li; Mn; Se; Ag; Sr; V; Zn |
|  | PACS-2 | NRC-CNRC, Canada | Harbour sediment | 183.0ppm | 7% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; S |
|  | TM-23.2 | NRC-CNRC, Canada | Lake water | 3.8ppb | 30% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | TM-24 | NRC-CNRC, Canada | Water | 7.3ppb | 40% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be |
|  | TM-26.2 | NRC-CNRC, Canada | Lake water | 9.9ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; Tl; V; Zn |
|  | TM-27 | NRC-CNRC, Canada | Water | 4.9ppb | 50% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Li; Be |
|  | TM-27.2 | NRC-CNRC, Canada | Water | 3.2ppb | 30% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Ag; Sr; V; Zn |
|  | TM-28 | NRC-CNRC, Canada | Lake water | 3.0ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Li; Mn; Mo; Se; Sr; V; Zn |
|  | TMRAIN-95 | NRC-CNRC, Canada | Rain water | 0.29ppb | 30% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Bi; Li; Be; U; Tl |
|  | TMDA-51.2 | NRC-CNRC, Canada | Lake water | 72.9ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; Tl; U; V; Zn |
|  | TMDA-52.2 | NRC-CNRC, Canada | Lake water | 368.0ppb | 10% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be; U |
|  | TMDA-53.2 | NRC-CNRC, Canada | Lake water | 360.0ppb | 10% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Cu; Fe; Mn; Mo; Se; Sr; U; V; Zn |
|  | TMDA-54.2 | NRC-CNRC, Canada | Lake water | 531.0ppb | 10% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be; U |
| Mercury and its compunds | BCR-320 | European Commission | River sediment | 1.03mg/kg | 0.13mg/kg | www.irmm.jrc.be | As; Cr; Cu; Sc; Se; Zn |
|  | BCR-060 | European Commission | Aquatic plant | 0.34mg/kg | 0.04mg/kg | www.irmm.jrc.be | Al; Cu; Mn; Zn |
|  | BCR-278R | European Commission | Mussel Tissue | 0.196mg/kg | 0.009mg/kg | Griepink et al.55 Muntau 56 | As; Cu; Se; Zn |
|  | BCR-414 | European Commission | Plankton | 0.276mg/kg | 0.018mg/kg | Papadakis et al.57 | As; Cr; Cu; Mn; Se; Zn |
|  | BCR-422 | European Commission | Cod muscle | 0.559mg/kg | 0.016mg/kg | Quevauviller et al.58 Muntau 56 | As; Cr; Cu; Mn; Se; V; Zn |
|  | BCR-579 | European Commission | Coastal sea-water | 1.85ng/kg | 0.20ng/kg | www.irmm.jrc.be | As; Cu; Fe; I; Mn; Se; Zn |
|  | BCR-463 | European Commission | Tuna fish | 2.85ppm | 6% | Quevauviller et al.63 | non |
|  | BCR-464 | European Commission | Tuna fish | 5.24ppm | 2% | Quevauviller et al.63 | non |
|  | BCR-580 | European Commission | Estuarine sediment | 132.0ppm | 2% | www.irmm.jrc.be | non |
|  | SRM-2704 | NIST | River sediment | 0.000147% | 20% | http//ts.nist.gov/ts | Al; Ca C; Fe; Mg; P; K; Si; Na; S; Ti; Sb; As; Cr; Co; Cu; Li,; Mn; Se; Tl; U; V; Zn |
|  | 8004-93 | Research and Control Centre of Water, Russia | Spectrometric standard in water | 0.0972% | 1% | http://www.comar.bam.de/ | non |
|  | GSO-3497-86 | SCTB-EP, Ukraine | Spectrometric standard in HNO3/water | 0.096% | 1% | http://www.comar.bam.de/ | non |
|  | GBW-08603 | NRCCRM, China | Water | 0.01ppm | 0.0004% | http://www.comar.bam.de/ | non |
|  | GBW-08301 | NRCCRM, China | River sediment | 0.22ppm | 20% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-08571 | NRCCRM, China | Mussel Tissue | 0.067ppm | 10% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-08609 | NRCCRM, China | Water | 1.0ppm | 5% | http://www.comar.bam.de/ | non |
|  | GBW-07314 | NRCCRM, China | Offshore marine sediment | 0.048ppm | 30% | http://www.comar.bam.de/ | Cu; Zn; Cr; As; Se; Co; Sr; Ba |
|  | SLRM-12-3-10 | Research Institute for Irrigation, Slovakia | Water (acidified) | 0.0011ppm | 30% | www.cm.inms.nrc.ca | Ag; Al; As; Co; Cr; Cu; Fe; K; Mn; Na; Zn |
|  | MESS-2 | NRC-CNRC, Canada | Marine sediment | 0.092ppm | 100% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; C; S |
|  | DORM-2 | NRC-CNRC, Canada | Dogfish muscle | 4.64ppm | 6% | Padberg et al.59 Goessler et al.60 | Al; As; Co; Cr; Cu; Fe; Mn; Se; Ag |
|  | LUTS-1 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 0.0167ppm | 20% | www.cm.inms.nrc.ca | As; Ca; Cr; Cu; Co; Fe; Mg; Mn; K; Ag; Se; Sr; Zn |
|  | TORT-2 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 0.27ppm | 20% | Padberg et al.59 | As; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | PACS-2 | NRC-CNRC, Canada | Harbour sediment | 3.04ppm | 7% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; S |
|  | WQB-1 | NRC-CNRC, Canada | Lake sediment | 1.09ppm | 20% | www.cm.inms.nrc.ca | As; Se |
|  | WQB-3 | NRC-CNRC, Canada | Lake sediment | 2.75ppm | 10% | www.cm.inms.nrc.ca | Co; Fe; Mn; Zn; As; Se |
| Naphthalene | SRM-1941a | NIST | Marine sediment | 1010.0ppb | 20% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.002562% | 6% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 2.68ug/kg | 0.5ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 0.25ppm | 30% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 4.1ppm | 30% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | DPAC-1 | NRC-CNRC, Canada | Deuterated calibration solution toluene | 116.0ppm | N/A | www.cm.inms.nrc.ca | 1-methylnaphthalene-D10; biphenyl-D10; acenaphtylene-D8; acenaphtene-D10; dibenzofuran-D8; fluorene-D10; dibenzo(b,d)thiophene-D8; phenanthrene-D10; pyrene-D10; benz(a)anthracene-D12; chrysene-D12; 9-methylanthracene-D12; coronene-D12; dibenzo(a,i)pyrene-D14 |
|  | HS-3B | NRC-CNRC, Canada | Marine sediment | 2.14ppm | 2% | www.cm.inms.nrc.ca | 1-methylnaphthalene, biphenyl, acenaphtylene, acenaphtene, dibenzofuran, fluorene, dibenzothiophene, phenanthrene, pyrene, benz(a)anthracene, chrysene, benzo(b,k)fluoranthene, dibenz(a, h)anthracene, coronene, dibenzo(a,i)pyrene |
|  | HS-4B | NRC-CNRC, Canada | Marine sediment | 0.22ppm | 9% | Fernandez et al.54 | 1-methylnaphthalene, biphenyl, acenaphtylene, acenaphthene, dibenzofuran, fluorene, dibenzothiophene, phenanthrene, pyrene, benz(a)anthracene, chrysene, benzo(b,k)fluoranthene, dibenz(a, h)anthracene, coronene, dibenzo(a,i)pyrene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 10.0ppb | 10% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| Nickel and its compounds | BCR-320 | European Commission | River sediment | 75.2mg/kg | 1.4 | www.irmm.jrc.be | As; Cr; Cu; Sc; Se; Zn |
|  | BCR-414 | European Commission | Plankton | 18.8mg/kg | 0.8mg/kg | Quevauviller et al.58 Muntau 56 | As; Cr; Cu; Mn; Se; V; Zn |
|  | BCR-403 | European Commission | Sea water | 4.4nmol/kg | 0.4nmol/kg | www.irmm.jrc.be | Al; As; Cu; Mo; V; Zn |
|  | BCR-505 | European Commission | Estuarine water | 24.1mg/kg | 2.0mg/kg | www.irmm.jrc.be | Cu; Zn |
|  | SRM-2704 | NIST | River sediment | 0.00441% | 7% | http//ts.nist.gov/ts | Al; Ca; C; Fe; Mg; P; K; Si; Na; S; Ti; Sb; As; Cr; Co; Cu; Li; Mn; Se; Tl; U; V; Zn |
|  | LGC-6175 | LGC-Promochem | Landfill leachate | 93.0ppb | 30% | www.lgc.co.uk | B; Ca; Mg; K; Na; Fe; Mn; Zn |
|  | LGC-6010 | LGC-Promochem | Hard drinking water | 51.0ppb | 8% | www.lgc.co.uk | Al; Ag; As; Ba; Cr; Fe; Mn; Se; Sb; Zn; Ca; K; Mg; Na |
|  | LGC-6011 | LGC-Promochem | Soft drinking water | 51.0ppb | 8% | www.lgc.co.uk | Al; Ag; Ba; Cr; Fe; Mn; Zn; Ca; K; Mg; Na |
|  | 8001-93 | Research and Control Centre of Water, Russia | Spectrometric standard in water | 0.1001% | 1% | http://www.comar.bam.de/ | non |
|  | 7272-96 | Research and Control Centre of Water, Russia | Spectrometric multi-element standard in water | 0.0102% | 2% | http://www.comar.bam.de/ | Fe; Mn; Zn |
|  | GSO-5230-90 | SCTB-EP, Ukraine | Spectrometric standard in HNO3/water | 0.97% | 1% | http://www.comar.bam.de/ | non |
|  | NIES-2 | NIES, Japan | Pond sediment | 40.0ppm | 8% | www.nies.go.jp | Al; Fe; Ca; K; Na; Zn; Cu; Cr; Co; As |
|  | JK-0013-Ni-1000 | CITI, Japan | Calibration solution in aqueous HNO3 | 0.1% | 0.3% | http://www.comar.bam.de/ | non |
|  | GBW-08607 | NRCCRM, China | Water | 0.5ppm | 2% | http://www.comar.bam.de/ | Cu; Cr; Zn |
|  | GBW-08608 | NRCCRM, China | Water | 60.0ppb | 5% | http://www.comar.bam.de/ | Cu; Cr; Zn |
|  | GBW-08571 | NRCCRM, China | Mussel Tissue | 1.03ppm | 10% | http://www.comar.bam.de/ | As; Ba; Co; Cr; Cu; Mn; Se; Fe |
|  | GBW-07314 | NRCCRM, China | Offshore marine sediment | 34.3ppm | 10% | http://www.comar.bam.de/ | Cu; Zn; Cr; As; Se; Co; Sr; Ba |
|  | SLRM-12-3-10 | Research Institute for Irrigation, Slovakia | water (acidified) | 0.061ppm | 10% | www.cm.inms.nrc.ca | Ag; Al; As; Co; Cr; Cu; Fe; K; Mn; Na; Zn |
|  | 96962 | RIZA, The Netherlands | Cyanide polluted sediment | 20.2ppm | 10% | Jenks et al.64 | Ca; Cu; Fe; N; Mn |
|  | MESS-2 | NRC-CNRC, Canada | Marine sediment | 49.3ppm | 4% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; C; S |
|  | CASS-3 | NRC-CNRC, Canada | Nearshore seawater | 0.386ppb | 20% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Mo; Zn |
|  | SLEW-2 | NRC-CNRC, Canada | Estuarine water | 0.709ppb | 8% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Zn |
|  | DORM-2 | NRC-CNRC, Canada | Dogfish muscle | 19.4ppm | 20% | Padberg et al.59 Goessler et al.60 | Al; As; Co; Cr; Cu; Fe; Mn; Se; Ag |
|  | LUTS-1 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 0.2ppm | 8% | www.cm.inms.nrc.ca | As; Ca; Cr; Cu; Co; Fe; Mg; Mn; K; Ag; Se; Sr; Zn |
|  | TORT-2 | NRC-CNRC, Canada | Lobster hepatopancreas tissue | 2.5ppm | 8% | Padberg et al.59 | As; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | NASS-5 | NRC-CNRC, Canada | Ocean seawater | 0.253ppb | 10% | www.cm.inms.nrc.ca | As; Cr; Co; Cu; Fe; Mn; Mo; Zn |
|  | SLRS-4 | NRC-CNRC, Canada | Riverine water | 0.67ppb | 10% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Sr; U; V; Zn; Ca; Mg; K; Na |
|  | PACS-2 | NRC-CNRC, Canada | Harbour sediment | 39.5ppm | 6% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; S |
|  | WQB-3 | NRC-CNRC, Canada | Lake sediment | 52.0ppm | 10% | www.cm.inms.nrc.ca | Co; Fe; Mn; Zn; As; Se |
|  | TM-23.2 | NRC-CNRC, Canada | Lake water | 5.3ppb | 30% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; V; Zn |
|  | TM-24 | NRC-CNRC, Canada | water | 3.5ppb | 90% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be |
|  | TM-26.2 | NRC-CNRC, Canada | lake water | 9.9ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; Tl; V; Zn |
|  | TM-27 | NRC-CNRC, Canada | water | 2.7ppb | 40% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Li; Be |
|  | TM-27.2 | NRC-CNRC, Canada | water | 3.2ppb | 30% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Ag; Sr; V; Zn |
|  | TM-28 | NRC-CNRC, Canada | lake water | 3.0ppb | 20% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Li; Mn; Mo; Se; Sr; V; Zn |
|  | TMRAIN-95 | NRC-CNRC, Canada | rain water | 0.8ppb | 20% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Bi; Li; Be; U; Tl |
|  | TMDA-51.2 | NRC-CNRC, Canada | lake water | 66.7ppb | 10% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Co; Cu; Fe; Mn; Mo; Se; Sr; Tl; U; V; Zn |
|  | TMDA-52.2 | NRC-CNRC, Canada | lake water | 268.0ppb | 9% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be; U |
|  | TMDA-53.2 | NRC-CNRC, Canada | lake water | 319.0ppb | 9% | www.cm.inms.nrc.ca | Al; Sb; As; Ba; Be; Cr; Cu; Fe; Mn; Mo; Se; Sr; U; V; Zn |
|  | TMDA-54.2 | NRC-CNRC, Canada | lake water | 325.0ppb | 100% | www.cm.inms.nrc.ca | Al; V; Cr; Mn; Fe; Co; Cu; Zn; Sr; Mo; Ba; As; Se; Sb; Be; U |
| Nonylphenols | non |  |  |  |  |  |  |
| Octylphenols | non |  |  |  |  |  |  |
| Pentachlorobenzene | EC-1 | NRC-CNRC, Canada | Harbour sediment | 1.7ppb | 30% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 48.6ppb | 5% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| Pentachlorophenol | SRM-1584 | NIST | Calibration solution in methanol | 0.00197% | 7% | http//ts.nist.gov/ts | 2-chlorophenol; phenol; 2-nitrophenol; 4-chloro-m-cresol; 2,4,6-trichlorophenol; 4-nitrophenol; 4,6-dinitro-o-cresol; 2,4-dimethylphenol; 2,4-dichlorophenol |
|  | LGC-1623 | LGC-Promochem | Pure compound | 99.3% | N/A | www.lgc.co.uk | non |
|  | 7102-97 | Research and Control Centre of Water, Russia | Pure compound | 99.4% | 0.1% | http://www.comar.bam.de/ | non |
|  | DMR-21a | CENAM, Mexico | Calibration solutions in methanol | 1.1ppm | N/A | http://www.comar.bam.de/ | 2-chlorophenol; phenol; 2-nitrophenol; 4-chloro-m-cresol; 2,4,6-trichlorophenol; 4-nitrophenol; 4,6-dinitro-o-cresol; 2,4-dimethylphenol; 2,4-dichlorophenol |
| PAHs |  |  |  |  |  |  |  |
| benzo(a)pyrene | BCR-535 | European Commission | Freshwater sediment | 1.54mg/kg | 0.1mg/kg | Wegener et al.65 | pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | BCR-51R | European Commission | Pure compound | 99.20% | N/A | www.irmm.jrc.be | non |
|  | SRM-1941a | NIST | Marine sediment | 628.0ppb | 8% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.000632% | 3% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 1.78ug/kg | 0.073ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | GBW-08701 | NRCCRM, China | Calibration solution in methanol | 5.75ppm | 2% | http://www.comar.bam.de/ | non |
|  | GBW-08702 | NRCCRM, China | Calibration solution in methanol | 10.0ppm | 2% | http://www.comar.bam.de/ | non |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 1.7ppm | 50% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 2.2ppm | 20% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | DPAC-1 | NRC-CNRC, Canada | Deuterated calibration solution toluene | 35.0ppm | N/A | www.cm.inms.nrc.ca | 1-methylnaphthalene-D10; biphenyl-D10; acenaphtylene-D8; acenaphthene-D10; dibenzofuran-D8; fluorene-D10; dibenzo(b,d)thiophene-D8, phenanthrene-D10; pyrene-D10; benz(a)anthracene-D12; chrysene-D12; 9-methylanthracene-D12, coronene-D12, dibenzo(a,i)pyrene-D14 |
|  | HS-3B | NRC-CNRC, Canada | Marine sediment | 5.8ppm | 3% | www.cm.inms.nrc.ca | 1-methylnaphthalene, biphenyl, acenaphtylene, acenaphthene, dibenzofuran, fluorene, dibenzothiophene, phenanthrene, pyrene, benz(a)anthracene, chrysene, benzo(b,k)fluoranthene, dibenz(a, h)anthracene, coronene, dibenzo(a,i)pyrene |
|  | HS-4B | NRC-CNRC, Canada | Marine sediment | 1.55ppm | 100% | Fernandez et al.54 | 1-methylnaphthalene; biphenyl; acenaphtylene; acenaphthene; dibenzofuran; fluorene; dibenzothiophene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(b,k)fluoranthene; dibenz(a, h)anthracene; coronene; dibenzo(a,i)pyrene |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 5.3ppm | 20% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 1.21ppm | 30% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-3 | NRC-CNRC, Canada | River sediment | 386.0ppb | 10% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; total PCB |
|  | EC-5 | NRC-CNRC, Canada | River sediment | 449.0ppb | 20% | www.cm.inms.nrc.ca | total PCB |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 207.0ppb | 10% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| fluoranthene | SRM-1941a | NIST | Marine sediment | 981.0ppb | 8% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.00098% | 1% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 18.6ug/kg | 1.0ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 8.4ppm | 30% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 3.54ppm | 20% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | DPAC-1 | NRC-CNRC, Canada | Deuterated calibration solution toluene | 116.0ppm | N/A | www.cm.inms.nrc.ca | 1-methylnaphthalene-D10; biphenyl-D10; acenaphtylene-D8; acenaphthene-D10; dibenzofuran-D8; fluorene-D10; dibenzo(b,d)thiophene-D8, phenanthrene-D10; pyrene-D10; benz(a)anthracene-D12; chrysene-D12; 9-methylanthracene-D12, coronene-D12, dibenzo(a,i)pyrene-D14 |
|  | HS-3B | NRC-CNRC, Canada | Marine sediment | 25.33ppm | 0.50% | www.cm.inms.nrc.ca | 1-methylnaphthalene, biphenyl, acenaphtylene, acenaphthene, dibenzofuran, fluorene, dibenzothiophene, phenanthrene, pyrene, benz(a)anthracene, chrysene, benzo(b,k)fluoranthene, dibenz(a, h)anthracene, coronene, dibenzo(a,i)pyrene |
|  | HS-4B | NRC-CNRC, Canada | Marine sediment | 3.33ppm | 4% | Fernandez et al.54 | 1-methylnaphthalene; biphenyl; acenaphtylene; acenaphthene; dibenzofuran; fluorene; dibenzothiophene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(b,k)fluoranthene; dibenz(a, h)anthracene; coronene; dibenzo(a,i)pyrene |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 23.2ppm | 9% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 3.55ppm | 10% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-3 | NRC-CNRC, Canada | River sediment | 558.0ppb | 8% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; total PCB |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 462.0ppb | 9% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| benzo(b)fluoranthene | BCR-535 | European Commission | Freshwater sediment | 2.29mg/kg | 0.15mg.kg | Wegener et al.65 | pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | BCR-047 | European Commission | Pure compound | 99.50% | N/A | www.irmm.jrc.be | non |
|  | SRM-1941a | NIST | Marine sediment | 740.0ppb | 20% | Schantz et al. 50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.000537% | 100% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 5.28ug/kg | 0.42ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 2.0ppm | 50% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 2.8ppm | 20% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | DPAC-1 | NRC-CNRC, Canada | Deuterated calibration solution toluene | 35.0ppm | N/A | www.cm.inms.nrc.ca | 1-methylnaphthalene-D10; biphenyl-D10; acenaphtylene-D8; acenaphthene-D10; dibenzofuran-D8; fluorene-D10; dibenzo(b,d)thiophene-D8, phenanthrene-D10; pyrene-D10; benz(a)anthracene-D12; chrysene-D12; 9-methylanthracene-D12, coronene-D12, dibenzo(a,i)pyrene-D14 |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 7.9ppm | 10% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 2.48ppm | 20% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 208.0ppb | 10% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| benzo(k)fluoranthene | BCR-535 | European Commission | Freshwater sediment | 1.09mg/kg | 0.15mg/kg | Wegener et al.65 | pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | BCR-048R | European Commission | Pure compound | 99.50% | N/A | www.irmm.jrc.be | non |
|  | SRM-1941a | NIST | Marine sediment | 361.0ppb | 5% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.000601% | 1% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 2.3ug/kg | 0.1ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | GBW-08701 | NRCCRM, China | Calibration solution in methanol | 5.75ppm | 2% | http://www.comar.bam.de/ | non |
|  | GBW-08702 | NRCCRM, China | Calibration solution in methanol | 10.0ppm | 2% | http://www.comar.bam.de/ | non |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 1.0ppm | 40% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 1.43ppm | 10% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 4.4ppm | 10% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 1.93ppm | 20% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 294.0ppb | 10% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5- tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| indeno(1,2,3-cd)pyrene | BCR-535 | European Commission | Freshwater sediment | 1.56mg/kg | 0.14mg/kg | Wegener et al.65 | pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | BCR-053 | European Commission | Pure compound | 99.0% | N/A | www.irmm.jrc.be | non |
|  | SRM-1941a | NIST | Marine sediment | 501.0ppb | 20% | Schantz et al.52 Wise et al.53 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.000553% | 1% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 1.62ug/kg | 0.32ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 1.3ppm | 60% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 1.95ppm | 30% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 5.7ppm | 10% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 1.55ppm | 20% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 34.0ppb | 20% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| benzo(g,h,i)perylene | BCR-052 | European Commission | Pure compound | 99.00% | N/A | www.irmm.jrc.be | non |
|  | SRM-1941a | NIST | Marine sediment | 525.0ppb | 10% | Schantz et al.50 Wise et al.51 | pyrene; benz(a)anthracene; perylene; fluorene; chrysene; triphenylene; benzo(a)fluoranthene; benzo(e)pyrene; dibenz(a,j)anthracene; dibenz(a,c)anthracene; dibenz(a,h)anthracene; benzo(b)chrysene; picene; pentaphene; cis-chlordane; biphenyl, acenaphtylene; acenaphthene; 1-methylphenanthrene; benzo(j)fluoranthene; benzo(c)phenanthrene; benzo(ghi)fluoranthene; indeno(1,2,3-cd)fluoranthene, anthanthrene;trans-nonachlor; 2,4'-DDE; 4,4'-DDE; 4,4'-DDD; decachlorobiphenyl; 2,2',3,5'-PCB; 2,2',5,5'-PCB; 2,3',4,4'-PCB; 2,2',4,5,5'-PCB; 2,3,3',4,4'-PCB; 2,3',4,4',5-PCB; 2,2',3,3',4,4'-PCB; 2,2',3,4,4',5'-PCB; 2,2',4,4',5,5'-PCB; 2,2',3,4,4',5,5'-PCB; 2,4'-PCB; 2,2'5-PCB; 2,4,4'-PCB; 2,2',4,5'-PCB; 2,2',3,4,5'-PCB; 2,2',3,5',6-PCB; 2,2',4,4',5'-PCB; 2,3,3',4',6-PCB; 2,2',3,4',5',6-PCB; 2,3,3',4,4',5-PCB; 2,2',3,3',4,4',5-PCB; 2,2',3,3',4,4',5,5'-PCB; 2,2',3,3',4,4',5,5',6-PCB |
|  | SRM-1647c | NIST | Calibration solution in Acetonitrile | 0.000473% | 1% | http//ts.nist.gov/ts | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | SRM-1974a | NIST | Organics in mussel tissue | 2.5ug/kg | 0.25ug/kg | Schantz et al.52 Schantz et al.53 | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene; chrysene; triphenylene; perylene; PCB 44; PCB 49; PCB 52; PCB 66; PCB 95; PCB 99; PCB 101; PCB 105; PCB 110; PCB 118; PCB 128; PCB 138; PCB 149; PCB 151; PCB 153; PCB 156; PCB 170; PCB 180; PCB 183; PCB 187 |
|  | HS-5 | NRC-CNRC, Canada | Marine sediment | 1.3ppm | 30% | www.cm.inms.nrc.ca | acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-6 | NRC-CNRC, Canada | Marine sediment | 1.78ppm | 40% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; dibenz(a,h)anthracene |
|  | HS-3B | NRC-CNRC, Canada | Marine sediment | 3.88ppm | 4% | www.cm.inms.nrc.ca | 1-methylnaphthalene, biphenyl, acenaphtylene, acenaphthene, dibenzofuran, fluorene, dibenzothiophene, phenanthrene, pyrene, benz(a)anthracene, chrysene, benzo(b,k)fluoranthene, dibenz(a, h)anthracene, coronene, dibenzo(a,i)pyrene |
|  | HS-4B | NRC-CNRC, Canada | Marine sediment | 1.23ppm | 10% | Fernandez et al.54 | 1-methylnaphthalene; biphenyl; acenaphtylene; acenaphthene; dibenzofuran; fluorene; dibenzothiophene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(b,k)fluoranthene; dibenz(a, h)anthracene; coronene; dibenzo(a,i)pyrene |
|  | EC-1 | NRC-CNRC, Canada | Harbour sediment | 4.9ppm | 20% | www.cm.inms.nrc.ca | phenanthrene; pyrene; benz(a)anthracene; benzo(e)pyrene |
|  | EC-2 | NRC-CNRC, Canada | Lake sediment | 1.47ppm | 20% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 176.0ppb | 20% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5- tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| Simazine | LGC-1004 | LGC-Promochem | Calibration solution in water | 25.0ppb | 8% | www.lgc.co.uk | trietazine; terbuthylazine |
|  | SRM-3070 | NIST | Calibration solution in acetone | 49.4mg/kg | 1.0mg/kg | http//ts.nist.gov/ts | bromacil; butachlor; metolachlor; metribuzin; prometon |
| Tributyltin compounds | BCR-462 | European Commission | Coastal sediment | 54.0ug/kg | 15ug/kg | Quevauviller et al.5 | dibutyltin |
|  | BCR-477 | European Commission | Mussel tissue | 2.20ug/kg | 0.19ug/kg | www.irmm.jrc.be | dibutyltin |
|  | BCR-646 | European Commission | Freshwater sediment | 480.0µk/kg | 80.0µg/kg | www.irmm.jrc.be | dibutyltin; monobutyltin; triphenyltin; diphenyltin; monophenyltin |
|  | PACS-2 | NRC-CNRC, Canada | Harbour sediment | 0.98ppm | 20% | www.cm.inms.nrc.ca | Sb; As; Be; Cr; Co; Cu; Li; Mn; Mo; Se; Ag; Sr; Sn; V; Zn; S |
| Trichlorobenzenes (1,2,4-trichlorobenzene) | EC-2 | NRC-CNRC, Canada | Lake sediment | 80.7ppb | 7% | www.cm.inms.nrc.ca | pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
|  | EC-8 | NRC-CNRC, Canada | Lake sediment | 67.0ppb | 6% | www.cm.inms.nrc.ca | acenaphtylene; acenaphthene; fluorene; phenanthrene; pyrene; benz(a)anthracene; chrysene; benzo(e)pyrene; perylene; dibenz(a,h)anthracene; 1,2-dichlorobenzene; 1,3-dichlorobenzene; 1,4-dichlorobenzene; 1,2,3-trichlorobenzene; 1,3,5-trichlorobenzene; 1,2,3,4-tetrachlorobenzene; 1,2,3,5-tetrachlorobenzene; 1,2,4,5-tetrachlorobenzene |
| Trichloromethane (Chloroform) | 7288-96 | Research and Control Centre of Water, Russia | Pure compound | 99.88% | 0.02% | http://www.comar.bam.de/ | non |
| Trifluralin | SRM-3069 | NIST | Calibration solution in acetone | 5.34mg/kg | 0.11mg/kg | http//ts.nist.gov/ts | non |