Supplementary data

The following is step listing for a custom "Begin" procedure for use on an ABI 394 DNA synthesizer which performs an automated nucleoside addition on a synthesis column containing underivatized support. Functions #200, 203, 206, and 209 are custom user defined functions which simultaneously deliver reagents from base positions #7 and #8 to columns 1, 2, 3, and 4, respectively. An acetonitrile solution of nucleoside-3'-O-hydroquinone-O,O'-diacetic acid hemiester (2) and diisopropylethylamine is installed on base position #7. An acetonitrile solution of HBTU/DMAP is installed on base position #8.

- 394 Begin Procedure Listing -           (Version 2.00)

Name : Loadl234&CAP                                      5:29:44P, 3/15/ 1 Steps:   48

Step   Function                 Step       Step Active for Bases      Safe

Number #   Name                 Time                                  Step

------   -------------------    -----   --------------------------    ----

1      106 Begin                                                      Yes

2       64 18 to Waste            5.0                                 Yes

3       42 18 to Column          30.0                                 Yes

4        2 Reverse Flush         10.0                                 Yes

5       42 18 to Column          30.0                                 Yes

6        2 Reverse Flush         10.0                                 Yes

7       42 18 to Column          10.0                                 Yes

8        2 Reverse Flush          5.0                                 Yes

9       42 18 to Column          10.0                                 Yes

10        2 Reverse Flush        10.0                                 Yes

11        I Block Flush           5.0                                 Yes

12      101 Phos Prep            10.0                                 Yes

13      200 7+8 to Colm 1         4.0                                 Yes

14      203 7+8 to Colm 2         4.0                                 Yes

15      206 7+8 to Colm 3         4.0                                 Yes

16      209 7+8 to Colm 4         4.0                                 Yes

17      103 Wait                600.0                                 Yes

18        2 Reverse Flush         5.0                                 Yes

19       42 18 to Column         20.0                                 Yes

20        2 Reverse Flush        10.0                                 Yes

21       42 18 to Column         20.0                                 Yes

22        2 Reverse Flush        10.0                                 Yes

23       42 18 to Column         20.0                                 Yes

24        2 Reverse Flush        10.0                                 Yes

25       42 18 to Column         20.0                                 Yes

26        2 Reverse Flush        10.0                                 Yes

27        1 Block Flush           5.0                                 Yes

28      102 Cap Prep             10.0                                 Yes

29       39 Cap to Column        12.0                                 Yes

30      103 Wait                100.0                                 Yes

31        2 Reverse Flush         5.0                                 Yes

32      102 Cap Prep             10.0                                 Yes

33       39 Cap to Column        10.0                                 Yes

34      103 Wait                 85.0                                 Yes

35        2 Reverse Flush         5.0                                 Yes

36      102 Cap Prep             10.0                                 Yes

37       39 Cap to Column        10.0                                 Yes

38      103 Wait                 85.0                                 Yes

39        2 Reverse Flush         5.0                                 Yes

40        1 Block Flush           3.0                                 Yes

41       42 18 to Column         20.0                                 Yes

42        4 Flush to Waste        4.0                                 Yes

43       42 18 to Column         20.0                                 Yes

44        4 Flush to Waste        5.0                                 Yes

45       42 18 to Column         10.0                                 Yes

46        2 Reverse Flush        10.0                                 Yes

47        1 Block Flush           4.0                                 Yes

48      107 End                                                       Yes